x86-64

Assembly Language

Programming

with

Ubuntu

Ed Jorgensen, Ph.D.

Version 1.1.44

May 2022

Cover image:

Top view of an Intel central processing unit Core i7 Skylake type core,

model 6700K, released in June 2015.

Source: Eric Gaba, https://commons.wikimedia.org/wiki/File:

Intel_CPU_Core_i7_6700K_Skylake_top.jpg

Cover background:

By Benjamint444 (Own work)

Source: http://commons.wikimedia.org/wiki/File%3ASwirly_belt444.jpg

Copyright © 2015 - 2022 by Ed Jorgensen

You are free:

To Share — to copy, distribute and transmit the work

To Remix — to adapt the work

Under the following conditions:

Attribution — you must attribute the work in the manner specified by the author

or licensor (but not in any way that suggests that they endorse you or your use of

the work).

Noncommercial — you may not use this work for commercial purposes.

Share Alike — if you alter, transform, or build upon this work, you may

distribute the resulting work only under the same or similar license to this one.

Table of Contents

Table of Contents

1

.0 Introduction...........................................................................................................1

1

1

1

.1 Prerequisites........................................................................................................1

.2 What is Assembly Language...............................................................................2

.3 Why Learn Assembly Language.........................................................................2

1

1

1

1

1

1

1

1

.3.1 Gain a Better Understanding of Architecture Issues...................................3

.3.2 Understanding the Tool Chain.....................................................................3

.3.3 Improve Algorithm Development Skills.....................................................3

.3.4 Improve Understanding of Functions/Procedures.......................................3

.3.5 Gain an Understanding of I/O Buffering.....................................................4

.3.6 Understand Compiler Scope........................................................................4

.3.7 Introduction Multi-processing Concepts.....................................................4

.3.8 Introduction Interrupt Processing Concepts................................................4

1

.4 Additional References.........................................................................................4

1

1

1

1

.4.1 Ubuntu References......................................................................................5

.4.2 BASH Command Line References..............................................................5

.4.3 Architecture References..............................................................................5

.4.4 Tool Chain References................................................................................5

1

1

.4.4.1 YASM References...............................................................................6

.4.4.2 DDD Debugger References.................................................................6

2

.0 Architecture Overview.........................................................................................7

2

2

2

.1 Architecture Overview........................................................................................7

.2 Data Storage Sizes...............................................................................................8

.3 Central Processing Unit.......................................................................................9

2

.3.1 CPU Registers...........................................................................................10

2

2

2

2

2

2

.3.1.1 General Purpose Registers (GPRs)....................................................10

.3.1.2 Stack Pointer Register (RSP).............................................................12

.3.1.3 Base Pointer Register (RBP)..............................................................12

.3.1.4 Instruction Pointer Register (RIP).....................................................12

.3.1.5 Flag Register (rFlags)........................................................................12

.3.1.6 XMM Registers..................................................................................13

2

.3.2 Cache Memory..........................................................................................14

2

2

.4 Main Memory....................................................................................................15

.5 Memory Layout.................................................................................................17

Page iii

Table of Contents

2

2

.6 Memory Hierarchy............................................................................................17

.7 Exercises............................................................................................................19

.7.1 Quiz Questions..........................................................................................19

2

3

.0 Data Representation...........................................................................................21

3

.1 Integer Representation.......................................................................................21

3

3

3

.1.1 Two's Complement....................................................................................23

.1.2 Byte Example............................................................................................23

.1.3 Word Example...........................................................................................24

3

3

.2 Unsigned and Signed Addition.........................................................................24

.3 Floating-point Representation...........................................................................24

3

.3.1 IEEE 32-bit Representation.......................................................................25

3.3.1.1 IEEE 32-bit Representation Examples..............................................26

3

3

3

.3.1.1.1 Example → -7.75 .....................................................................26

10

.3.1.1.2 Example → -0.125 ...................................................................26

10

.3.1.1.3 Example → 41440000 .............................................................27

16

3

3

.3.2 IEEE 64-bit Representation.......................................................................27

.3.3 Not a Number (NaN).................................................................................27

3

3

.4 Characters and Strings.......................................................................................28

3

.4.1 Character Representation...........................................................................28

3

3

.4.1.1 American Standard Code for Information Interchange.....................28

.4.1.2 Unicode..............................................................................................29

3

.4.2 String Representation................................................................................29

.5 Exercises............................................................................................................29

3.5.1 Quiz Questions..........................................................................................30

4

.0 Program Format.................................................................................................33

4

4

4

4

4

4

4

4

.1 Comments..........................................................................................................33

.2 Numeric Values.................................................................................................33

.3 Defining Constants............................................................................................34

.4 Data Section......................................................................................................34

.5 BSS Section.......................................................................................................35

.6 Text Section.......................................................................................................36

.7 Example Program..............................................................................................37

.8 Exercises............................................................................................................39

4

.8.1 Quiz Questions..........................................................................................39

5

.0 Tool Chain...........................................................................................................41

5

5

.1 Assemble/Link/Load Overview........................................................................41

.2 Assembler..........................................................................................................43

Page iv

Table of Contents

5

5

5

.2.1 Assemble Commands................................................................................43

.2.2 List File......................................................................................................43

.2.3 Two-Pass Assembler.................................................................................45

5

5

.2.3.1 First Pass............................................................................................46

.2.3.2 Second Pass........................................................................................46

5

.2.4 Assembler Directives.................................................................................47

5

.3 Linker................................................................................................................47

5

5

5

.3.1 Linking Multiple Files...............................................................................48

.3.2 Linking Process.........................................................................................48

.3.3 Dynamic Linking.......................................................................................49

5

5

5

5

.4 Assemble/Link Script........................................................................................50

.5 Loader................................................................................................................51

.6 Debugger...........................................................................................................52

.7 Exercises............................................................................................................52

5

.7.1 Quiz Questions..........................................................................................52

6

.0 DDD Debugger....................................................................................................55

6

6

.1 Starting DDD.....................................................................................................55

.1.1 DDD Configuration Settings.....................................................................57

.2 Program Execution with DDD..........................................................................57

6

6

6

.2.1 Setting Breakpoints...................................................................................57

.2.2 Executing Programs...................................................................................58

6

6

.2.2.1 Run / Continue...................................................................................60

.2.2.2 Next / Step.........................................................................................60

6

6

.2.3 Displaying Register Contents....................................................................60

.2.4 DDD/GDB Commands Summary.............................................................62

6

.2.4.1 DDD/GDB Commands, Examples....................................................64

6

6

.2.5 Displaying Stack Contents........................................................................65

.2.6 Interactive Debugger Commands File.......................................................65

6

6

.2.6.1 Debugger Commands File (non-interactive).....................................66

.2.6.2 Non-interactive Debugger Commands File.......................................67

6

.3 Exercises............................................................................................................67

6

6

.3.1 Quiz Questions..........................................................................................67

.3.2 Suggested Projects.....................................................................................68

7

.0 Instruction Set Overview....................................................................................71

7

.1 Notational Conventions.....................................................................................71

7.1.1 Operand Notation......................................................................................72

7

.2 Data Movement.................................................................................................73

Page v

Table of Contents

7

7

.3 Addresses and Values........................................................................................75

.4 Conversion Instructions.....................................................................................76

7

7

.4.1 Narrowing Conversions.............................................................................76

.4.2 Widening Conversions..............................................................................76

7

7

.4.2.1 Unsigned Conversions.......................................................................77

.4.2.2 Signed Conversions...........................................................................78

7

.5 Integer Arithmetic Instructions.........................................................................80

7

.5.1 Addition.....................................................................................................80

7.5.1.1 Addition with Carry...........................................................................83

7

7

.5.2 Subtraction.................................................................................................86

.5.3 Integer Multiplication................................................................................89

7

7

.5.3.1 Unsigned Multiplication....................................................................89

.5.3.2 Signed Multiplication........................................................................93

7

.5.4 Integer Division.........................................................................................97

7

7

.6 Logical Instructions.........................................................................................104

7

7

.6.1 Logical Operations..................................................................................105

.6.2 Shift Operations.......................................................................................106

7

7

.6.2.1 Logical Shift....................................................................................106

.6.2.2 Arithmetic Shift...............................................................................108

7

.6.3 Rotate Operations....................................................................................110

.7 Control Instructions.........................................................................................111

7

7

7

.7.1 Labels......................................................................................................112

.7.2 Unconditional Control Instructions.........................................................112

.7.3 Conditional Control Instructions.............................................................112

7

.7.3.1 Jump Out of Range..........................................................................115

7

.7.4 Iteration....................................................................................................118

7

7

.8 Example Program, Sum of Squares.................................................................120

.9 Exercises..........................................................................................................121

7

7

.9.1 Quiz Questions........................................................................................121

.9.2 Suggested Projects...................................................................................125

8

.0 Addressing Modes.............................................................................................129

8

.1 Addresses and Values......................................................................................129

8

8

8

.1.1 Register Mode Addressing......................................................................130

.1.2 Immediate Mode Addressing..................................................................130

.1.3 Memory Mode Addressing......................................................................130

8

8

8

.2 Example Program, List Summation................................................................133

.3 Example Program, Pyramid Areas and Volumes............................................135

.4 Exercises..........................................................................................................140

Page vi

Table of Contents

8

8

.4.1 Quiz Questions........................................................................................140

.4.2 Suggested Projects...................................................................................143

9

.0 Process Stack.....................................................................................................145

9

9

9

.1 Stack Example.................................................................................................145

.2 Stack Instructions............................................................................................146

.3 Stack Implementation......................................................................................147

9

9

.3.1 Stack Layout............................................................................................147

.3.2 Stack Operations......................................................................................149

9

9

.4 Stack Example.................................................................................................151

.5 Exercises..........................................................................................................152

9

9

.5.1 Quiz Questions........................................................................................152

.5.2 Suggested Projects...................................................................................153

1

0.0 Program Development....................................................................................155

1

1

1

1

1

0.1 Understand the Problem................................................................................155

0.2 Create the Algorithm.....................................................................................156

0.3 Implement the Program.................................................................................158

0.4 Test/Debug the Program................................................................................160

0.5 Error Terminology.........................................................................................161

1

1

1

0.5.1 Assembler Error.....................................................................................161

0.5.2 Run-time Error.......................................................................................161

0.5.3 Logic Error............................................................................................162

1

0.6 Exercises........................................................................................................162

1

1

0.6.1 Quiz Questions......................................................................................162

0.6.2 Suggested Projects.................................................................................162

1

1.0 Macros..............................................................................................................165

1

1

1.1 Single-Line Macros.......................................................................................165

1.2 Multi-Line Macros........................................................................................166

1

1

1.2.1 Macro Definition...................................................................................166

1.2.2 Using a Macro.......................................................................................166

1

1

1

1.3 Macro Example.............................................................................................167

1.4 Debugging Macros........................................................................................169

1.5 Exercises........................................................................................................169

1

1

1.5.1 Quiz Questions......................................................................................169

1.5.2 Suggested Projects.................................................................................170

1

2.0 Functions..........................................................................................................171

1

2.1 Updated Linking Instructions........................................................................171

Page vii

Table of Contents

1

2.2 Debugger Commands....................................................................................172

1

1

2.2.1 Debugger Command, next.....................................................................172

2.2.2 Debugger Command, step.....................................................................172

1

1

1

1

1

1

2.3 Stack Dynamic Local Variables....................................................................172

2.4 Function Declaration.....................................................................................173

2.5 Standard Calling Convention........................................................................173

2.6 Linkage..........................................................................................................174

2.7 Argument Transmission................................................................................175

2.8 Calling Convention.......................................................................................175

1

1

1

2.8.1 Parameter Passing..................................................................................176

2.8.2 Register Usage.......................................................................................177

2.8.3 Call Frame.............................................................................................178

1

2.8.3.1 Red Zone........................................................................................180

1

1

2.9 Example, Statistical Function 1 (leaf)...........................................................180

1

1

2.9.1 Caller.....................................................................................................181

2.9.2 Callee.....................................................................................................181

2.10 Example, Statistical Function2 (non-leaf)...................................................183

1

1

2.10.1 Caller...................................................................................................183

2.10.2 Callee...................................................................................................184

1

1

1

2.11 Stack-Based Local Variables......................................................................187

2.12 Summary.....................................................................................................190

2.13 Exercises......................................................................................................192

1

1

2.13.1 Quiz Questions....................................................................................192

2.13.2 Suggested Projects...............................................................................193

1

3.0 System Services...............................................................................................197

1

1

1

3.1 Calling System Services................................................................................197

3.2 Newline Character.........................................................................................198

3.3 Console Output..............................................................................................199

1

3.3.1 Example, Console Output......................................................................200

3.4 Console Input................................................................................................203

3.4.1 Example, Console Input........................................................................204

3.5 File Open Operations....................................................................................208

1

1

1

1

1

3.5.1 File Open...............................................................................................209

3.5.2 File Open/Create....................................................................................210

1

1

1

3.6 File Read.......................................................................................................211

3.7 File Write.......................................................................................................211

3.8 File Operations Examples.............................................................................212

1

3.8.1 Example, File Write...............................................................................212

Page viii

Table of Contents

1

3.8.2 Example, File Read...............................................................................217

1

3.9 Exercises........................................................................................................223

1

1

3.9.1 Quiz Questions......................................................................................223

3.9.2 Suggested Projects.................................................................................223

1

4.0 Multiple Source Files......................................................................................225

1

1

4.1 Extern Statement...........................................................................................225

4.2 Example, Sum and Average..........................................................................226

1

1

1

4.2.1 Assembly Main......................................................................................226

4.2.2 Function Source.....................................................................................228

4.2.3 Assemble and Link................................................................................229

1

1

4.3 Interfacing with a High-Level Language......................................................230

1

1

4.3.1 Example, C++ Main / Assembly Function............................................230

4.3.2 Compile, Assemble, and Link...............................................................232

4.4 Exercises........................................................................................................232

1

1

4.4.1 Quiz Questions......................................................................................233

4.4.2 Suggested Projects.................................................................................233

1

5.0 Stack Buffer Overflow....................................................................................235

1

1

1

1

5.1 Understanding a Stack Buffer Overflow.......................................................236

5.2 Code to Inject................................................................................................237

5.3 Code Injection...............................................................................................240

5.4 Code Injection Protections............................................................................241

1

1

1

5.4.1 Data Stack Smashing Protector (or Canaries).......................................242

5.4.2 Data Execution Prevention....................................................................242

5.4.3 Data Address Space Layout Randomization.........................................242

1

5.5 Exercises........................................................................................................242

1

1

5.5.1 Quiz Questions......................................................................................242

5.5.2 Suggested Projects.................................................................................243

1

1

6.0 Command Line Arguments............................................................................245

1

1

1

1

1

6.1 Parsing Command Line Arguments..............................................................245

6.2 High-Level Language Example....................................................................246

6.3 Argument Count and Argument Vector Table..............................................247

6.4 Assembly Language Example.......................................................................248

6.5 Exercises........................................................................................................252

1

1

6.5.1 Quiz Questions......................................................................................252

6.5.2 Suggested Projects.................................................................................252

7.0 Input/Output Buffering..................................................................................255

Page ix

Table of Contents

1

1

1

7.1 Why Buffer?..................................................................................................255

7.2 Buffering Algorithm......................................................................................257

7.3 Exercises........................................................................................................260

1

1

7.3.1 Quiz Questions......................................................................................260

7.3.2 Suggested Projects.................................................................................261

1

8.0 Floating-Point Instructions............................................................................263

1

1

1

1

1

8.1 Floating-Point Values....................................................................................263

8.2 Floating-Point Registers................................................................................264

8.3 Data Movement.............................................................................................264

8.4 Integer / Floating-Point Conversion Instructions..........................................266

8.5 Floating-Point Arithmetic Instructions..........................................................268

1

1

1

1

1

8.5.1 Floating-Point Addition.........................................................................268

8.5.2 Floating-Point Subtraction.....................................................................270

8.5.3 Floating-Point Multiplication................................................................271

8.5.4 Floating-Point Division.........................................................................273

8.5.5 Floating-Point Square Root...................................................................275

1

8.6 Floating-Point Control Instructions...............................................................277

1

8.6.1 Floating-Point Comparison...................................................................277

1

1

1

1

8.7 Floating-Point Calling Conventions..............................................................281

8.8 Example Program, Sum and Average...........................................................281

8.9 Example Program, Absolute Value...............................................................283

8.10 Exercises......................................................................................................284

1

1

8.10.1 Quiz Questions....................................................................................284

8.10.2 Suggested Projects...............................................................................284

1

2

9.0 Parallel Processing..........................................................................................287

1

1

9.1 Distributed Computing..................................................................................288

9.2 Multiprocessing.............................................................................................288

1

1

9.2.1 POSIX Threads......................................................................................289

9.2.2 Race Conditions.....................................................................................290

1

9.3 Exercises........................................................................................................293

1

1

9.3.1 Quiz Questions......................................................................................293

9.3.2 Suggested Projects.................................................................................294

0.0 Interrupts.........................................................................................................295

20.1 Multi-user Operating System........................................................................295

2

2

0.1.1 Interrupt Classification..........................................................................296

0.1.2 Interrupt Timing....................................................................................296

2

0.1.2.1 Asynchronous Interrupts................................................................296

Page x

Table of Contents

2

0.1.2.2 Synchronous Interrupts..................................................................296

2

0.1.3 Interrupt Categories...............................................................................297

2

0.1.3.1 Hardware Interrupt.........................................................................297

0.1.3.1.1 Exceptions..............................................................................297

0.1.3.2 Software Interrupts........................................................................298

2

2

2

2

0.2 Interrupt Types and Levels............................................................................298

2

2

0.2.1 Interrupt Types......................................................................................298

0.2.2 Privilege Levels.....................................................................................299

0.3 Interrupt Processing.......................................................................................300

2

2

0.3.1 Interrupt Service Routine (ISR).............................................................300

0.3.2 Processing Steps....................................................................................301

2

2

2

2

2

0.3.2.1 Suspension.....................................................................................301

0.3.2.2 Obtaining ISR Address..................................................................301

0.3.2.3 Jump to ISR...................................................................................301

0.3.2.4 Suspension Execute ISR................................................................302

0.3.2.5 Resumption....................................................................................302

2

2

0.4 Suspension Interrupt Processing Summary...................................................302

0.5 Exercises........................................................................................................304

2

2

0.5.1 Quiz Questions......................................................................................304

0.5.2 Suggested Projects.................................................................................305

2

2

1.0 Appendix A – ASCII Table............................................................................307

2.0 Appendix B – Instruction Set Summary.......................................................309

2

2

2

2

2

2

2

2

2

2

2

2

2.1 Notation.........................................................................................................309

2.2 Data Movement Instructions.........................................................................310

2.3 Data Conversion instructions........................................................................310

2.4 Integer Arithmetic Instructions.....................................................................311

2.5 Logical, Shift, and Rotate Instructions..........................................................313

2.6 Control Instructions.......................................................................................315

2.7 Stack Instructions..........................................................................................317

2.8 Function Instructions.....................................................................................317

2.9 Floating-Point Data Movement Instructions.................................................317

2.10 Floating-Point Data Conversion Instructions..............................................318

2.11 Floating-Point Arithmetic Instructions........................................................319

2.12 Floating-Point Control Instructions.............................................................323

2

3.0 Appendix C – System Services.......................................................................325

2

2

3.1 Return Codes.................................................................................................325

3.2 Basic System Services...................................................................................325

Page xi

Table of Contents

2

2

3.3 File Modes.....................................................................................................327

3.4 Error Codes...................................................................................................328

2

4.0 Appendix D – Quiz Question Answers..........................................................331

2

2

2

2

2

2

2

2

2

2

2

2

2

2

2

2

2

2

2

2

4.1 Quiz Question Answers, Chapter 1...............................................................331

4.2 Quiz Question Answers, Chapter 2...............................................................331

4.3 Quiz Question Answers, Chapter 3...............................................................332

4.4 Quiz Question Answers, Chapter 4...............................................................334

4.5 Quiz Question Answers, Chapter 5...............................................................335

4.6 Quiz Question Answers, Chapter 6...............................................................336

4.7 Quiz Question Answers, Chapter 7...............................................................337

4.8 Quiz Question Answers, Chapter 8...............................................................340

4.9 Quiz Question Answers, Chapter 9...............................................................341

4.10 Quiz Question Answers, Chapter 10...........................................................341

4.11 Quiz Question Answers, Chapter 11...........................................................342

4.12 Quiz Question Answers, Chapter 12...........................................................342

4.13 Quiz Question Answers, Chapter 13...........................................................343

4.14 Quiz Question Answers, Chapter 14...........................................................343

4.15 Quiz Question Answers, Chapter 15...........................................................344

4.16 Quiz Question Answers, Chapter 16...........................................................344

4.17 Quiz Question Answers, Chapter 17...........................................................345

4.18 Quiz Question Answers, Chapter 18...........................................................345

4.19 Quiz Question Answers, Chapter 19...........................................................346

4.20 Quiz Question Answers, Chapter 20...........................................................346

2

5.0 Alphabetical Index..........................................................................................349

Page xii

Table of Contents

Illustration Index

Illustration 1: Computer Architecture................................................................................7

Illustration 2: CPU Block Diagram..................................................................................15

Illustration 3: Little-Endian Data Layout.........................................................................16

Illustration 4: General Memory Layout...........................................................................17

Illustration 5: Memory Hierarchy....................................................................................18

Illustration 6: Overview: Assemble, Link, Load.............................................................42

Illustration 7: Little-Endian, Multiple Variable Data Layout..........................................44

Illustration 8: Linking Multiple Files...............................................................................49

Illustration 9: Initial Debugger Screen.............................................................................56

Illustration 10: Debugger Screen with Breakpoint Set....................................................58

Illustration 11: Debugger Screen with Green Arrow.......................................................59

Illustration 12: DDD Command Bar................................................................................60

Illustration 13: Register Window.....................................................................................61

Illustration 14: MOV Instruction Overview....................................................................73

Illustration 15: Integer Multiplication Overview.............................................................90

Illustration 16: Integer Division Overview......................................................................99

Illustration 17: Logical Operations................................................................................105

Illustration 18: Logical Shift Overview.........................................................................107

Illustration 19: Logical Shift Operations.......................................................................107

Illustration 20: Arithmetic Left Shift.............................................................................109

Illustration 21: Arithmetic Right Shift...........................................................................109

Illustration 22: Process Memory Layout........................................................................148

Illustration 23: Process Memory Layout Example........................................................149

Illustration 24: Stack Frame Layout..............................................................................179

Illustration 25: Stack Frame Layout with Red Zone......................................................180

Illustration 26: Stack Call Frame Example....................................................................236

Illustration 27: Stack Call Frame Corruption.................................................................241

Illustration 28: Argument Vector Layout......................................................................248

Illustration 29: Privilege Levels.....................................................................................300

Illustration 30: Interrupt Processing Overview..............................................................303

Page xiii

Table of Contents

Page xiv

Chapter

1

If you give someone a program, you will

frustrate them for a day; if you teach them

to program, you will frustrate them for a

lifetime.

1.0 Introduction

The purpose of this text is to provide a reference for University level assembly language

and systems programming courses. Specifically, this text addresses the x86-64

1

instruction set for the popular x86-64 class of processors using the Ubuntu 64-bit

Operating System (OS). While the provided code and various examples should work

under any Linux-based 64-bit OS, they have only been tested under Ubuntu 14.04 LTS

(64-bit).

The x86-64 is a Complex Instruction Set Computing (CISC ) CPU design. This refers

2

to the internal processor design philosophy. CISC processors typically include a wide

variety of instructions (sometimes overlapping), varying instructions sizes, and a wide

range of addressing modes. The term was retroactively coined in contrast to Reduced

Instruction Set Computer (RISC ).

3

1

.1 Prerequisites

It must be noted that the text is not geared toward learning how to program. It is

assumed that the reader has already become proficient in a high-level programming

language. Specifically, the text is generally geared toward a compiled, C-based high-

level language such as C, C++, or Java. Many of the explanations and examples assume

the reader is already familiar with programming concepts such as declarations,

arithmetic operations, control structures, iteration, function calls, functions, indirection

(i.e., pointers), and variable scoping issues.

Additionally, the reader should be comfortable using a Linux-based operating system

including using the command line. If the reader is new to Linux, the Additional

References section has links to some useful documentation.

1

2

3

For more information, refer to: http://en.wikipedia.org/wiki/X86-64

For more information, refer to: http://en.wikipedia.org/wiki/Complex_instruction_set_computing

For more information, refer to: http://en.wikipedia.org/wiki/Reduced_instruction_set_computing

Page 1

Chapter 1.0 ◄ Introduction

1

.2 What is Assembly Language

The typical question asked by students is 'why learn assembly?'. Before addressing that

question, let's clarify what exactly assembly language is.

Assembly language is machine specific. For example, code written for an x86-64

processor will not run on a different processor such as a RISC processor (popular in

tablets and smart-phones).

Assembly language is a “low-level” language and provides the basic instructional

interface to the computer processor. Assembly language is as close to the processor as

you can get as a programmer. Programs written in a high-level language are translated

into assembly language in order for the processor to execute the program. The high-

level language is an abstraction between the language and the actual processor

instructions. As such, the idea that “assembly is dead” is nonsense.

Assembly language gives you direct control of the system's resources. This involves

setting processor registers, accessing memory locations, and interfacing with other

hardware elements. This requires a significantly deeper understanding of exactly how

the processor and memory work.

1

.3 Why Learn Assembly Language

The goal of this text is to provide a comprehensive introduction to programming in

assembly language. The reasons for learning assembly language are more about

understanding how a computer works instead of developing large programs. Since

assembly language is machine specific, the lack of portability is very limiting for

programming projects.

The process of actually learning assembly language involves writing non-trivial

programs to perform specific low-level actions including arithmetic operations, function

calls, using stack-dynamic local variables, and operating system interaction for activities

such as input/output. Just looking at small assembly language programs will not be

enough.

In the long run, learning the underlying principles, including assembly language, is what

makes the difference between a coding technician unable to cope with changing

languages and a computer scientist who is able to adapt to the ever-changing

technologies.

The following sections provide some detail on the various, more specific reasons for

learning assembly language.

Page 2

Chapter 1.0 ◄ Introduction

1

.3.1 Gain a Better Understanding of Architecture Issues

Learning and spending some time working at the assembly language level provides a

richer understanding of the underlying computer architecture. This includes the basic

instruction set, processor registers, memory addressing, hardware interfacing, and

Input/Output. Since ultimately all programs execute at this level, knowing the

capabilities of assembly language provides useful insights into what is possible, what is

easy, and what might be more difficult or slower.

1

.3.2 Understanding the Tool Chain

The tool chain is the name for the process of taking code written by a human and

converting it into something that the computer can directly execute. This includes the

compiler, or assembler in our case, the linker, the loader, and the debugger. In reference

to compiling, beginning programmers are told “just do this” with little explanation of the

complexity involved in the process. Working at the low-level can help provide the basis

for understanding and appreciating the details of the tool chain.

1

.3.3 Improve Algorithm Development Skills

Working with assembly language and writing low-level programs helps programmers

improve algorithm development skills by practicing with a language that requires more

thought and more attention to detail. In the highly unlikely event that a program does

not work the first time, debugging assembly language also provides practice debugging

and requires a more nuanced approach since just adding a bunch of output statements is

more difficult at the assembly language level. This typically involves a more

comprehensive use of a debugger which is a useful skill for any programmer.

1

.3.4 Improve Understanding of Functions/Procedures

Working with assembly language provides a greatly improved understanding of how

function/procedure calls work. This includes the contents and structure of the function

call frame, also referred to as the activation record. Depending on the specific instance,

the activation record might include stack-based arguments, preserved registers, and/or

stack dynamic local variables. There are some significant implementation and security

implications regarding stack dynamic local variables that are best understood working at

a low-level. Due to the security implications, it would be appropriate to remind readers

to always use their powers for good. Additionally, use of the stack and the associated

call frame is the basis for recursion and understanding the fairly straightforward

implementation of recursive functions.

Page 3

Chapter 1.0 ◄ Introduction

1

.3.5 Gain an Understanding of I/O Buffering

In a high-level language, input/output instructions and the associated buffering

operations can appear magical. Working at the assembly language level and performing

some low-level input/output operations provides a more detailed understanding of how

input/output and buffering really works. This includes the differences between

interactive input/output, file input/output, and the associated operating system services.

1

.3.6 Understand Compiler Scope

Programming with assembly language, after having already learned a high-level

language, helps ensure programmers understand the scope and capabilities of a

compiler. Specifically, this means learning what the compiler does and does not do in

relation to the computer architecture.

1

.3.7 Introduction Multi-processing Concepts

This text will also provide a brief introduction to multi-processing concepts. The

general concepts of distributed and multi-core programming are presented with the

focus being placed on shared memory, threaded processing. It is the author’s belief that

truly understanding the subtle issues associated with threading such as shared memory

and race conditions is most easily understood at the low-level.

1

.3.8 Introduction Interrupt Processing Concepts

The underlying fundamental mechanism in which modern multi-user computers work is

based on interrupts. Working at a low-level is the best place to provide an introduction

to the basic concepts associated with interrupt handling, interrupt service handles, and

vector interrupts.

1

.4 Additional References

Some key references for additional information are noted in the following sections.

These references provide much more extensive and detailed information.

If any of these locations change, a web search will be able to find the new location.

Page 4

Chapter 1.0 ◄ Introduction

1

.4.1 Ubuntu References

There is significant documentation available for the Ubuntu OS. The principal user

guide is as follows:

◦

◦

Ubuntu Community Wiki

Getting Started with Ubuntu 16.04

In addition, there are many other sites dedicated to providing help using Ubuntu (or

other Linux-based OS's).

1

.4.2 BASH Command Line References

BASH is the default shell for Ubuntu. The reader should be familiar with basic

command line operations. Some additional references are as follows:

◦

◦

Linux Command Line (on-line Tutorial and text)

An Introduction to the Linux Command Shell For Beginners (pdf)

In addition, there are many other sites dedicated to providing information regarding the

BASH command shell.

1

.4.3 Architecture References

Some key references published by Intel provide a detailed technical description of the

architecture and programming environment of Intel processors supporting IA-32 and

Intel 64 Architectures.

◦

◦

◦

Intel® 64 and IA-32 Architectures Software Developer's Manual: Basic

Architecture.

Intel 64 and IA-32 Architectures Software Developer's Manual: Instruction

Set Reference.

Intel 64 and IA-32 Architectures Software Developer's Manual: System

Programming Guide.

If the embedded links do not work, an Internet search can help find the new location.

1

.4.4 Tool Chain References

The tool chain includes the assembler, linker, loader, and debugger. Chapter 5, Tool

Chain, provides an overview of the tool chain being used in this text. The following

references provide more detailed information and documentation.

Page 5

Chapter 1.0 ◄ Introduction

.4.4.1 YASM References

1

The YASM assembler is an open source assembler commonly available on Linux-based

systems. The YASM references are as follows:

◦

◦

Yasm Web Site

Yasm Documentation

Additional information regarding YASM may be available a number of assembly

language sites and can be found through an Internet search.

1.4.4.2 DDD Debugger References

The DDD debugger is an open source debugger capable of supporting assembly

language.

◦

◦

DDD Web Site

DDD Documentation

Additional information regarding DDD may be at a number of assembly language sites

and can be found through an Internet search.

Page 6

Chapter

2

Warning, keyboard not found. Press enter

to continue.

2.0 Architecture Overview

This chapter presents a basic, general overview of the x86-64 architecture. For a more

detailed explanation, refer to the additional references noted in Chapter 1, Introduction.

2

.1 Architecture Overview

The basic components of a computer include a Central Processing Unit (CPU), Primary

Storage or Random Access Memory (RAM), Secondary Storage, Input/Output devices

(e.g., screen, keyboard, mouse), and an interconnection referred to as the Bus.

A very basic diagram of the computer architecture is as follows:

Primary Storage

Random Access

Memory (RAM)

CPU

BUS

(Interconnection)

Screen / Keyboard /

Mouse

Secondary Storage

(i.e., SSD / Disk Drive /

Other Storage Media)

Illustration 1: Computer Architecture

Page 7

Chapter 2.0 ◄ Architecture Overview

The architecture is typically referred to as the Von Neumann Architecture , or the

4

Princeton architecture, and was described in 1945 by the mathematician and physicist

John von Neumann.

Programs and data are typically stored on secondary storage (e.g., disk drive or solid

state drive). When a program is executed, it must be copied from secondary storage into

the primary storage or main memory (RAM). The CPU executes the program from

primary storage or RAM.

Primary storage or main memory is also referred to as volatile memory since when

power is removed, the information is not retained and thus lost. Secondary storage is

referred to as non-volatile memory since the information is retained when powered off.

For example, consider storing a term paper on secondary storage (i.e., disk). When the

user starts to write or edit the term paper, it is copied from the secondary storage

medium into primary storage (i.e., RAM or main memory). When done, the updated

version is typically stored back to the secondary storage (i.e., disk). If you have ever

lost power while editing a document (assuming no battery or uninterruptible power

supply), losing the unsaved work will certainly clarify the difference between volatile

and non-volatile memory.

2

.2 Data Storage Sizes

The x86-64 architecture supports a specific set of data storage size elements, all based

on powers of two. The supported storage sizes are as follows:

Storage

Size (bits) Size (bytes)

Byte

8-bits

16-bits

32-bits

64-bits

128-bits

1 byte

2 bytes

4 bytes

8 bytes

16 bytes

Word

Double-word

Quadword

Double quadword

Lists or arrays (sets of memory) can be reserved in any of these types.

These storage sizes have a direct correlation to variable declarations in high-level

languages (e.g., C, C++, Java, etc.).

4

For more information, refer to: http://en.wikipedia.org/wiki/Von_Neumann_architecture

Page 8

Chapter 2.0 ◄ Architecture Overview

For example, C/C++ declarations are mapped as follows:

C/C++ Declaration Storage

Size (bits) Size (bytes)

char

Byte

8-bits

16-bits

32-bits

32-bits

64-bits

64-bits

64-bits

64-bits

32-bits

64-bits

1 byte

2 bytes

4 bytes

4 bytes

8 bytes

8 bytes

8 bytes

8 bytes

4 bytes

8 bytes

short

Word

int

Double-word

Double-word

Quadword

Quadword

Quadword

Quadword

Double-word

Quadword

unsigned int

long5

long long

char *

int *

float

double

The asterisk indicates an address variable. For example, int * means the address of

an integer. Other high-level languages typically have similar mappings.

2

.3 Central Processing Unit

The Central Processing Unit

6

(CPU) is typically referred to as the “brains” of the

computer since that is where the actual calculations are performed. The CPU is housed

in a single chip, sometimes called a processor, chip, or die

such CPU.

7

. The cover image shows one

The CPU chip includes a number of functional units, including the Arithmetic Logic

Unit (ALU) which is the part of the chip that actually performs the arithmetic and

logical calculations. In order to support the ALU, processor registers and cache10

8

9

memory are also included “on the die” (term for inside the chip). The CPU registers and

cache memory are described in subsequent sections.

5

6

7

8

9

1

Note, the 'long' type declaration is compiler dependent. Type shown is for gcc and g++ compilers.

For more information, refer to: http://en.wikipedia.org/wiki/Central_processing_unit

For more information, refer to: http://en.wikipedia.org/wiki/Die_(integrated_circuit)

For more information, refer to: http://en.wikipedia.org/wiki/Arithmetic_logic_unit

For more information, refer to: http://en.wikipedia.org/wiki/Processor_register

0 For more information, refer to: http://en.wikipedia.org/wiki/Cache_(computing)

Page 9

Chapter 2.0 ◄ Architecture Overview

It should be noted that the internal design of a modern processor is quite complex. This

section provides a very simplified, high-level view of some key functional units within a

CPU. Refer to the footnotes or additional references for more information.

2

.3.1 CPU Registers

A CPU register, or just register, is a temporary storage or working location built into the

CPU itself (separate from memory). Computations are typically performed by the CPU

using registers.

2.3.1.1 General Purpose Registers (GPRs)

There are sixteen, 64-bit General Purpose Registers (GPRs). The GPRs are described in

the following table. A GPR register can be accessed with all 64-bits or some portion or

subset accessed.

6

4-bit register Lowest

Lowest

16-bits

Lowest

8-bits

3

2-bits

eax

ebx

ecx

edx

esi

edi

ebp

esp

r8d

r9d

rax

rbx

rcx

rdx

rsi

rdi

rbp

rsp

r8

ax

bx

al

bl

cx

cl

dx

dl

si

sil

dil

bpl

spl

r8b

r9b

r10b

r11b

r12b

r13b

r14b

r15b

di

bp

sp

r8w

r9w

r10w

r11w

r12w

r13w

r14w

r15w

r9

r10

r11

r12

r13

r14

r15

r10d

r11d

r12d

r13d

r14d

r15d

Page 10

Chapter 2.0 ◄ Architecture Overview

Additionally, some of the GPR registers are used for dedicated purposes as described in

the later sections.

When using data element sizes less than 64-bits (i.e., 32-bit, 16-bit, or 8-bit), the lower

portion of the register can be accessed by using a different register name as shown in the

table.

For example, when accessing the lower portions of the 64-bit rax register, the layout is

as follows:

←

eax

→

→

←

ax

rax =

ah al

As shown in the diagram, the first four registers, rax, rbx, rcx, and rdx also allow the

bits 8-15 to be accessed with the ah, bh, ch, and dh register names. With the exception

of ah, these are provided for legacy support and will not be used in this text.

The ability to access portions of the register means that, if the quadword rax register is

set to 50,000,000,000 (fifty billion), the rax register would contain the following value

10

in hex.

rax = 0000 000B A43B 7400

If a subsequent operation sets the word ax register to 50,000 (fifty thousand, which is

10

C350 ), the rax register would contain the following value in hex.

16

rax = 0000 000B A43B C350

In this case, when the lower 16-bit ax portion of the 64-bit rax register is set, the upper

8-bits are unaffected. Note the change in ax (from 7400 to C350 ).

4

16

16

If a subsequent operation sets the byte sized al register to 50 (fifty, which is 32 ), the

10

16

rax register would contain the following value in hex.

rax = 0000 000B A43B C332

When the lower 8-bit al portion of the 64-bit rax register is set, the upper 56-bits are

unaffected. Note the change in al (from 50 to 32 ).

16

16

For 32-bit register operations, the upper 32-bits is cleared (set to zero). Generally, this

is not an issue since operations on 32-bit registers do not use the upper 32-bits of the

register. For unsigned values, this can be useful to convert from 32-bits to 64-bits.

Page 11

Chapter 2.0 ◄ Architecture Overview

However, this will not work for signed conversions from 32-bit to 64-bit values.

Specifically, it will potentially provide incorrect results for negative values. Refer to

Chapter 3, Data Representation for additional information regarding the representation

of signed values.

2.3.1.2 Stack Pointer Register (RSP)

One of the CPU registers, rsp, is used to point to the current top of the stack. The rsp

register should not be used for data or other uses. Additional information regarding the

stack and stack operations is provided in Chapter 9, Process Stack.

2.3.1.3 Base Pointer Register (RBP)

One of the CPU registers, rbp, is used as a base pointer during function calls. The rbp

register should not be used for data or other uses. Additional information regarding the

functions and function calls is provided in Chapter 12, Functions.

2.3.1.4 Instruction Pointer Register (RIP)

In addition to the GPRs, there is a special register, rip, which is used by the CPU to

point to the next instruction to be executed. Specifically, since the rip points to the

next instruction, that means the instruction being pointed to by rip, and shown in the

debugger, has not yet been executed. This is an important distinction which can be

confusing when reviewing code in a debugger.

2.3.1.5 Flag Register (rFlags)

The flag register, rFlags, is used for status and CPU control information. The rFlag

register is updated by the CPU after each instruction and not directly accessible by

programs. This register stores status information about the instruction that was just

executed. Of the 64-bits in the rFlag register, many are reserved for future use.

The following table shows some of the status bits in the flag register.

Name Symbol

Bit Use

Carry

Parity

Adjust

CF

PF

AF

0

2

4

Used to indicate if the previous operation

resulted in a carry.

Used to indicate if the last byte has an even

number of 1's (i.e., even parity).

Used to support Binary Coded Decimal

operations.

Page 12

Chapter 2.0 ◄ Architecture Overview

Zero

Sign

ZF

SF

6

7

Used to indicate if the previous operation

resulted in a zero result.

Used to indicate if the result of the

previous operation resulted in a 1 in the

most significant bit (indicating negative in

the context of signed data).

Direction DF

Overflow OF

10 Used to specify the direction (increment or

decrement) for some string operations.

11 Used to indicate if the previous operation

resulted in an overflow.

There are a number of additional bits not specified in this text. More information can be

obtained from the additional references noted in Chapter 1, Introduction.

2.3.1.6 XMM Registers

There are a set of dedicated registers used to support 64-bit and 32-bit floating-point

operations and Single Instruction Multiple Data (SIMD) instructions. The SIMD

instructions allow a single instruction to be applied simultaneously to multiple data

items. Used effectively, this can result in a significant performance increase. Typical

applications include some graphics processing and digital signal processing.

The XMM registers as follows:

1

28-bit Registers

xmm0

xmm1

xmm2

xmm3

xmm4

xmm5

xmm6

xmm7

xmm8

xmm9

Page 13

Chapter 2.0 ◄ Architecture Overview

xmm10

xmm11

xmm12

xmm13

xmm14

xmm15

Note, some of the more recent X86-64 processors support 256-bit XMM registers. This

will not be an issue for the programs in this text.

Additionally, the XMM registers are used to support the Streaming SIMD Extensions

(SSE). The SSE instructions are out of the scope of this text. More information can be

obtained from the Intel references (as noted in Chapter 1, Introduction).

2

.3.2 Cache Memory

Cache memory is a small subset of the primary storage or RAM located in the CPU

chip. If a memory location is accessed, a copy of the value is placed in the cache.

Subsequent accesses to that memory location that occur in quick succession are

retrieved from the cache location (internal to the CPU chip). A memory read involves

sending the address via the bus to the memory controller, which will obtain the value at

the requested memory location, and send it back through the bus. Comparatively, if a

value is in cache, it would be much faster to access that value.

A cache hit occurs when the requested data can be found in a cache, while a cache miss

occurs when it cannot. Cache hits are served by reading data from the cache, which is

faster than reading from main memory. The more requests that can be served from

cache, the faster the system will typically perform. Successive generations of CPU

chips have increased cache memory and improved cache mapping strategies in order to

improve overall performance.

Page 14

Chapter 2.0 ◄ Architecture Overview

A block diagram of a typical CPU chip configuration is as follows:

CPU Chip

Core 0

Core 1

L1 Cache

L1 Cache

L2 Cache

BUS

Illustration 2: CPU Block Diagram

Current chip designs typically include an L1 cache per core and a shared L2 cache.

Many of the newer CPU chips will have an additional L3 cache.

As can be noted from the diagram, all memory accesses travel through each level of

cache. As such, there is a potential for multiple, duplicate copies of the value (CPU

register, L1 cache, L2 cache, and main memory). This complication is managed by the

CPU and is not something the programmer can change. Understanding the cache and

associated performance gain is useful in understanding how a computer works.

2

.4 Main Memory

Memory can be viewed as a series of bytes, one after another. That is, memory is byte

addressable. This means each memory address holds one byte of information. To store

a double-word, four bytes are required which use four memory addresses.

Page 15

Chapter 2.0 ◄ Architecture Overview

Additionally, architecture is little-endian. This means that the Least Significant Byte

(LSB) is stored in the lowest memory address. The Most Significant Byte (MSB) is

stored in the highest memory location.

For a double-word (32-bits), the MSB and LSB are allocated as shown below.

3

1

30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10

9

8

7

6

5

4

3

2

1

0

MSB

LSB

For example, assuming the value of, 5,000,000 (004C4B40 ), is to be placed in a

10

16

double-word variable named var1.

For a little-endian architecture, the memory picture would be as follows:

variable

name

value

Address

(in hex)

?

0100100C

0100100B

0100100A

01001009

01001008

00

4C

4B

var1 →

40

?

01001007

Illustration 3: Little-Endian Data Layout

Based on the little-endian architecture, the LSB is stored in the lowest memory address

and the MSB is stored in the highest memory location.

Page 16

Chapter 2.0 ◄ Architecture Overview

2

.5 Memory Layout

The general memory layout for a program is as shown:

high memory

stack

.

.

.

heap

BSS – uninitialized data

data

text (code)

reserved

low memory

Illustration 4: General Memory Layout

The reserved section is not available to user programs. The text (or code) section is

where the machine language11 (i.e., the 1's and 0's that represent the code) is stored. The

data section is where the initialized data is stored. This includes declared variables that

have been provided an initial value at assemble-time. The uninitialized data section,

typically called BSS section, is where declared variables that have not been provided an

initial value are stored. If accessed before being set, the value will not be meaningful.

The heap is where dynamically allocated data will be stored (if requested). The stack

starts in high memory and grows downward.

Later sections will provide additional detail for the text and data sections.

2

.6 Memory Hierarchy

In order to fully understand the various different memory levels and associated usage, it

is useful to review the memory hierarchy12. In general terms, faster memory is more

expensive and slower memory blocks are less expensive. The CPU registers are small,

1

1

1 For more information, refer to: http://en.wikipedia.org/wiki/Machine_code

2 For more information, refer to: http://en.wikipedia.org/wiki/Memory_hierarchy

Page 17

Chapter 2.0 ◄ Architecture Overview

fast, and expensive. Secondary storage devices such as disk drives and Solid State

Drives (SSD's) are larger, slower, and less expensive. The overall goal is to balance

performance with cost.

An overview of the memory hierarchy is as follows:

Smaller, faster,

and more

expensive

CPU

Registers

Cache

Primary Storage

Main Memory (RAM)

Secondary Storage

(disk drives, SSD's, etc.)

Tertiary Storage

(remote storage, optical, backups, etc.)

Larger, slower,

and less expensive

Illustration 5: Memory Hierarchy

Where the top of the triangle represents the fastest, smallest, and most expensive

memory. As we move down levels, the memory becomes slower, larger, and less

expensive. The goal is to use an effective balance between the small, fast, expensive

memory and the large, slower, and cheaper memory.

Page 18

Chapter 2.0 ◄ Architecture Overview

Some typical performance and size characteristics are as follows:

Memory Unit

Registers

Example Size

Typical Speed

16, 64-bit registers

~1 nanoseconds13

Cache Memory

4 - 8+ Megabytes14 ~5-60 nanoseconds

(L1 and L2)

Primary Storage

2 – 32+ Gigabytes15 ~100-150 nanoseconds

(i.e., main memory)

Secondary Storage 500 Gigabytes –

(i.e., disk, SSD's, etc.) 4+ Terabytes16

~3-15 milliseconds17

Based on this table, a primary storage access at 100 nanoseconds (100 ´ 10-9) is 30,000

times faster than a secondary storage access, at 3 milliseconds (3 ´ 10-3).

The typical speeds improve over time (and these are already out of date). The key point

is the relative difference between each memory unit is significant. This difference

between the memory units applies even as newer, faster SSDs are being utilized.

2

.7 Exercises

Below are some questions based on this chapter.

2

.7.1 Quiz Questions

Below are some quiz questions.

1

2

3

4

5

6

) Draw a picture of the Von Neumann Architecture.

) What architecture component connects the memory to the CPU?

) Where are programs stored when the computer is turned off?

) Where must programs be located when they are executing?

) How does cache memory help overall performance?

) How many bytes does a C++ integer declared with the declaration int use?

1

1

1

1

1

3 For more information, refer to: http://en.wikipedia.org/wiki/Nanosecond

4 For more information, refer to: http://en.wikipedia.org/wiki/Megabyte

5 For more information, refer to: http://en.wikipedia.org/wiki/Gigabyte

6 For more information, refer to: http://en.wikipedia.org/wiki/Terabyte

7 For more information, refer to: http://en.wikipedia.org/wiki/Millisecond

Page 19

Chapter 2.0 ◄ Architecture Overview

7

8

) On the Intel X86-64 architecture, how many bytes can be stored at each address?

) Given the 32-bit hex 004C4B40 what is the:

16

1

2

. Least Significant Byte (LSB)

. Most Significant Byte (MSB)

9

) Given the 32-bit hex 004C4B40 , show the little-endian memory layout showing

16

each byte in memory.

1

1

0) Draw a picture of the layout for the rax register.

1) How many bits does each of the following represent:

1

2

3

4

5

6

7

8

. al

. rcx

. bx

. edx

. r11

. r8b

. sil

. r14w

1

1

1

2) Which register points to the next instruction to be executed?

3) Which register points to the current top of the stack?

4) If al is set to 05 and ax is set to 0007 , eax is set to 00000020 , and rax is set

16

16

16

to 0000000000000000 , and show the final complete contents of the complete

16

rax register.

1

5) If the rax register is set to 81,985,529,216,486,895 (123456789ABCDEF ),

10

16

what are the contents of the following registers in hex?

1

2

3

4

. al

. ax

. eax

. rax

Page 20

Chapter

3

There are 10 types of people in the world;

those that understand binary and those that

don't.

3.0 Data Representation

Data representation refers to how information is stored within the computer. There is a

specific method for storing integers which is different than storing floating-point values

which is different than storing characters. This chapter presents a brief summary of the

integer, floating-point, and ASCII representation schemes.

It is assumed the reader is already generally familiar with binary, decimal, and hex

numbering systems.

It should be noted that if not specified, a number is in base-10. Additionally, a number

preceded by 0x is a hex value. For example, 19 = 19 = 13 = 0x13.

10

16

3

.1 Integer Representation

Representing integer numbers refers to how the computer stores or represents a number

in memory. The computer represents numbers in binary (1's and 0's). However, the

computer has a limited amount of space that can be used for each number or variable.

This directly impacts the size, or range, of the number that can be represented. For

example, a byte (8-bits) can be used to represent 2

8

or 256 different numbers. Those

2

56 different numbers can be unsigned (all positive) in which case we can represent any

number between 0 and 255 (inclusive). If we choose signed (positive and negative

values), then we can represent any number between -128 and +127 (inclusive).

If that range is not large enough to handle the intended values, a larger size must be

used. For example, a word (16-bits) can be used to represent 216 or 65,536 different

values, and a double-word (32-bits) can be used to represent 232 or 4,294,967,296

different numbers. So, if you wanted to store a value of 100,000 then a double-word

would be required.

Page 21

Chapter 3.0 ◄ Data Representation

As you may recall from C, C++, or Java, an integer declaration (e.g., int <variable>) is

a single double-word which can be used to represent values between -231

(−2,147,483,648) and +231 - 1 (+2,147,483,647).

The following table shows the ranges associated with typical sizes:

Size

Size Unsigned Range

Signed Range

-128 to +127

Bytes (8-bits)

Words (16-bits)

28

0 to 255

216

0 to 65,535

−32,768 to +32,767

Double-words (32-bits) 232 0 to 4,294,967,295

−2,147,483,648 to

+2,147,483,647

Quadword

264

0 to 264 - 1

0 to 2128 - 1

-(263) to 263 - 1

-(2127) to 2127 - 1

Double quadword

2128

In order to determine if a value can be represented, you will need to know the size of the

storage element (byte, word, double-word, quadword, etc.) being used and if the values

are signed or unsigned.

•

•

For representing unsigned values within the range of a given storage size,

standard binary is used.

For representing signed values within the range, two's complement is used.

Specifically, the two's complement encoding process applies to the values in the

negative range. For values within the positive range, standard binary is used.

For example, the unsigned byte range can be represented using a number line as follows:

0

255

For example, the signed byte range can also be represented using a number line as

follows:

-128

0

+127

The same concept applies to halfwords and words which have larger ranges.

Page 22

Chapter 3.0 ◄ Data Representation

Since unsigned values have a different, positive only, range than signed values, there is

overlap between the values. This can be very confusing when examining variables in

memory (with the debugger).

For example, when the unsigned and signed values are within the overlapping positive

range (0 to +127):

•

•

A signed byte representation of 12 is 0x0C

10 16

An unsigned byte representation of -12 is also 0x0C

16

10

When the unsigned and signed values are outside the overlapping range:

•

•

A signed byte representation of -15 is 0xF1

10 16

An unsigned byte representation of 241 is also 0xF1

16

10

This overlap can cause confusion unless the data types are clearly and correctly defined.

3

.1.1 Two's Complement

The following describes how to find the two's complement representation for negative

values (not positive values).

To take the two's complement of a number:

1

2

. take the one's complement (negate)

. add 1 (in binary)

The same process is used to encode a decimal value into two's complement and from

two's complement back to decimal. The following sections provide some examples.

3

.1.2 Byte Example

For example, to find the byte size (8-bits), two's complement representation of -9 and

-12.

9

(8+1) = 00001001

Step 1 11110110

Step 2 11110111

12 (8+4) = 00001100

Step 1: 11110011

11110100

-9 (in hex) =  F7

-12 (in hex) =  F4

Note, all bits for the given size, byte in this example, must be specified.

Page 23

Chapter 3.0 ◄ Data Representation

3

.1.3 Word Example

To find the word size (16-bits), two's complement representation of -18 and -40.

1

8 (16+2) = 0000000000010010

Step 1 1111111111101101

Step 2 1111111111101110

40 (32+8) = 0000000000101000

Step 1 1111111111010111

Step 2 1111111111011000

-18 (hex) =

0xFFEE

-40 (hex) =

0xFFD8

Note, all bits for the given size, words in these examples, must be specified.

3

.2 Unsigned and Signed Addition

As previously noted, the unsigned and signed representations may provide different

interpretations for the final value being represented. However, the addition and

subtraction operations are the same. For example:

2

41 11110001

7 00000111

-15 11110001

7 00000111

+

+

2

48 11111000

-8 11111000

2

48 =

F8

-8 =

F8

The final result of 0xF8 may be interpreted as 248 for unsigned representation and -8 for

a signed representation. Additionally, 0xF8 is the º (degree symbol) in the ASCII

16

table.

As such, it is very important to have a clear definition of the sizes (byte, halfword, word,

etc.) and types (signed, unsigned) of data for the operations being performed.

3

.3 Floating-point Representation

The representation issues for floating-point numbers are more complex. There are a

series of floating-point representations for various ranges of the value. For simplicity,

we will look primarily at the IEEE 754 32-bit floating-point standard.

Page 24

Chapter 3.0 ◄ Data Representation

3

.3.1 IEEE 32-bit Representation

The IEEE 754 32-bit floating-point standard is defined as follows:

3

1 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

s

biased exponent

fraction

Where s is the sign (0 => positive and 1 => negative). More formally, this can be

written as;

N = (−1)S × 1. F × 2E−127

When representing floating-point values, the first step is to convert floating-point value

into binary. The following table provides a brief reminder of how binary handles

fractional components:

2

3

22 21 20

2-1 2-2 2-3

.

..

8

4

2

1

.

1/2 1/4 1/8 ...

0

0

0

0

.

0

0

0

For example, 100.101 would be 4.625 . For repeating decimals, calculating the binary

2

10

value can be time consuming. However, there is a limit since computers have finite

storage sizes (32-bits in this example).

The next step is to show the value in normalized scientific notation in binary. This

means that the number should have a single, non-zero leading digit to the left of the

decimal point. For example, 8.125 is 1000.001 (or 1000.001 x 2

0

) and in binary

(since the decimal

point was moved three places to the left). Of course, if the number was 0.125 the

10

2

2

normalized scientific notation that would be written as 1.000001 x 2

3

10

binary would be 0.001 (or 0.001 x 2

0

) and the normalized scientific notation would be

2

2

1

.0 x 2-3 (since the decimal point was moved three places to the right). The numbers

after the leading 1, not including the leading 1, are stored left-justified in the fraction

portion of the double-word.

The next step is to calculate the biased exponent, which is the exponent from the

normalized scientific notation plus the bias. The bias for the IEEE 754 32-bit floating-

point standard is 127 . The result should be converted to a byte (8-bits) and stored in

10

the biased exponent portion of the word.

Page 25

Chapter 3.0 ◄ Data Representation

Note, converting from the IEEE 754 32-bit floating-point representation to the decimal

value is done in reverse, however leading 1 must be added back (as it is not stored in the

word). Additionally, the bias is subtracted (instead of added).

3.3.1.1 IEEE 32-bit Representation Examples

This section presents several examples of encoding and decoding floating-point

representation for reference.

3.3.1.1.1 Example → -7.7510

For example, to find the IEEE 754 32-bit floating-point representation for -7.75 :

10

Example 1: -7.75

•

•

determine sign

convert to binary

-7.75 => 1 (since negative)

-7.75

=

-0111.112

•

•

normalized scientific notation

compute biased exponent

=

1.1111 x 22

2 + 127 = 12910

1

0

10

◦

and convert to binary

= 100000012

•

•

write components in binary:

sign exponent mantissa

1

10000001 11110000000000000000000

convert to hex (split into groups of 4)

1

1

1000000111110000000000000000000

100 0000 1111 1000 0000 0000 0000 0000

C

0

F

8

0

0

0

0

•

final result: C0F8 000016

3.3.1.1.2 Example → -0.12510

For example, to find the IEEE 754 32-bit floating-point representation for -0.125 :

10

Example 2: -0.125

•

•

determine sign

convert to binary

-0.125 => 1 (since negative)

-0.125 =

-0.0012

•

•

normalized scientific notation

compute biased exponent

=

1.0 x 2-3

-310 + 12710 = 12410

= 011111002

◦

and convert to binary

•

write components in binary:

sign exponent mantissa

1

01111100 00000000000000000000000

Page 26

Chapter 3.0 ◄ Data Representation

•

•

convert to hex (split into groups of 4)

1

1

0111110000000000000000000000000

011 1110 0000 0000 0000 0000 0000 0000

B

E

0

0

0

0

0

0

final result: BE00 000016

3.3.1.1.3 Example → 4144000016

For example, given the IEEE 754 32-bit floating-point representation 41440000 find

16

the decimal value:

Example 3: 4144000016

•

•

•

convert to binary

0

100 0001 0100 0100 0000 0000 0000 00002

split into components

0

10000010 100010000000000000000002

determine exponent

100000102 = 13010

130 - 127 = 310

◦

and remove bias

10

10

•

•

determine sign

write result

0 => positive

+1.10001 x 23 = +1100.01 = +12.25

3

.3.2 IEEE 64-bit Representation

The IEEE 754 64-bit floating-point standard is defined as follows:

6

3

62

52 51

0

s

biased exponent

fraction

The representation process is the same, however the format allows for an 11-bit biased

exponent (which support large and smaller values). The 11-bit biased exponent uses a

bias of ±1023.

3

.3.3 Not a Number (NaN)

When a value is interpreted as a floating-point value and it does not conform to the

defined standard (either for 32-bit or 64-bit), then it cannot be used as a floating-point

value. This might occur if an integer representation is treated as a floating-point

representation or a floating-point arithmetic operation (add, subtract, multiply, or divide)

results in a value that is too large or too small to be represented. The incorrect format or

unrepresentable number is referred to as a NaN which is an abbreviation for not a

number.

Page 27

Chapter 3.0 ◄ Data Representation

3

.4 Characters and Strings

In addition to numeric data, symbolic data is often required. Symbolic or non-numeric

data might include an important message such as “Hello World”18 a common greeting

for first programs. Such symbols are well understood by English language speakers.

Computer memory is designed to store and retrieve numbers. Consequently, the

symbols are represented by assigning numeric values to each symbol or character.

3

.4.1 Character Representation

In a computer, a character19 is a unit of information that corresponds to a symbol such as

a letter in the alphabet. Examples of characters include letters, numerical digits,

common punctuation marks (such as "." or "!"), and whitespace. The general concept

also includes control characters, which do not correspond to symbols in a particular

language, but to other information used to process text. Examples of control characters

include carriage return or tab.

3.4.1.1 American Standard Code for Information Interchange

Characters are represented using the American Standard Code for Information

Interchange (ASCII20). Based on the ASCII table, each character and control character

is assigned a numeric value. When using ASCII, the character displayed is based on the

assigned numeric value. This only works if everyone agrees on common values, which

is the purpose of the ASCII table. For example, the letter “A” is defined as 65 (0x41).

10

The 0x41 is stored in computer memory, and when displayed to the console, the letter

“A” is shown. Refer to Appendix A for the complete ASCII table.

Additionally, numeric symbols can be represented in ASCII. For example, “9” is

represented as 57 (0x39) in computer memory. The “9” can be displayed as output to

10

the console. If sent to the console, the integer value 9 (0x09) would be interpreted as

10

an ASCII value which in the case would be a tab.

It is very important to understand the difference between characters (such as “2”) and

integers (such a 2 ). Characters can be displayed to the console, but cannot be used for

10

calculations. Integers can be used for calculations, but cannot be displayed to the

console (without changing the representation).

A character is typically stored in a byte (8-bits) of space. This works well since memory

is byte addressable.

1

1

2

8 For more information, refer to: http://en.wikipedia.org/wiki/”Hello,_World!”_program

9 For more information, refer to: http://en.wikipedia.org/wiki/Character_(computing)

0 For more information, refer to: http://en.wikipedia.org/wiki/ASCII

Page 28

Chapter 3.0 ◄ Data Representation

3.4.1.2 Unicode

It should be noted that Unicode21 is a current standard that includes support for different

languages. The Unicode Standard provides series of different encoding schemes (UTF-

8

, UTF-16, UTF-32, etc.) in order to provide a unique number for every character, no

matter what platform, device, application or language. In the most common encoding

scheme, UTF-8, the ASCII English text looks exactly the same in UTF-8 as it did in

ASCII. Additional bytes are used for other characters as needed. Details regarding

Unicode representation are not addressed in this text.

3

.4.2 String Representation

A string22 is a series of ASCII characters, typically terminated with a NULL. The

NULL is a non-printable ASCII control character. Since it is not printable, it can be

used to mark the end of a string.

For example, the string “Hello” would be represented as follows:

Character

ASCII Value (decimal) 72 101 108 108 111

ASCII Value (hex) 0x48 0x65 0x6C 0x6C 0x6F 0x0

“H” “e” “l” “l” “o” NULL

0

A string may consist partially or completely of numeric symbols. For example, the

string “19653” would be represented as follows:

Character

ASCII Value (decimal) 49 57 54 53 51

ASCII Value (hex) 0x31 0x39 0x36 0x35 0x33 0x0

“1” “9” “6” “5” “3” NULL

0

Again, it is very important to understand the difference between the string “19653”

(using 6 bytes) and the single integer 19,653 (which can be stored in a single word

10

which is 2 bytes).

3

.5 Exercises

Below are some questions based on this chapter.

2

2

1 For more information, refer to: http://en.wikipedia.org/wiki/Unicode

2 For more information, refer to: http://en.wikipedia.org/wiki/String_(computer_science)

Page 29

Chapter 3.0 ◄ Data Representation

3

.5.1 Quiz Questions

Below are some quiz questions.

1

) Provide the range for each of the following:

1

2

3

4

5

6

. signed byte

. unsigned byte

. signed word

. unsigned word

. signed double-word

. unsigned double-word

2

3

) Provide the decimal values of the following binary numbers:

1

2

3

4

. 00001012

. 00010012

. 00011012

. 00101012

) Provide the hex, byte size, two's complement values of the following decimal

values. Note, two hex digits expected.

1

2

3

4

. -310

. +1110

. -910

. -2110

4

) Provide the hex, word size, two's complement values of the following decimal

values. Note, four hex digits expected.

1

2

3

4

. -1710

. +1710

. -3110

. -13810

Page 30

Chapter 3.0 ◄ Data Representation

5

6

) Provide the hex, double-word size, two's complement values of the following

decimal values. Note, eight hex digits expected.

1

2

3

4

. -1110

. -2710

. +710

. -26110

) Provide the decimal values of the following hex, double-word sized, two's

complement values.

1

2

3

4

. FFFFFFFB16

. FFFFFFEA16

. FFFFFFF316

. FFFFFFF816

7

8

) Which of the following decimal values has an exact representation in binary?

1

2

3

4

5

. 0.1

. 0.2

. 0.3

. 0.4

. 0.5

) Provide the decimal representation of the following IEEE 32-bit floating-point

values.

1

2

3

4

. 0xC1440000

. 0x41440000

. 0xC0D00000

. 0xC0F00000

Page 31

Chapter 3.0 ◄ Data Representation

9

) Provide hex, IEEE 32-bit floating-point representation of the following floating-

point values.

1

2

3

4

. +11.2510

. -17.12510

. +21.87510

. -0.7510

1

0) What is the ASCII code, in hex, for each of the following characters:

1

2

3

4

5

. “A”

. “a”

. “0”

. “8”

. tab

1

1) What are the ASCII values, in hex, for each of the following strings:

1

2

3

. “World”

. “123”

. “Yes!?”

Page 32

Chapter

4

I would love to change the world, but they

won't give me the source code.

4.0 Program Format

This chapter summarizes the formatting requirements for assembly language programs.

The formatting requirements are specific to the yasm assembler. Other assemblers may

be slightly different. A complete assembly language program is presented to

demonstrate the appropriate program formatting.

A properly formatted assembly source file consists of several main parts;

•

•

•

Data section where initialized data is declared and defined.

BSS section where uninitialized data is declared.

Text section where code is placed.

The following sections summarize the basic formatting requirements. Only the basic

formatting and assembler syntax are presented. For additional information, refer to the

yasm reference manual (as noted in Chapter 1, Introduction).

4

.1 Comments

The semicolon (;) is used to note program comments. Comments (using the ;) may be

placed anywhere, including after an instruction. Any characters after the ; are ignored

by the assembler. This can be used to explain steps taken in the code or to comment out

sections of code.

4

.2 Numeric Values

Number values may be specified in decimal, hex, or octal.

When specifying hex, or base-16 values, they are preceded with a 0x. For

example, to specify 127 as hex, it would be 0x7f.

When specifying octal, or-base-8 values, they are followed by a q. For example,

Page 33

Chapter 4.0 ◄ Program Format

to specify 511 as octal, it would be 777q.

The default radix (base) is decimal, so no special notation is required for decimal (base-

1

0) numbers.

4

.3 Defining Constants

Constants are defined with equ. The general format is:

<name>    equ     <value>

The value of a constant cannot be changed during program execution.

The constants are substituted for their defined values during the assembly process. As

such, a constant is not assigned a memory location. This makes the constant more

flexible since it is not assigned a specific type/size (byte, word, double-word, etc.). The

values are subject to the range limitations of the intended use. For example, the

following constant,

SIZE equ

10000

could be used as a word or a double-word, but not a byte.

4

.4 Data Section

The initialized data must be declared in the "section .data" section. There must be a

space after the word 'section'. All initialized variables and constants are placed in this

section. Variable names must start with a letter, followed by letters or numbers,

including some special characters (such as the underscore, "_"). Variable definitions

must include the name, the data type, and the initial value for the variable.

The general format is:

<variableName>     <dataType>     <initialValue>

Refer to the following sections for a series of examples using various data types.

The supported data types are as follows:

Declaration

db

8-bit variable(s)

Page 34

Chapter 4.0 ◄ Program Format

dw

dd

16-bit variable(s)

32-bit variable(s)

64-bit variable(s)

dq

ddq

dt

128-bit variable(s) → integer

128-bit variable(s) → float

These are the primary assembler directives for initialized data declarations. Other

directives are referenced in different sections.

Initialized arrays are defined with comma separated values.

Some simple examples include:

bVar

cVar

strng

wVar

dVar

arr

db

db

db

dw

dd

dd

dd

dq

10

"H"

; byte variable

; single character

; string

; 16-bit variable

; 32-bit variable

; 3 element array

; 32-bit float

; 64-bit variable

"Hello World"

5000

50000

100, 200, 300

3.14159

1000000000

flt1

qVar

The value specified must be able to fit in the specified data type. For example, if the

value of a byte sized variables is defined as 500, it would generate an assembler error.

4

.5 BSS Section

Uninitialized data is declared in the "section .bss" section. There must be a space after

the word 'section'. All uninitialized variables are declared in this section. Variable

names start with a letter followed by letters or numbers including some special

characters (such as the underscore, "_"). Variable definitions must include the name, the

data type, and the count.

The general format is:

<variableName>    <resType>     <count>

Refer to the following sections for a series of examples using various data types.

Page 35

Chapter 4.0 ◄ Program Format

The supported data types are as follows:

Declaration

resb

resw

resd

resq

resdq

8-bit variable(s)

16-bit variable(s)

32-bit variable(s)

64-bit variable(s)

128-bit variable(s)

These are the primary assembler directives for uninitialized data declarations. Other

directives are referenced in different sections.

Some simple examples include:

bArr

wArr

dArr

qArr

resb

resw

resd

resq

10

50

100

200

; 10 element byte array

; 50 element word array

; 100 element double array

; 200 element quad array

The allocated array is not initialized to any specific value.

4

.6 Text Section

The code is placed in the "section .text" section. There must be a space after the word

section'. The instructions are specified one per line and each must be a valid instruction

'

with the appropriate required operands.

The text section will include some headers or labels that define the initial program entry

point. For example, assuming a basic program using the standard system linker, the

following declarations must be included.

global _start

_start:

No special label or directives are required to terminate the program. However, a system

service should be used to inform the operating system that the program should be

terminated and the resources, such as memory, recovered and re-utilized. Refer to the

example program in the following section.

Page 36

Chapter 4.0 ◄ Program Format

4

.7 Example Program

A very simple assembly language program is presented to demonstrate the appropriate

program formatting.

; Simple example demonstrating basic program format and layout.

;

;

Ed Jorgensen

July 18, 2014

;

;

************************************************************

Some basic data declarations

section   .data

;

-----

;  Define constants

EXIT_SUCCESS    equ     0       ; successful operation

SYS_exit        equ     60      ; call code for terminate

; -----

;  Byte (8-bit) variable declarations

bVar1           db      17

bVar2           db      9

bResult         db      0

;

-----

;  Word (16-bit) variable declarations

wVar1           dw      17000

wVar2           dw      9000

wResult         dw      0

; -----

;  Double-word (32-bit) variable declarations

dVar1           dd      17000000

dVar2           dd      9000000

dResult         dd      0

Page 37

Chapter 4.0 ◄ Program Format

;

-----

;  quadword (64-bit) variable declarations

qVar1           dq      170000000

qVar2           dq      90000000

qResult         dq      0

;

;

************************************************************

Code Section

section     .text

global _start

_start:

;

;

Performs a series of very basic addition operations

to demonstrate basic program format.

;

;

;

----------

Byte example

bResult = bVar1 + bVar2

mov     al, byte [bVar1]

add     al, byte [bVar2]

mov     byte [bResult], al

;

;

;

----------

Word example

wResult = wVar1 + wVar2

mov     ax, word [wVar1]

add     ax, word [wVar2]

mov     word [wResult], ax

;

;

;

----------

Double-word example

dResult = dVar1 + dVar2

mov     eax, dword [dVar1]

add     eax, dword [dVar2]

mov     dword [dResult], eax

Page 38

Chapter 4.0 ◄ Program Format

;

;

;

----------

Quadword example

qResult = qVar1 + qVar2

mov     rax, qword [qVar1]

add     rax, qword [qVar2]

mov     qword [qResult], rax

;

;

************************************************************

Done, terminate program.

last:

mov     rax, SYS_exit       ; Call code for exit

mov     rdi, EXIT_SUCCESS   ; Exit program with success

syscall

This example program will be referenced and further explained in the following

chapters.

4

.8 Exercises

Below are some questions based on this chapter.

4

.8.1 Quiz Questions

Below are some quiz questions.

1

2

3

4

5

6

) What is the name of the assembler being used in this chapter?

) How are comments marked in an assembly language program?

) What is the name of the section where the initialized data declared?

) What is the name of the section where the uninitialized data declared?

) What is the name of the section where the code is placed?

) What is the data declaration for each of the following variables with the given

values:

1

2

3

. byte sized variable bNum set to 1010

. word sized variable wNum set to 10,29110

. double-word sized variable dwNum set to 2,126,01010

Page 39

Chapter 4.0 ◄ Program Format

4

. quadword sized variable qwNum set to 10,000,000,00010

7

) What is the uninitialized data declaration for each of the following:

1

2

3

4

. byte sized array named bArr with 100 elements

. word sized array named wArr with 3000 elements

. double-word sized array named dwArr with 200 elements

. quadword sized array named qArr with 5000 elements

8

) What are the required declarations to signify the start of a program (in the text

section)?

Page 40

Chapter

5

There are two ways to write error-free

programs; only the third works.

5.0 Tool Chain

In general, the set of programming tools used to create a program is referred to as the

tool chain23. For the purposes of this text, the tool chain consists of the following;

•

•

•

•

Assembler

Linker

Loader

Debugger

While there are many options for the tool chain, this text uses a fairly standard set of

open source tools that work well together and fully support the x86 64-bit environment.

Each of these programming tools is explained in the following sections.

5

.1 Assemble/Link/Load Overview

In broad terms, the assemble, link, and load process is how programmer written source

files are converted into an executable program.

The human readable source file is converted into an object file by the assembler. In the

most basic form, the object file is converted into an executable file by the linker. The

loader will load the executable file into memory.

2

3 For more information, refer to: http://en.wikipedia.org/wiki/Toolchain

Page 41

Chapter 5.0 ◄ Tool Chain

An overview of the process is provided in the following diagram.

Assembly

Language

Source

File

ASSEMBLE

Other

Object

Files

Library

Routines

(if any)

List

File

Object

File

(if any)

LINK

Executable

File

Shared

Object

Files

LOAD

RAM

(if any)

Illustration 6: Overview: Assemble, Link, Load

The assemble, link, and load steps are described in more detail in the following sections.

Page 42

Chapter 5.0 ◄ Tool Chain

5

.2 Assembler

The assembler24 is a program that will read an assembly language input file and convert

the code into a machine language binary file. The input file is an assembly language

source file containing assembly language instructions in human readable form. The

machine language output is referred to as an object file. As part of this process, the

comments are removed, and the variable names and labels are converted into

appropriate addresses (as required by the CPU during execution).

The assembler used in this text is the yasm25 assembler. Links to the yasm web site and

documentation can be found in Chapter 1, Introduction

5

.2.1 Assemble Commands

The appropriate yasm assembler command for reading the assembly language source

file, such as the example from the previous chapter, is as follows:

yasm -g dwarf2 -f elf64 example.asm -l example.lst

Note, the -l is a dash lower-case letter L (which is easily confused with the number 1).

The -g dwarf226 option is used to inform the assembler to include debugging

information in the final object file. This increases the size of the object file, but is

necessary to allow effective debugging. The -f elf64 informs the assembler to create the

object file in the ELF6427 format which is appropriate for 64-bit, Linux-based systems.

The example.asm is the name of the assembly language source file for input. The -l

example.lst (dash lower-case letter L) informs the assembler to create a list file named

example.lst.

If an error occurs during the assembly process, it must be resolved before continuing to

the link step.

5

.2.2 List File

In addition, the assembler is optionally capable of creating a list file. The list file shows

the line number, the relative address, the machine language version of the instruction

(including variable references), and the original source line. The list file can be useful

when debugging.

2

2

2

2

4 For more information, refer to: http://en.wikipedia.org/wiki/Assembler_(computing)#Assembler

5 For more information, refer to: https://en.wikipedia.org/wiki/Yasm

6 For more information, refer to: https://en.wikipedia.org/wiki/DWARF

7 For more information, refer to: http://en.wikipedia.org/wiki/Executable_and_Linkable_Format

Page 43

Chapter 5.0 ◄ Tool Chain

For example, a fragment from the list file data section, from the example program in the

previous chapter is as follows:

36 00000009 40660301          dVar1    dd  17000000

37 0000000D 40548900          dVar2    dd  9000000

38 00000011 00000000          dResult  dd  0

On the first line, the 36 is the line number. The next number, 0x00000009, is the

relative address in the data area of where that variable will be stored. Since dVar1 is a

double-word, which requires four bytes, the address for the next variable is

0

0

x0000000D. The dVar1 variable uses 4 bytes as addresses 0x00000009, 0x0000000A,

x0000000B, and 0x0000000C. The rest of the line is the data declaration as typed in

the original assembly language source file.

The 0x40660301 is the value, in hex, as placed in memory. The 17,000,00010 is

0

x01036640. Recalling that the architecture is little-endian, the least significant byte

(0x40) is placed in the lowest memory address. As such, the 0x40 is placed in relative

address 0x00000009, the next byte, 0x66, is placed in address 0x0000000A and so forth.

This can be confusing as at first glance the number may appear backwards or garbled

(depending on how it is viewed).

To help visualize, the memory picture would be as follows:

variable

name

value

address

0

8

5

0

9

4

0x00000010

0x0000000F

0x0000000E

0x0000000D

0x0000000C

0x0000000B

0x0000000A

0x00000009

dVar2 →

dVar1 →

40

01

03

66

40

Illustration 7: Little-Endian, Multiple Variable Data Layout

Page 44

Chapter 5.0 ◄ Tool Chain

For example, a fragment of the list file text section, excerpted from the example

program in the previous chapter is as follows:

95                            last:

96 0000005A 48C7C03C000000       mov     rax, SYS_exit

97 00000061 48C7C300000000       mov     rdi, EXIT_SUCCESS

98 00000068 0F05                 syscall

Again, the numbers to the left are the line numbers. The next number, 0x0000005A, is

the relative address of where the line of code will be placed.

The next number, 0x48C7C03C000000, is the machine language version of the

instruction, in hex, that the CPU reads and understands. The rest of the line is the

original assembly language source instruction.

The label, last:, does not have a machine language instruction since the label is used to

reference a specific address and is not an executable instruction.

5

.2.3 Two-Pass Assembler

The assembler28 will read the source file and convert each assembly language

instruction, typed by the programmer, into a set of 1's and 0's that the CPU knows to be

that instruction. The 1's and 0's are referred to as machine language. There is a one-to-

one correspondence between the assembly language instructions and the binary machine

language. This relationship means that machine language, in the form of an executable

file can be converted back into human readable assembly language. Of course, the

comments, variable names, and label names are missing, so the resulting code can be

very difficult to read.

As the assembler reads each line of assembly language, it generates machine code for

that instruction. This will work well for instructions that do not perform jumps.

However, for instructions that might change the control flow (e.g., IF statements,

unconditional jumps), the assembler is not able to convert the instruction. For example,

given the following code fragment:

mov

jmp

rax, 0

skipRest

.

.

..

..

skipRest:

2

8 For more information, refer to: http://en.wikipedia.org/wiki/Assembly_language#Assembler

Page 45

Chapter 5.0 ◄ Tool Chain

This is referred to as a forward reference. If the assembler reads the assembly file one

line at a time, it has not read the line where skipRest is defined. In fact, it does not even

know for sure if skipRest is defined at all.

This situation can be resolved by reading the assembly source file twice. The entire

process is referred to as a two-pass assembler. The steps required for each pass are

detailed in the following sections.

5.2.3.1 First Pass

The steps taken on the first pass vary based on the design of the specific assembler.

However, some of the basic operations performed on the first pass include the

following:

•

•

•

Create symbol table

Expand macros

Evaluate constant expressions

A macro is a program element that is expanded into a set of programmer predefined

instructions. For more information, refer to Chapter 11, Macros.

A constant expression is an expression composed entirely of constants. Since the

expression is constants only, it can be fully evaluated at assemble-time. For example,

assuming the constant BUFF is defined, the following instruction contains a constant

expression;

mov

rax, BUFF+5

This type of constant expression is used commonly in large or complex programs.

Addresses are assigned to all statements in the program. The symbol table is a listing or

table of all the program symbols, variable names and program labels, and their

respective addresses in the program.

As appropriate, some assembler directives are processed in the first pass.

5.2.3.2 Second Pass

The steps taken on the second pass vary based on the design of the specific assembler.

However, some of the basic operations performed on the second pass include the

following:

•

•

Final generation of code

Creation of list file (if requested)

Page 46

Chapter 5.0 ◄ Tool Chain

•

Create object file

The term code generation refers to the conversion of the programmer provided assembly

language instruction into the CPU executable machine language instruction. Due to the

one-to-one correspondence, this can be done for instructions that do not use symbols on

either the first or second pass.

It should be noted that, based on the assembler design, much of the code generation

might be done on the first pass or all done on the second pass. Either way, the final

generation is performed on the second pass. This will require using the symbol table to

check program symbols and obtain the appropriate addresses from the table.

The list file, while optional, can be useful for debugging. If requested, it would be

generated on the second pass.

If there are no errors, the final object file is created on the second pass.

5

.2.4 Assembler Directives

Assembler directives are instructions to the assembler that direct the assembler to do

something. This might be formatting or layout. These directives are not translated into

instructions for the CPU.

5

.3 Linker

The linker29, sometimes referred to as linkage editor, will combine one or more object

files into a single executable file. Additionally, any routines from user or system

libraries are included as necessary. The GNU gold linker, ld30, is used. The appropriate

linker command for the example program from the previous chapter is as follows:

ld -g -o example example.o

Note, the -o is a dash lower-case letter O, which can be confused with the number 0.

The -g option is used to inform the linker to include debugging information in the final

executable file. This increases the size of the executable file, but is necessary to allow

effective debugging. The -o example specifies to create the executable file named

example (with no extension). If the -o <fileName> option is omitted, the output file is

named a.out (by default). The example.o is the name of the input object file read by the

linker. It should be noted that the executable file could be named anything and does not

need to have the same name as any of the input object files.

2

3

9 For more information, refer to: http://en.wikipedia.org/wiki/Linker_(computing)

0 For more information, refer to: http://en.wikipedia.org/wiki/Gold_(linker)

Page 47

Chapter 5.0 ◄ Tool Chain

5

.3.1 Linking Multiple Files

In programming, large problems are typically solved by breaking them into smaller

problems. The smaller problems can be addressed individually, possibly by different

programmers.

Additional input object files, if any, would be listed, in order, separated with a space.

For example, if there are two object files, main.o and funcs.o the link command to create

an executable file name example, with debugging information included, would be as

follows:

ld -g -o example main.o funcs.o

This would typically be required for larger or more complex programs.

When using functions located in a different, external source file, any function or

functions not in the current source file must be declared as extern. Variables, such as

global variables, in other source files can be accessed by using the extern statement as

well, however data is typically transferred as arguments of the function call.

5

.3.2 Linking Process

Linking is the fundamental process of combining the smaller solutions into a single

executable unit. If any user or system library routines are used, the linker will include

the appropriate routines. The object files and library routines are combined into a single

executable module. The machine language code is copied from each object file into a

single executable.

As part of combining the object files, the linker must adjust the relocatable addresses as

necessary. Assuming there are two source files, the main and a secondary source file

containing some functions, both of which have been assembled into object files main.o

and funcs.o. When each file is assembled, the calls to routines outside the file being

assembled are declared with the external assembler directive. The code is not available

for an external reference and such references are marked as external in the object file.

The list file will show an R for such relocatable addresses. The linker must satisfy the

external references. Additionally, the final location of the external references must be

placed in the code.

For example, if the main.o object file calls a function in the funcs.o file, the linker must

update the call with the appropriate address as shown in the following illustration.

Page 48

Chapter 5.0 ◄ Tool Chain

main.o

..

executable

.

call fnc1

…

call 0x0400

funcs.o

.

..

0

x0400:

.

..

…

fnc1:

0

x0100:

…

Illustration 8: Linking Multiple Files

Here, the function fnc1 is external to the main.o object file and is marked with an R.

The actual function fnc1 is in the funcs.o file, which starts its relative addressing from

0

x0 (in the text section) since it does not know about the main code. When the object

files are combined, the original relative address of fnc1 (shown as 0x0100:) is changed

to its final address in executable file (shown as 0x0400:). The linker must insert this

final address into the call statement in the main (shown as call 0x0400:) in order to

complete the linking process and ensure the function call will work correctly.

This will occur with all relocatable addresses for both code and data.

5

.3.3 Dynamic Linking

The Linux operating system supports dynamic linking31, which allows for postponing

the resolution of some symbols until a program is being executed.

3

1 For more information, refer to: http://en.wikipedia.org/wiki/Dynamic_linker

Page 49

Chapter 5.0 ◄ Tool Chain

The actual instructions are not placed in executable file and instead, if needed, resolved

and accessed at run-time.

While more complex, this approach offers two advantages:

•

Often-used libraries (e.g., the standard system libraries) can be stored in only one

location, not duplicated in every single binary.

•

If a bug in a library function is corrected, all programs using it dynamically will

benefit from the correction (at the next execution). Otherwise, programs that

utilize this function by static linking would have to be re-linked before the

correction is applied.

There are also disadvantages:

•

An incompatible updated library will break executable’s that depended on the

behavior of the previous version of the library.

•

A program, together with the libraries it uses, might be certified (e.g. as to

correctness, documentation requirements, or performance) as a package, but not

if components can be replaced.

In Linux/Unix, the dynamically linked object files typically a have .so (shared object)

extension. In Windows, it is a .dll (dynamically linked library) extension. Further

details of dynamic linking are outside the scope of this text.

5

.4 Assemble/Link Script

When programming, it is often necessary to type the assemble and link commands many

times with various different programs. Instead of typing the assemble (yasm) and link

(ld) commands each time, it is possible to place them in a file, called a script file. Then,

the script file can be executed which will just execute the commands that were entered

in the file. While not required, using a script file can save time and make things easier

when working on a program.

A simple example bash32 assemble/link script is as follows:

#

!/bin/bash

# Simple assemble/link script.

if [ -z $1 ]; then

echo "Usage:  ./asm64 <asmMainFile> (no extension)"

exit

3

2 For more information, refer to: http://en.wikipedia.org/wiki/Bash_(Unix_shell)

Page 50

Chapter 5.0 ◄ Tool Chain

fi

#  Verify no extensions were entered

if [ ! -e "$1.asm" ]; then

echo "Error, $1.asm not found."

echo "Note, do not enter file extensions."

exit

fi

#  Compile, assemble, and link.

yasm -Worphan-labels -g dwarf2 -f elf64 $1.asm -l $1.lst

ld -g -o $1 $1.o

The above script should be placed in a file. For this example, the file will be named

asm64 and placed in the current working directory (where the source files are located).

Once created, execute privilege will need to be added to the script file as follows:

chmod +x asm64

This will only need to be done once for each script file.

The script file will read the source file name from the command line. For example, to

use the script file to assemble the example from the previous chapter (named

example.asm), type the following:

./asm64 example

The ".asm" extension on the example.asm file should not be included (since it is added

in the script). The script file will assemble and link the source file, creating the list file,

object file, and executable file.

Use of this, or any script file, is optional. The name of the script file can be changed as

desired.

5

.5 Loader

The loader33 is a part of the operating system that will load the program from secondary

storage into primary storage (i.e., main memory). In broad terms, the loader will

attempt to find, and if found, read a properly formatted executable file, create a new

3

3 For more information, refer to: http://en.wikipedia.org/wiki/Loader_(computing)

Page 51

Chapter 5.0 ◄ Tool Chain

process, and load the code into memory and mark the program as ready for execution.

The operating system scheduler will make the decisions about which process is executed

and when the process is executed.

The loader is implicitly invoked by typing the program name. For example, on the

previous example program, named example, the Linux command would be:

./example

which will execute the file named example created via the previous steps (assemble and

link). Since the example program does not perform any output, nothing will be

displayed to the console. As such, a debugger can be used to check the results.

5

.6 Debugger

The debugger34 is used to control execution of a program. This allows for testing and

debugging activities to be performed.

In the previous example, the program computed a series of calculations, but did not

output any of the results. The debugger can be used to check the results. The

executable file is created with the assemble and link command previously described and

must include the -g option.

The debugger used is the GNU DDD debugger which provides a visual front-end for the

GNU command line debugger, gdb. The DDD web site and documentation are noted in

the references section of Chapter 1, Introduction.

Due to the complexity and importance of the debugger, a separate chapter for the

debugging is provided.

5

.7 Exercises

Below are some questions based on this chapter.

5

.7.1 Quiz Questions

Below are some quiz questions.

1

2

3

) What is the relationship between assembly language and machine language?

) What actions are performed on the first pass of the assembler?

) What actions are performed on the second pass of the assembler?

3

4 For more information, refer to: http://en.wikipedia.org/wiki/Debugger

Page 52

Chapter 5.0 ◄ Tool Chain

4

5

6

7

8

9

) What actions are performed by the linker?

) What actions are performed by the loader?

) Provide an example of a constant expression.

) Draw a diagram of the entire assemble, link, and load process.

) When is a shared object file linked with a program?

) What is contained in the symbol table (two things)?

Page 53

Chapter 5.0 ◄ Tool Chain

Page 54

Chapter

6

My software never has bugs. It just

develops random features.

6.0 DDD Debugger

A debugger allows the user to control execution of a program, examine variables, other

memory (i.e., stack space), and display program output (if any). The open source GNU

Data Display Debugger (DDD35) is a visual front-end to the GNU Debugger (GDB36

)

and is widely available. Other debuggers can easily be used if desired.

Only the basic debugger commands are addressed in this chapter. The DDD debugger

has many more features and options not covered here. As you gain experience, it would

be worth reviewing the DDD documentation, referenced in Chapter 1, to learn more

about additional features in order to help improve overall debugging efficiency.

DDD functionality can be extended using various plug-ins. The plug-ins are not

required and will not be addressed in this Chapter.

This chapter addresses using the GNU DDD debugger as a tool. The logical process of

how to debug a program is not addressed in this chapter.

6

.1 Starting DDD

The ddd debugger is started with the executable file. The program must be assembled

and linked with the correct options (as noted in the previous chapter). For example,

using the previous sample program, example, the command would be:

ddd example

Upon starting DDD/GDB, something similar to the screen, shown below, should be

displayed (with the appropriate source code displayed).

3

3

5 For more information, refer to: http://en.wikipedia.org/wiki/Data_Display_Debugger

6 For more information, refer to: http://en.wikipedia.org/wiki/GNU_Debugger

Page 55

Chapter 6.0 ◄ DDD Debugger

Illustration 9: Initial Debugger Screen

If the code is not displayed in a similar manner as shown above, the assemble and link

steps should be verified. Specifically, the -g qualifier must be included in both the

assemble and link steps.

Built in help is available by clicking on the Help menu item (upper right-hand corner).

The DDD and GDB manuals are available from the virtual library link on the class web

page. To exit DDD/GDB, select File → Exit (from the top menu bar).

Page 56

Chapter 6.0 ◄ DDD Debugger

6

.1.1 DDD Configuration Settings

Some additional DDD/GDB configuration settings suggestions include:

Edit → Preferences → General → Suppress X Warning

Edit → Preferences → Source → Display Source Line Numbers

These are not required, but can make using the debugger easier. If set, the options will

be saved and remembered for successive uses of the debugger (on the same machine).

6

.2 Program Execution with DDD

To execute the program, click on the Run button from the command tool menu (shown

below). Alternately, you can type run at the (gdb) prompt (bottom GDB console

window). However, this will execute the program entirely and, when done, the results

will be reset (and lost).

6

.2.1 Setting Breakpoints

In order to control program execution, it will be necessary to set a breakpoint (execution

pause location) to pause the program at a user selected location. This can be done by

selecting the source location (line to stop at). For this example, we will stop at line 95.

The breakpoint can be done one of three ways:

•

•

•

Right click on the line number and select: Set Breakpoint

In the GDB Command Console, at the (gdb) prompt, type: break last

In the GDB Command Console, at the (gdb) prompt, type: break 95

In the following example, line 94 is a label with no instruction. If a breakpoint is set on

label, it will stop at the next executable instruction (line 95 in this example).

Page 57

Chapter 6.0 ◄ DDD Debugger

When set correctly, the “stop” icon will appear to the left of line number (as shown in

the diagram).

Illustration 10: Debugger Screen with Breakpoint Set

DDD/GDB commands can be typed inside the bottom window (at the (gdb) prompt) at

any time. Multiple breakpoints can be set if desired.

6

.2.2 Executing Programs

Once the debugger is started, in order to effectively use the debugger, an initial

breakpoint must be set.

Page 58

Chapter 6.0 ◄ DDD Debugger

Once the breakpoint is set, the run command can be performed via clicking Run menu

window or typing run at the (gdb) prompt. The program will execute up to, but not

including the statement with the green arrow.

Illustration 11: Debugger Screen with Green Arrow

The breakpoint is indicated with the stop sign on the left and the current location is

indicated with a green arrow (see example above). Specifically, the green arrow points

to the next instruction to be executed. That is, the statement pointed to by the green

arrow has not yet been executed.

Page 59

Chapter 6.0 ◄ DDD Debugger

.2.2.1 Run / Continue

6

As needed, additional breakpoints can be set. However, click the Run command will

re-start execution from the beginning and stop at the initial breakpoint.

Illustration 12: DDD Command Bar

After the initial Run command, to continue to the next breakpoint, the continue

command must be used (by clicking Cont menu window or typing cont at the (gdb)

prompt). Single lines can also be executed one line at a time by typing the step or next

commands (via clicking Step or Next menu window or typing step or next at the (gdb)

prompt).

6.2.2.2 Next / Step

The next command will execute to the next instruction. This includes executing an

entire function if necessary. The step command will execute one step, stepping into

functions if necessary. For a single, non-function instruction, there is no difference

between the next and step commands.

6

.2.3 Displaying Register Contents

The simplest method to see the contents of the registers is to use the registers window.

The registers window is not displayed by default, but can be viewed by selecting Status

→

Registers (from the top menu bar). When displayed, the register window will show

register contents by register name (left column), in both hex (middle column) and

unsigned decimal (right column). Since the right column will display the unsigned

Page 60

Chapter 6.0 ◄ DDD Debugger

value of the entire register it can be confusing when the data is signed (as it will be

displayed as unsigned). Additionally, for some registers, such as rbp and rsp, both

columns are shown in hex (since they are typically used for addresses). The examine

memory command will, as described in the following section, allow more specific

control over what format (e.g., signed, unsigned, hex) in which to display values.

Illustration 13: Register Window

Depending on the machine and screen resolution, the register window may need to be

resized to view the entire contents.

The third column of the register window generally shows the decimal quadword

representation except for some special purpose registers (rbp and rsp). The signed

quadword decimal representation may not always be meaningful. For example, if

unsigned data is being used (such as addresses), the signed representation would be

incorrect. Additionally, when character data is used, the signed representation would

not be meaningful.

By default, only the integer registers are displayed. Clicking on the “All registers” box

will add the floating-point registers to the display. Viewing will require scrolling down

within the register window.

Page 61

Chapter 6.0 ◄ DDD Debugger

6

.2.4 DDD/GDB Commands Summary

The following table provides a small subset of the most common DDD commands.

When typed, most commands may be abbreviated. For example, quit can be

abbreviated as q. The command and the abbreviation are shown in the table.

Command

Description

quit | q

Quit the debugger.

break <label/addr>

b <label/addr>

Set a break point (stop point) at <label> or

<address>.

|

run <args> | r <args>

continue | c

Execute the program (to the first breakpoint).

Continue execution (to the next breakpoint).

continue <n> | c <n>

Continue execution (to the next breakpoint),

skipping n-1 crossing of the breakpoint. This is

can be used to quickly get to the nth iteration of

a loop.

step | s

next | n

Step into next instruction (i.e., steps into

function/procedure calls).

Next

instruction

(steps

through

function/procedure calls).

F3

Re-start program (and stop at first breakpoint).

Current activation (call depth).

where

x/<n><f><u> $rsp

Examine contents of the stack.

Page 62

Chapter 6.0 ◄ DDD Debugger

Command

Description

x/<n><f><u> &<variable> Examine memory location <variable>

n> number of locations to display, 1 is

default.

<

<f> format: d – decimal (signed)

x – hex

u – decimal (unsigned)

c – character

s – string

f – floating-point

<

u> unit size: b – byte (8-bits)

h – halfword (16-bits)

w – word (32-bits)

g – giant (64-bits)

source <filename>

Read commands from file <filename>.

set logging file <filename> Set logging file to <filename>, default is

gdb.txt.

set logging on

Turn logging (to a file) on.

Turn logging (to a file) off.

set logging off

set logging overwrite

When logging (to a file) is turned on, overwrite

previous log file (if any).

More information can be obtained via the built-in help facility or from the

documentation on the ddd website (referenced from Chapter 1).

Page 63

Chapter 6.0 ◄ DDD Debugger

6.2.4.1 DDD/GDB Commands, Examples

For example, given the below data declarations:

bnum1

wnum2

dnum3

qnum

class

twopi

db

dw

dd

dq

db

dd

5

-2000

100000

1234567890

"Assembly", 0

6.28

Assuming signed data, the commands to examine memory commands would be as

follows:

x/db &bnum1

x/dh &wnum2

x/dw &dnum3

x/dg &qnum

x/s &class

x/f &twopi

If an inappropriate memory dump command is used (i.e., incorrect size), there is no

error message and the debugger will display what was requested (even if it does not

make sense). Examining variables will require use of the appropriate memory dump

command based on the data declarations. Additional options can be accessed across the

menu at the top of the screen.

To display an array in DDD, the basic examine memory command is used.

x/<n><f><u> &<variable>

For example, assuming the declaration of:

list1

dd

100001, -100002, 100003, 100004, 100005

The examine memory commands would be as follows:

x/5dw &list1

Where the 5 is the array length. The d indicates signed data (u would have been

unsigned data). The w indicates 32-bit sized data (which is what the dd, define double,

definition declares in the source file). The &list1 refers to the address of the variable.

Note, the address points to the first element (and only the first element). As such, it is

possible to display less or more elements that are actually declared in the array.

Page 64

Chapter 6.0 ◄ DDD Debugger

The basic examine memory command can be used with a memory address directly (as

opposed to a variable name). For example:

x/dw 0x600d44

Addresses are typically displayed in hexadecimal, so a 0x would be required in order to

enter the hexadecimal address directly as shown.

6

.2.5 Displaying Stack Contents

There are some occasions when displaying the contents of the stack may be useful. The

stack is normally comprised of 64-bit, unsigned elements. The examine memory

command is used, however the address is in the rsp register (not a variable name). The

examine memory command to display the current top of the stack would be as follows:

x/ug $rsp

The examine memory command to display the top 6 items on the stack would be as

follows:

x/6ug $rsp

Due to the stack implementation, the first item shown will always be current top of the

stack.

6

.2.6 Interactive Debugger Commands File

Since the data display command(s) must be entered correctly, it can be tedious.

Additionally, typically there is no error message if the command is incorrect which can

be confusing. To help reduce errors, the correct execution and display command can be

stored in a text file. The debugger can then read the commands from the file (instead of

typing them by hand). While the results are typically displayed to the screen, the results

can be redirected to an output file. This can be useful for easy review.

For example, some typical debugger commands to set the breakpoint, run the program,

display some variables, and redirect the output to a log file might be as follows:

#

#

#

------------------------------------

Debugger Input Script

------------------------------------

echo \n\n

break last

run

set pagination off

Page 65

Chapter 6.0 ◄ DDD Debugger

set logging file out.txt

set logging overwrite

set logging on

set prompt

echo ------------------------------------ \n

echo display variables \n

echo \n

x/100dw &list

x/dw &length

echo \n

x/dw &listMin

x/dw &listMid

x/dw &listMax

x/dw &listSum

x/dw &listAve

echo \n \n

set logging off

quit

Note 1; this example assumes a label 'last' is defined in the source program (as is

done on the example program).

Note 2; this example exits the debugger. If that is not desired, the 'quit' command

can be removed. When exiting from the input file, the debugger may request user

confirmation of the exit (yes or no).

These commands should be placed in a file (such as gdbIn.txt), so they can be read from

within the debugger.

6.2.6.1 Debugger Commands File (non-interactive)

The debugger command to read a file is ''source <filename>''. For example, if the

command file is named gdbIn.txt,

(gdb) source gdbIn.txt

Based on the above commands, the output will be placed in the file out.txt. The output

file name can be changed as desired.

Each program will require a custom set of input command based on the specific

variables and associated sizes in that program. The debugger input commands file will

only be useful when the program is fairly close to working. Program crashes and other

more significant errors will require interactive debugging to determine the specific error

or errors.

Page 66

Chapter 6.0 ◄ DDD Debugger

6.2.6.2 Non-interactive Debugger Commands File

It is possible to obtain the output file directly without an interactive DDD session. The

following command, entered at the command line, will execute the command in the

input file on the given program, create the output file, and exit the program.

gbd prog <gdbIn.txt

Which will create the output file (as specified in the gdbIn.txt input file) and exit the

debugger. Once the input file is created, this is the fastest option for obtaining the final

output file for a working program. Again, this would only be useful if the program is

working or very close to working correctly.

6

.3 Exercises

Below are some quiz questions based on this chapter.

6

.3.1 Quiz Questions

Below are some quiz questions.

1

2

) How is the debugger started (from the command line)?

) What option is required during the assemble and link step in order to ensure the

program be easily debugged.

3

4

5

6

) What does the run command do specifically?

) What does the continue command do specifically?

) How is the register window displayed?

) There are three columns in the register window. The first shows the register.

What do the other two columns show?

7

8

9

1

) Once the debugger is started, how can the user exit?

) Describe how a breakpoint is set (multiple ways).

) What is the debugger command to read debugger commands from a file?

0) When the DDD shows a green arrow pointing to an instruction, what does that

mean?

1

1) Provide the debugger command to display each of the following variables in

decimal.

1

. bVar1 (byte sized variable)

Page 67

Chapter 6.0 ◄ DDD Debugger

2

3

4

5

6

7

. wVar1 (word sized variable)

. dVar1 (double-word sized variable)

. qVar1 (quadword sized variable)

. bArr1 (30 element array of bytes)

. wArr1 (50 element array of words)

. dArr1 (75 element array of double-words)

1

2) Provide the debugger command to display each of the following variables in

hexadecimal format.

1

2

3

4

5

6

7

. bVar1 (byte sized variable)

. wVar1 (word sized variable)

. dVar1 (double-word sized variable)

. qVar1 (quadword sized variable)

. bArr1 (30 element array of bytes)

. wArr1 (50 element array of words)

. dArr1 (75 element array of double-words)

1

1

3) What is the debugger command to display the value at the current top of the

stack?

4) What is the debugger command to display five (5) values at the current top of the

stack?

6

.3.2 Suggested Projects

Below are some suggested projects based on this chapter.

1

) Type in the example program from Chapter 4, Program Format. Assemble and

link the program as described in Chapter 5, Tool Chain. Execute the debugger as

noted in this chapter. Set a breakpoint at the label last and execute the program

(to that breakpoint). Interactively verify that the calculations performed resulted

in the correct values. This will require typing the appropriate debugger examine

memory commands (based on the variable size).

2

) After completing the previous problem, create a debugger input file that will set

the send the output to a text file, set a breakpoint, execute the program, and

display the results for each variable (based on the appropriate variable size).

Page 68

Chapter 6.0 ◄ DDD Debugger

Execute the debugger and read the source file. Review the input file worked

correctly and that the program calculations are correct based on the results

shown in the output file.

3

) Create an assemble and link script file, as described in Chapter 5, Tool Chain.

Use the script to assemble and link the program. Ensure that the script correctly

assembles and links.

Page 69

Chapter 6.0 ◄ DDD Debugger

Page 70

Chapter

7

Why are math books sad?

Because they have so many problems.

7.0 Instruction Set Overview

This chapter provides a basic overview for a simple subset of the x86-64 instruction set

focusing on the integer operations. This will cover only the subset of instructions

required for the topics and programs discussed within the scope of this text. This will

exclude some of the more advanced instructions and restricted mode instructions. For a

complete listing of all processor instructions, refer to the references listed in Chapter 1.

The instructions are presented in the following order:

•

•

•

•

•

Data Movement

Conversion Instructions

Arithmetic Instructions

Logical Instructions

Control Instructions

The instructions for function calls are discussed in the chapter in Chapter 12, Functions.

A complete listing of the instructions covered in this text is located in Appendix B for

reference.

7

.1 Notational Conventions

This section summarizes the notation used within this text which is fairly common in the

technical literature. In general, an instruction will consist of the instruction or operation

itself (i.e., add, sub, mul, etc.) and the operands. The operands refer to where the data

(to be operated on) is coming from and/or where the result is to be placed.

Page 71

Chapter 7.0 ◄ Instruction Set Overview

7

.1.1 Operand Notation

The following table summarizes the notational conventions used in the remainder of the

document.

Operand Notation Description

<reg>

Register operand. The operand must be a register.

<

<

reg8>, <reg16>, Register operand with specific size requirement. For

reg32>, <reg64> example, reg8 means a byte sized register (e.g., al, bl,

etc.) only and reg32 means a double-word sized register

(e.g., eax, ebx, etc.) only.

<dest>

Destination operand. The operand may be a register or

memory. Since it is a destination operand, the contents

will be overwritten with the new result (based on the

specific instruction).

<RXdest>

Floating-point destination register operand. The operand

must be a floating-point register. Since it is a destination

operand, the contents will be overwritten with the new

result (based on the specific instruction).

<src>

<imm>

<mem>

Source operand. Operand value is unchanged after the

instruction.

Immediate value. May be specified in decimal, hex, octal,

or binary.

Memory location. May be a variable name or an indirect

reference (i.e., a memory address).

<op> or <operand> Operand, register or memory.

<

<

op8>, <op16>, Operand, register or memory, with specific size

op32>, <op64> requirement. For example, op8 means a byte sized

operand only and reg32 means a double-word sized

operand only.

<label>

Program label.

By default, the immediate values are decimal or base-10. Hexadecimal or base-16

immediate values may be used but must be preceded with a 0x to indicate the value is

hex. For example, 15 could be entered in hex as 0x0F.

10

Page 72

Chapter 7.0 ◄ Instruction Set Overview

Refer to Chapter 8, Addressing Modes for more information regarding memory

locations and indirection.

7

.2 Data Movement

Typically, data must be moved into a CPU register from RAM in order to be operated

upon. Once the calculations are completed, the result may be copied from the register

and placed into a variable. There are a number of simple formulas in the example

program that perform these steps. This basic data movement operation is performed

with the move instruction.

The general form of the move instruction is:

mov <dest>, <src>

The source operand is copied from the source operand into the destination operand. The

value of the source operand is unchanged. The destination and source operand must be

of the same size (both bytes, both words, etc.). The destination operand cannot be an

immediate. Both operands cannot be memory. If a memory to memory operation is

required, two instructions must be used.

mov

eax, dword [myVariable]

what where

to do to place

how much memory

to get location

Illustration 14: MOV Instruction Overview

When the destination register operand is of double-word size and the source operand is

of double-word size, the upper-order double-word of the quadword register is set to

zero. This only applies when the destination operand is a double-word sized integer

register.

Specifically, if the following operations are performed,

mov

mov

mov

eax, 100

rcx, -1

ecx, eax

; eax = 0x00000064

; rcx = 0xffffffffffffffff

; ecx = 0x00000064

Initially, the rcx register is set to -1 (which is all 0xF's). When the positive number

from the eax register (100 ) is moved into the rcx register, the upper-order portion of

10

Page 73

Chapter 7.0 ◄ Instruction Set Overview

the quadword register rcx is set to 0 over-writing the 1's from the previous instruction.

The move instruction is summarized as follows:

Instruction

Explanation

mov   <dest>, <src>

Copy source operand to the destination

operand.

Note 1, both operands cannot be memory.

Note 2, destination operands cannot be an

immediate.

Note 3, for double-word destination and source

operand, the upper-order portion of the

quadword register is set to 0.

Examples:

mov   ax, 42

mov   cl, byte [bvar]

mov   dword [dVar], eax

mov   qword [qVar], rdx

A more complete list of the instructions is located in Appendix B.

For example, assuming the following data declarations:

dValue

bNum

wNum

dNum

qNum

bAns

wAns

dAns

qAns

dd

db

dw

dd

dq

db

dw

dd

dq

0

42

5000

73000

73000000

0

0

0

0

To perform, the basic operations of:

dValue = 27

bAns = bNum

wAns = wNum

dAns = dNum

qAns = qNum

Page 74

Chapter 7.0 ◄ Instruction Set Overview

The following instructions could be used:

mov

dword [dValue], 27

; dValue = 27

; bAns = bNum

; wAns = wNum

; dAns = dNum

; qAns = qNum

mov

mov

al, byte [bNum]

byte [bAns], al

mov

mov

ax, word [wNum]

word [wAns], ax

mov

mov

eax, dword [dNum]

dword [dAns], eax

mov

mov

rax, qword [qNum]

qword [qAns], rax

For some instructions, including those above, the explicit type specification (e.g., byte,

word, dword, qword) can be omitted as the other operand will clearly define the size. In

the text it will be included for consistency and good programming practice.

7

.3 Addresses and Values

The only way to access memory is with the brackets ([]'s). Omitting the brackets will

not access memory and instead obtain the address of the item. For example:

mov  rax, qword [var1]

mov  rax, var1

; value of var1 in rax

; address of var1 in rax

Since omitting the brackets is not an error, the assembler will not generate error

messages or warnings. This can lead to confusion.

In addition, the address of a variable can be obtained with the load effective address, or

lea, instruction. The load effective address instruction is summarized as follows:

Instruction

Explanation

lea   <reg64>, <mem>

Place address of <mem> into reg64.

Examples:

lea   rcx, byte [bvar]

lea   rsi, dword [dVar]

A more complete list of the instructions is located in Appendix B.

Page 75

Chapter 7.0 ◄ Instruction Set Overview

Additional information and extensive examples are presented in Chapter 8, Addressing

Modes.

7

.4 Conversion Instructions

It is sometimes necessary to convert from one size to another size. For example, a byte

might need to be converted to a double-word for some calculations in a formula. The

process used for conversions depends on the size and type of the operand. The

following sections summarize how conversions are performed.

7

.4.1 Narrowing Conversions

Narrowing conversions are converting from a larger type to a smaller type (i.e., word to

byte or double-word to word).

No special instructions are needed for narrowing conversions. The lower portion of the

memory location or register may be accessed directly. For example, if the value of 50

(0x32) is placed in the rax register, the al register may be accessed directly to obtain the

value as follows:

mov rax, 50

mov byte [bVal], al

This example is reasonable since the value of 50 will fit in a byte value. However, if the

value of 500 (0x1f4) is placed in the rax register, the al register can still be accessed.

mov rax, 500

mov byte [bVal], al

In this example, the bVal variable will contain 0xf4 which may lead to incorrect results.

The programmer is responsible for ensuring that narrowing conversions are performed

appropriately. Unlike a compiler, no warnings or error messages will be generated.

7

.4.2 Widening Conversions

Widening conversions are from a smaller type to a larger type (e.g., byte to word or

word to double-word). Since the size is being expanded, the upper-order bits must be

set based on the sign of the original value. As such, the data type, signed or unsigned,

must be known and the appropriate process or instructions must be used.

Page 76

Chapter 7.0 ◄ Instruction Set Overview

7.4.2.1 Unsigned Conversions

For unsigned widening conversions, the upper part of the memory location or register

must be set to zero. Since an unsigned value can only be positive, the upper-order bits

can only be zero. For example, to convert the byte value of 50 in the al register, to a

quadword value in rbx, the following operations can be performed.

mov

mov

mov

al, 50

rbx, 0

bl, al

Since the rbx register was set to 0 and then the lower 8-bits were set to the value from al

(50 in this example), the entire 64-bit rbx register is now 50.

This general process can be performed on memory or other registers. It is the

programmer's responsibility to ensure that the values are appropriate for the data sizes

being used.

An unsigned conversion from a smaller size to a larger size can also be performed with a

special move instruction, as follows:

movzx

<dest>, <src>

Which will fill the upper-order bits with zero. The movzx instruction does not allow a

quadword destination operand with a double-word source operand. As previously

noted, a mov instruction with a double-word register destination operand with a double-

word source operand will zero the upper-order double-word of the quadword destination

register.

A summary of the instructions that perform the unsigned widening conversion are as

follows:

Instruction

Explanation

movzx   <dest>, <src>

Unsigned widening conversion.

Note 1, both operands cannot be memory.

Note 2, destination operands cannot be an

immediate.

movzx   <reg16>, <op8>

movzx   <reg32>, <op8>

movzx   <reg32>, <op16>

movzx   <reg64>, <op8>

movzx   <reg64>, <op16>

Note 3, immediate values not allowed.

Examples:

movzx   cx, byte [bVar]

movzx   dx, al

movzx   ebx, word [wVar]

Page 77

Chapter 7.0 ◄ Instruction Set Overview

movzx   ebx, cx

movzx   rbx, cl

movzx   rbx, cx

A more complete list of the instructions is located in Appendix B.

.4.2.2 Signed Conversions

7

For signed widening conversions, the upper-order bits must be set to either 0's or 1's

depending on if the original value was positive or negative.

This is performed by a sign-extend operation. Specifically, the upper-order bit of the

original value indicates if the value is positive (with a 0) or negative (with a 1). The

upper-order bit of the original value is extended into the higher bits of the new, widened

value.

For example, given that the ax register is set to -7 (0xfff9), the bits would be set as

follows:

1

5

14 13 12 11 10

9

8

7

6

5

4

3

2

1

0

1

1

1

1

1

1

1

1

1

1

1

1

1

0

0

1

Since the value is negative, the upper-order bit (bit 15) is a 1. To convert the word value

in the ax register into a double-word value in the eax register, the upper-order bit (1 in

this example) is extended or copied into the entire upper-order word (bits 31-16)

resulting in the following:

3

1

30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10

9

8

7

6

5

4

3

2

1

0

1

1

1

1

1

1

1

1

1

1

1

1

1

1

1

1

1

1

1

1

1

1

1

1

1

1

1

1

1

0

0

1

There are a series of dedicated instructions used to convert signed values in the A

register from a smaller size into a larger size. These instructions work only on the A

register, sometimes using the D register for the result. For example, the cwd instruction

will convert a signed value in the ax register into a double-word value in the dx (upper-

order portion) and ax (lower-order portion) registers. This is typically by convention

written as dx:ax. The cwde instruction will convert a signed value in the ax register

into a double-word value in the eax register.

Page 78

Chapter 7.0 ◄ Instruction Set Overview

A more generalized signed conversion from a smaller size to a larger size can also be

performed with some special move instructions, as follows:

movsx

movsxd

<dest>, <src>

<dest>, <src>

Which will perform the sign extension operation on the source argument. The movsx

instruction is the general form and the movsxd instruction is used to allow a quadword

destination operand with a double-word source operand.

A summary of the instructions that perform the signed widening conversion are as

follows:

Instruction

Explanation

cbw

cwd

cwde

cdq

Convert byte in al into word in ax.

Note, only works for al to ax register.

Examples:

Examples:

Examples:

cbw

Convert word in ax into double-word in dx:ax.

Note, only works for ax to dx:ax registers.

cwd

Convert word in ax into double-word in eax.

Note, only works for ax to eax register.

cwde

Convert double-word in eax into quadword in

edx:eax.

Note, only works for eax to edx:eax registers.

Examples:

Examples:

cdq

cdqe

Convert double-word in eax into quadword in

rax.

Note, only works for rax register.

cdqe

Page 79

Chapter 7.0 ◄ Instruction Set Overview

Instruction

Explanation

cqo

Convert quadword in rax into word in double-

quadword in rdx:rax.

Note, only works for rax to rdx:rax registers.

Examples:

cqo

movsx   <dest>, <src>

Signed widening conversion (via sign

extension).

Note 1, both operands cannot be memory.

Note 2, destination operands cannot be an

immediate.

Note 3, immediate values not allowed.

Note 4, special instruction (movsxd) required

for 32-bit to 64-bit signed extension.

movsx   <reg16>, <op8>

movsx   <reg32>, <op8>

movsx   <reg32>, <op16>

movsx   <reg64>, <op8>

movsx   <reg64>, <op16>

movsxd  <reg64>, <op32>

Examples:

movsx   cx, byte [bVar]

movsx   dx, al

movsx   ebx, word [wVar]

movsx   ebx, cx

movsxd  rbx, dword [dVar]

A more complete list of the instructions is located in Appendix B.

7

.5 Integer Arithmetic Instructions

The integer arithmetic instructions perform arithmetic operations such as addition,

subtraction, multiplication, and division on integer values. The following sections

present the basic integer arithmetic operations.

7

.5.1 Addition

The general form of the integer addition instruction is as follows:

add   <dest>, <src>

Where operation performs the following:

<dest> = <dest> + <src>

Specifically, the source and destination operands are added and the result is placed in

Page 80

Chapter 7.0 ◄ Instruction Set Overview

the destination operand (over-writing the previous contents). The value of the source

operand is unchanged. The destination and source operand must be of the same size

(both bytes, both words, etc.). The destination operand cannot be an immediate. Both

operands, cannot be memory. If a memory to memory addition operation is required,

two instructions must be used.

For example, assuming the following data declarations:

bNum1

bNum2

bAns

db

db

db

42

73

0

wNum1

wNum2

wAns

dw

dw

dw

4321

1234

0

dNum1

dNum2

dAns

dd

dd

dd

42000

73000

0

qNum1

qNum2

qAns

dq

dq

dq

42000000

73000000

0

To perform the basic operations of:

bAns = bNum1 + bNum2

wAns = wNum1 + wNum2

dAns = dNum1 + dNum2

qAns = qNum1 + qNum2

The following instructions could be used:

;

bAns = bNum1 + bNum2

mov

add

mov

al, byte [bNum1]

al, byte [bNum2]

byte [bAns], al

; wAns = wNum1 + wNum2

mov

add

mov

ax, word [wNum1]

ax, word [wNum2]

word [wAns], ax

;

dAns = dNum1 + dNum2

mov eax, dword [dNum1]

Page 81

Chapter 7.0 ◄ Instruction Set Overview

add

mov

eax, dword [dNum2]

dword [dAns], eax

; qAns = qNum1 + qNum2

mov

add

mov

rax, qword [qNum1]

rax, qword [qNum2]

qword [qAns], rax

For some instructions, including those above, the explicit type specification (e.g., byte,

word, dword, qword) can be omitted as the other operand will clearly define the size. It

is included for consistency and good programming practice.

In addition to the basic add instruction, there is an increment instruction that will add

one to the specified operand. The general form of the increment instruction is as

follows:

inc   <operand>

Where operation is as follows:

<operand> = <operand> + 1

The result is exactly the same as using the add instruction (and adding one). When

using a memory operand, the explicit type specification (e.g., byte, word, dword, qword)

is required to clearly define the size.

For example, assuming the following data declarations:

bNum

wNum

dNum

qNum

db

dw

dd

dq

42

4321

42000

42000000

To perform, the basic operations of:

rax = rax + 1

bNum = bNum + 1

wNum = wNum + 1

dNum = dNum + 1

qNum = qNum + 1

The following instructions could be used:

inc

inc

rax

byte [bNum]

; rax = rax + 1

; bNum = bNum + 1

Page 82

Chapter 7.0 ◄ Instruction Set Overview

inc

inc

inc

word [wNum]

dword [dNum]

qword [qNum]

; wNum = wNum + 1

; dNum = dNum + 1

; qNum = qNum + 1

The addition instruction operates the same on signed and unsigned data. It is the

programmer's responsibility to ensure that the data types and sizes are appropriate for

the operations being performed.

The integer addition instructions are summarized as follows:

Instruction

Explanation

add   <dest>, <src>

Add two operands, (<dest> + <src>) and place

the result in <dest> (over-writing previous

value).

Note 1, both operands cannot be memory.

Note 2, destination operand cannot be an

immediate.

Examples:

add   cx, word [wVvar]

add   rax, 42

add   dword [dVar], eax

add   qword [qVar], 300

inc   <operand>

Increment <operand> by 1.

Note, <operand> cannot be an immediate.

Examples:

inc   word [wVvar]

inc   rax

inc   dword [dVar]

inc   qword [qVar]

A more complete list of the instructions is located in Appendix B.

.5.1.1 Addition with Carry

7

The add with carry is a special add instruction that will include a carry from a previous

addition operation. This is useful when adding very large numbers, specifically

numbers larger than the register size of the machine.

Page 83

Chapter 7.0 ◄ Instruction Set Overview

Using a carry in addition is fairly standard. For example, consider the following

operation.

17

+ 25

----

42

As you may recall, the least significant digits (7 and 5) are added first. The result of 12

is noted as a 2 with a 1 carry. The most significant digits (1 and 2) are added along with

the previous carry (1 in this example) resulting in a 4.

As such, two addition operations are required. Since there is no carry possible with the

least significant portion, a regular addition instruction is used. The second addition

operation would need to include a possible carry from the previous operation and must

be done with an add with carry instruction. Additionally, the add with carry must

immediately follow the initial addition operation to ensure that the rFlag register is not

altered by an unrelated instruction (thus possibly altering the carry bit).

For assembly language programs the Least Significant Quadword (LSQ) is added with

the add instruction and then immediately the Most Significant Quadword (MSQ) is

added with the adc which will add the quadwords and include a carry from the previous

addition operation.

The general form of the integer add with carry instruction is as follows:

adc   <dest>, <src>

Where operation performs the following:

<dest> = <dest> + <src> + <carryBit>

Specifically, the source and destination operands along with the carry bit are added and

the result is placed in the destination operand (over-writing the previous value). The

carry bit is part of the rFlag register. The value of the source operand is unchanged.

The destination and source operand must be of the same size (both bytes, both words,

etc.). The destination operand cannot be an immediate. Both operands, cannot be

memory. If a memory to memory addition operation is required, two instructions must

be used.

For example, given the following declarations;

dquad1

ddq

0x1A000000000000000

Page 84

Chapter 7.0 ◄ Instruction Set Overview

dquad2

dqSum

ddq

ddq

0x2C000000000000000

0

Each of the variables dquad1, dquad2, and dqSum are 128-bits and thus will exceed the

machine 64-bit register size. However, two 64-bit registers can be used for each of the

1

28-bit values. This requires two move instructions, one for each 64-bit register. For

example,

mov

mov

rax, qword [dquad1]

rdx, qword [dquad1+8]

The first move to the rax register accesses the first 64-bits of the 128-bit variable. The

second move to the rdx register access the next 64-bits of the 128-bit variable. This is

accomplished by using the variable starting address, dquad1 and adding 8 bytes, thus

skipping the first 64-bits (or 8 bytes) and accessing the next 64-bits.

If the LSQ's are added and then the MSQ's are added including any carry, the 128-bit

result can be correctly obtained. For example,

mov

mov

rax, qword [dquad1]

rdx, qword [dquad1+8]

add

adc

rax, qword [dquad2]

rdx, qword [dquad2+8]

mov

mov

qword [dqSum], rax

qword [dqSum+8], rdx

Initially, the LSQ of dquad1 is placed in rax and the MSQ is placed in rdx. Then the

add instruction will add the 64-bit rax with the LSQ of dquad2 and, in this example,

provide a carry of 1 with the result in rax. Then the rdx is added with the MSQ of

dquad2 along with the carry via the adc instruction and the result placed in rdx.

The integer add with carry instruction is summarized as follows:

Page 85

Chapter 7.0 ◄ Instruction Set Overview

Instruction

Explanation

adc   <dest>, <src>

Add two operands, (<dest> + <src>) and any

previous carry (stored in the carry bit in the

rFlag register) and place the result in <dest>

(over-writing previous value).

Note 1, both operands cannot be memory.

Note 2, destination operand cannot be an

immediate.

Examples:

adc   rcx, qword [dVvar1]

adc   rax, 42

A more complete list of the instructions is located in Appendix B.

7

.5.2 Subtraction

The general form of the integer subtraction instruction is as follows:

sub   <dest>, <src>

Where operation performs the following:

<dest> = <dest> - <src>

Specifically, the source operand is subtracted from the destination operand and the result

is placed in the destination operand (over-writing the previous value). The value of the

source operand is unchanged. The destination and source operand must be of the same

size (both bytes, both words, etc.). The destination operand cannot be an immediate.

Both operands, cannot be memory. If a memory to memory subtraction operation is

required, two instructions must be used.

For example, assuming the following data declarations:

bNum1

bNum2

bAns

db

db

db

73

42

0

wNum1

wNum2

wAns

dNum1

dNum2

dw

dw

dw

dd

dd

1234

4321

0

73000

42000

Page 86

Chapter 7.0 ◄ Instruction Set Overview

dAns

dd

0

qNum1

qNum2

qAns

dq

dq

dd

73000000

73000000

0

To perform, the basic operations of:

bAns = bNum1 - bNum2

wAns = wNum1 - wNum2

dAns = dNum1 - dNum2

qAns = qNum1 - qNum2

The following instructions could be used:

;

bAns = bNum1 - bNum2

mov

sub

mov

al, byte [bNum1]

al, byte [bNum2]

byte [bAns], al

; wAns = wNum1 – wNum2

mov

sub

mov

ax, word [wNum1]

ax, word [wNum2]

word [wAns], ax

;

dAns = dNum1 – dNum2

mov

sub

mov

eax, dword [dNum1]

eax, dword [dNum2]

dword [dAns], eax

; qAns = qNum1 - qNum2

mov

sub

mov

rax, qword [qNum1]

rax, qword [qNum2]

qword [qAns], rax

For some instructions, including those above, the explicit type specification (e.g., byte,

word, dword, qword) can be omitted as the other operand will clearly define the size. It

is included for consistency and good programming practices.

In addition to the basic subtract instruction, there is a decrement instruction that will

subtract one from the specified operand. The general form of the decrement instruction

is as follows:

dec   <operand>

Page 87

Chapter 7.0 ◄ Instruction Set Overview

Where operation performs the following:

<operand> = <operand> - 1

The result is exactly the same as using the subtract instruction (and subtracting one).

When using a memory operand, the explicit type specification (e.g., byte, word, dword,

qword) is required to clearly define the size.

For example, assuming the following data declarations:

bNum

wNum

dNum

qNum

db

dw

dd

dq

42

4321

42000

42000000

To perform, the basic operations of:

rax = rax - 1

bNum = bNum - 1

wNum = wNum - 1

dNum = dNum - 1

qNum = qNum - 1

The following instructions could be used:

dec

dec

dec

dec

dec

rax

; rax = rax - 1

byte [bNum]

word [wNum]

dword [dNum]

qword [qNum]

; bNum = bNum - 1

; wNum = wNum - 1

; dNum = dNum - 1

; qNum = qNum - 1

The subtraction instructions operate the same on signed and unsigned data. It is the

programmer's responsibility to ensure that the data types and sizes are appropriate for

the operations being performed.

The integer subtraction instructions are summarized as follows:

Instruction

Explanation

sub   <dest>, <src>

Subtract two operands, (<dest> - <src>) and

place the result in <dest> (over-writing

previous value).

Note 1, both operands cannot be memory.

Note 2, destination operand cannot be an

immediate.

Page 88

Chapter 7.0 ◄ Instruction Set Overview

Instruction

Explanation

Examples:

Examples:

sub   cx, word [wVvar]

sub   rax, 42

sub   dword [dVar], eax

sub   qword [qVar], 300

dec   <operand>

Decrement <operand> by 1.

Note, <operand> cannot be an immediate.

dec   word [wVvar]

dec   rax

dec   dword [dVar]

dec   qword [qVar]

A more complete list of the instructions is located in Appendix B.

7

.5.3 Integer Multiplication

The multiply instruction multiplies two integer operands. Mathematically, there are

special rules for handling multiplication of signed values. As such, different instructions

are used for unsigned multiplication (mul) and signed multiplication (imul).

Multiplication typically produces double sized results. That is, multiplying two n-bit

values produces a 2n-bit result. Multiplying two 8-bit numbers will produce a 16-bit

result. Similarly, multiplication of two 16-bit numbers will produce a 32-bit result,

multiplication of two 32-bit numbers will produce a 64-bit result, and multiplication of

two 64-bit numbers will produce a 128-bit result.

There are many variants for the multiply instruction. For the signed multiply, some

forms will truncate the result to the size of the original operands. It is the programmer's

responsibility to ensure that the values used will work for the specific instructions

selected.

7.5.3.1 Unsigned Multiplication

The general form of the unsigned multiplication is as follows:

mul

<src>

Where the source operand must be a register or memory location. An immediate

operand is not allowed.

For the single operand multiply instruction, the A register (al/ax/eax/rax) must be used

Page 89

Chapter 7.0 ◄ Instruction Set Overview

for one of the operands (al for 8-bits, ax for 16-bits, eax for 32-bits, and rax for 64-bit).

The other operand can be a memory location or register, but not an immediate.

Additionally, the result will be placed in the A and possibly D registers, based on the

sizes being multiplied. The following table shows the various options for the byte,

word, double-word, and quadword unsigned multiplications.

Bytes

al

op8

ah al

Words

Double-words

ax

eax

x

x

op16

ax

x

op32

dx

edx

eax

Quadwords

rax

x

op64

rdx

rax

Illustration 15: Integer Multiplication Overview

As shown in the chart, for most cases the integer multiply uses a combination of the A

and D registers. This can be very confusing.

For example, when multiplying a rax (64-bits) times a quadword operand (64-bits), the

multiplication instruction provides a double quadword result (128-bit). This can be

useful and important when dealing with very large numbers. Since the 64-bit

architecture only has 64-bit registers, the 128-bit result is, and must be, placed in two

different quadword (64-bit) registers, rdx for the upper-order result and rax for the

lower-order result, which is typically written as rdx:rax (by convention).

Page 90

Chapter 7.0 ◄ Instruction Set Overview

However, this use of two registers is applied to smaller sizes as well. For example, the

result of multiplying ax (16-bits) times a word operand (also 16-bits) provides a double-

word (32-bit) result. However, the result is not placed in eax (which might be easier), it

is placed in two registers, dx for the upper-order result (16-bits) and ax for the lower-

order result (16-bits), typically written as dx:ax (by convention). Since the double-word

(32-bit) result is in two different registers, two moves may be required to save the result.

This pairing of registers, even when not required, is due to legacy support for previous

earlier versions of the architecture. While this helps ensure backwards compatibility, it

can be quite confusing.

For example, assuming the following data declarations:

bNumA

bNumB

wAns

db

db

dw

dw

42

73

0

wAns1

0

wNumA

wNumB

dAns2

dw

dw

dd

4321

1234

0

dNumA

dNumB

qAns3

dd

dd

dq

42000

73000

0

qNumA

qNumB

dqAns4

dq

dq

ddq

420000

730000

0

To perform, the basic operations of:

wAns = bNumA^2

; bNumA squared

bAns1 = bNumA * bNumB

wAns1 = bNumA * bNumB

wAns2 = wNumA * wNumB

dAns2 = wNumA * wNumB

dAns3 = dNumA * dNumB

qAns3 = dNumA * dNumB

qAns4 = qNumA * qNumB

dqAns4 = qNumA * qNumB

Page 91

Chapter 7.0 ◄ Instruction Set Overview

The following instructions could be used:

;

wAns = bNumA^2 or bNumA squared

mov

mul

mov

al, byte [bNumA]

al

word [wAns], ax

; result in ax

; wAns1 = bNumA * bNumB

mov

mul

mov

al, byte [bNumA]

byte [bNumB]

word [wAns1], ax

; result in ax

;

dAns2 = wNumA * wNumB

mov

mul

mov

mov

ax, word [wNumA]

word [wNumB]

word [dAns2], ax

word [dAns2+2], dx

; result in dx:ax

;

qAns3 = dNumA * dNumB

mov

mul

mov

mov

eax, dword [dNumA]

dword [dNumB]

dword [qAns3], eax

dword [qAns3+4], edx

; result in edx:eax

; result in rdx:rax

;

dqAns4 = qNumA * qNumB

mov

mul

mov

mov

rax, qword [qNumA]

qword [qNumB]

qword [dqAns4], rax

qword [dqAns4+8], rdx

For some instructions, including those above, the explicit type specification (e.g., byte,

word, dword, qword) is required to clearly define the size.

The integer unsigned multiplication instruction is summarized as follows:

Page 92

Chapter 7.0 ◄ Instruction Set Overview

Instruction

Explanation

mul <src>

Multiply A register (al, ax, eax, or rax) times

the <src> operand.

mul <op8>

mul <op16>

mul <op32>

mul <op64>

Byte: ax = al * <src>

Word: dx:ax = ax * <src>

Double: edx:eax = eax * <src>

Quad: rdx:rax = rax * <src>

Note, <src> operand cannot be an immediate.

Examples:

mul   word [wVvar]

mul   al

mul   dword [dVar]

mul   qword [qVar]

A more complete list of the instructions is located in Appendix B.

.5.3.2 Signed Multiplication

7

The signed multiplication allows a wider range of operands and operand sizes. The

general forms of the signed multiplication are as follows:

imul <source>

imul <dest>, <src/imm>

imul <dest>, <src>, <imm>

In all cases, the destination operand must be a register. For the multiple operand

multiply instruction, byte operands are not supported.

When using a single operand multiply instruction, the imul is the same layout as the

mul (as previously presented). However, the operands are interpreted only as signed.

When two operands are used, the destination operand and the source operand are

multiplied and the result placed in the destination operand (over-writing the previous

value).

Specifically, the action performed is:

<dest> = <dest> * <src/imm>

For two operands, the <src/imm> operand may be a register, memory location, or

immediate value. The size of the immediate value is limited to the size of the source

operand, up to a double-word size (32-bit), even for quadword (64-bit) multiplications.

The final result is truncated to the size of the destination operand. A byte sized

destination operand is not supported.

Page 93

Chapter 7.0 ◄ Instruction Set Overview

When three operands are used, two operands are multiplied and the result placed in the

destination operand. Specifically, the action performed is:

<dest> = <src> * <imm>

For three operands, the <src> operand must be a register or memory location, but not an

immediate. The <imm> operand must be an immediate value. The size of the

immediate value is limited to the size of the source operand, up to a double-word size

(32-bit), even for quadword multiplications. The final result is truncated to the size of

the destination operand. A byte sized destination operand is not supported.

It should be noted that when the multiply instruction provides a larger type, the original

type may be used. For this to work, the values multiplied must fit into the smaller size

which limits the range of the data. For example, when two double-words are multiplied

and a quadword result is provided, the least significant double-word (of the quadword)

will contain the answer if the values are sufficiently small which is often the case. This

is typically done in high-level languages when an int (32-bit integer) variable is

multiplied by another int variable and assigned to an int variable.

For example, assuming the following data declarations:

wNumA

wNumB

wAns1

wAns2

dw

dw

dw

dw

1200

-2000

0

0

dNumA

dNumB

dAns1

dAns2

dd

dd

dd

dd

42000

-13000

0

0

qNumA

qNumB

qAns1

qAns2

dq

dq

dq

dq

120000

-230000

0

0

To perform, the basic operations of:

wAns1 = wNumA * -13

wAns2 = wNumA * wNumB

dAns1 = dNumA * 113

dAns2 = dNumA * dNumB

Page 94

Chapter 7.0 ◄ Instruction Set Overview

qAns1 = qNumA * 7096

qAns2 = qNumA * qNumB

The following instructions could be used:

;

wAns1 = wNumA * -13

mov ax, word [wNumA]

imul ax, -13

mov word [wAns1], ax

; result in ax

; result in ax

; result in eax

; result in eax

; result in rax

; result in rax

;

mov

wAns2 = wNumA * wNumB

ax, word [wNumA]

imul ax, word [wNumB]

mov word [wAns2], ax

;

dAns1 = dNumA * 113

mov eax, dword [dNumA]

imul eax, 113

mov dword [dAns1], eax

; dAns2 = dNumA * dNumB

mov

eax, dword [dNumA]

imul eax, dword [dNumB]

mov dword [dAns2], eax

;

qAns1 = qNumA * 7096

mov rax, qword [qNumA]

imul rax, 7096

mov qword [qAns1], rax

; qAns2 = qNumA * qNumB

mov

rax, qword [qNumA]

imul rax, qword [qNumB]

mov qword [qAns2], rax

Another way to perform the multiplication of

qAns1 = qNumA * 7096

Would be as follows:

; qAns1 = qNumA * 7096

mov rcx, qword [qNumA]

Page 95

Chapter 7.0 ◄ Instruction Set Overview

imul rbx, rcx, 7096

; result in rbx

mov

qword [qAns1], rbx

This example shows the three-operand multiply instruction using different registers.

In these examples, the multiplication result is truncated to the size of the destination

operand. For a full-sized result, the single operand instruction should be used (as fully

described in the section regarding unsigned multiplication).

For some instructions, including those above, the explicit type specification (e.g., byte,

word, dword, qword) may not be required to clearly define the size.

The integer signed multiplication instruction is summarized as follows:

Instruction

Explanation

imul <src>

imul <dest>, <src/imm32>

imul <dest>, <src>, <imm32>

Signed multiply instruction.

For single operand:

Byte: ax = al * <src>

imul <op8>

imul <op16>

imul <op32>

imul <op64>

imul <reg16>, <op16/imm>

imul <reg32>, <op32/imm>

imul <reg64>, <op64/imm>

imul <reg16>, <op16>, <imm>

imul <reg32>, <op32>, <imm>

imul <reg64>, <op64>, <imm>

Word: dx:ax = ax * <src>

Double: edx:eax = eax * <src>

Quad: rdx:rax = rax * <src>

Note, <src> operand cannot be an immediate.

For two operands:

<reg16> = <reg16> * <op16/imm>

<

<

reg32> = <reg32> * <op32/imm>

reg64> = <reg64> * <op64/imm>

For three operands:

<

<

<

reg16> = <op16> * <imm>

reg32> = <op32> * <imm>

reg64> = <op64> * <imm>

Examples:

imul  ax, 17

imul  al

imul  ebx, dword [dVar]

imul  rbx, dword [dVar], 791

imul  rcx, qword [qVar]

imul  qword [qVar]

A more complete list of the instructions is located in Appendix B.

Page 96

Chapter 7.0 ◄ Instruction Set Overview

7

.5.4 Integer Division

The division instruction divides two integer operands. Mathematically, there are special

rules for handling division of signed values. As such, different instructions are used for

unsigned division (div) and signed division (idiv).

dividend

divisor

Recall that

= quotient

Division requires that the dividend must be a larger size than the divisor. In order to

divide by an 8-bit divisor, the dividend must be 16-bits (i.e., the larger size). Similarly,

a 16-bit divisor requires a 32-bit dividend. And, a 32-bit divisor requires a 64-bit

dividend.

Like the multiplication, for most cases the integer division uses a combination of the A

and D registers. This pairing of registers is due to legacy support for previous earlier

versions of the architecture. While this helps ensure backwards compatibility, it can be

quite confusing.

Further, the A, and possibly the D register, must be used in combination for the

dividend.

•

•

•

•

Byte Divide:

Word Divide:

Double-word divide:

Quadword Divide:

ax for 16-bits

dx:ax for 32-bits

edx:eax for 64-bits

rdx:rax for 128-bits

Setting the dividend (top operand) correctly is a key source of problems. For the word,

double-word, and quadword division operations, the dividend requires both the D

register (for the upper-order portion) and A (for the lower-order portion).

Setting these correctly depends on the data type. If a previous multiplication was

performed, the D and A registers may already be set correctly. Otherwise, a data item

may need to be converted from its current size to a larger size with the upper-order

portion being placed in the D register. For unsigned data, the upper portion will always

be zero. For signed data, the existing data must be sign extended as noted in a previous

section, Signed Conversions.

The divisor can be a memory location or register, but not an immediate. Additionally,

the result will be placed in the A register (al/ax/eax/rax) and the remainder in either the

ah, dx, edx, or rdx register. Refer to the Integer Division Overview table to see the

layout more clearly.

Page 97

Chapter 7.0 ◄ Instruction Set Overview

The use of a larger size operand for the dividend matches the single operand

multiplication. For simple divisions, an appropriate conversion may be required in order

to ensure the dividend is set correctly. For unsigned divisions, the upper-order part of

the dividend can set to zero. For signed divisions, the upper-order part of the dividend

can be set with an applicable conversion instruction.

As always, division by zero will crash the program and damage the space-time

continuum. So, try not to divide by zero.

Page 98

Chapter 7.0 ◄ Instruction Set Overview

The following tables provide an overview of the divide instruction for bytes, words,

double-words, and quadwords.

Bytes

Words

dx

ah al

ax

=

al

=

ax

dx

op8

op16

rem ah

rem

Double-words

edx

eax

=

eax

op32

rem

edx

rax

Quadwords

rdx

op64

=

rax

rdx

rem

Illustration 16: Integer Division Overview

Page 99

Chapter 7.0 ◄ Instruction Set Overview

The signed and unsigned division instructions operate in the same manner. However,

the range of values that can be divided is different. The programmer is responsible for

ensuring that the values being divided are appropriate for the operand sizes being used.

The general forms of unsigned and signed division are as follows:

div

<src>

; unsigned division

; signed division

idiv <src>

The source operand and destination operands (A and D registers) are described in the

preceding table.

For example, assuming the following data declarations:

bNumA

bNumB

bNumC

bAns1

bAns2

bRem2

bAns3

db

db

db

db

db

db

db

63

17

5

0

0

0

0

wNumA

wNumB

wNumC

wAns1

wAns2

wRem2

wAns3

dw

dw

dw

dw

dw

dw

dw

4321

1234

167

0

0

0

0

dNumA

dNumB

dNumC

dAns1

dAns2

dRem2

dAns3

dd

dd

dd

dd

dd

dd

dd

42000

-3157

-293

0

0

0

0

qNumA

qNumB

qNumC

qAns1

qAns2

qRem2

qAns3

dq

dq

dq

dq

dq

dq

dq

730000

-13456

-1279

0

0

0

0

Page 100

Chapter 7.0 ◄ Instruction Set Overview

To perform, the basic operations of:

bAns1 = bNumA / 3

; unsigned

; unsigned

; % is modulus

; unsigned

bAns2 = bNumA / bNumB

bRem2 = bNumA % bNumB

bAns3 = (bNumA * bNumC) / bNumB

wAns1 = wNumA / 5

; unsigned

; unsigned

; % is modulus

; unsigned

wAns2 = wNumA / wNumB

wRem2 = wNumA % wNumB

wAns3 = (wNumA * wNumC) / wNumB

dAns = dNumA / 7

; signed

dAns3 = dNumA * dNumB

dRem1 = dNumA % dNumB

dAns3 = (dNumA * dNumC) / dNumB

; signed

; % is modulus

; signed

qAns = qNumA / 9

; signed

; signed

; % is modulus

; signed

qAns4 = qNumA * qNumB

qRem1 = qNumA % qNumB

qAns3 = (qNumA * qNumC) / qNumB

The following instructions could be used:

; -----

;  example byte operations, unsigned

;

bAns1 = bNumA / 3 (unsigned)

mov

mov

mov

div

mov

al, byte [bNumA]

ah, 0

bl, 3

bl

; al = ax / 3

byte [bAns1], al

; bAns2 = bNumA / bNumB (unsigned)

mov

mov

div

mov

mov

ax, 0

al, byte [bNumA]

byte [bNumB]

byte [bAns2], al

byte [bRem2], ah

; al = ax / bNumB

; ah = ax % bNumB

;

mov

mul

bAns3 = (bNumA * bNumC) / bNumB (unsigned)

al, byte [bNumA]

byte [bNumC]

; result in ax

Page 101

Chapter 7.0 ◄ Instruction Set Overview

div

mov

byte [bNumB]

byte [bAns3], al

; al = ax / bNumB

; -----

;  example word operations, unsigned

;

wAns1 = wNumA / 5 (unsigned)

mov

mov

mov

div

mov

ax, word [wNumA]

dx, 0

bx, 5

bx

; ax = dx:ax / 5

word [wAns1], ax

; wAns2 = wNumA / wNumB (unsigned)

mov

mov

div

mov

mov

dx, 0

ax, word [wNumA]

word [wNumB]

word [wAns2], ax

word [wRem2], dx

; ax = dx:ax / wNumB

;

wAns3 = (wNumA * wNumC) / wNumB (unsigned)

mov

mul

div

mov

ax, word [wNumA]

word [wNumC]

word [wNumB]

; result in dx:ax

; ax = dx:ax / wNumB

word [wAns3], ax

;

-----

;  example double-word operations, signed

; dAns1 = dNumA / 7 (signed)

mov

cdq

mov

eax, dword [dNumA]

ebx, 7

; eax → edx:eax

idiv ebx

mov dword [dAns1], eax

; eax = edx:eax / 7

;

mov

cdq

dAns2 = dNumA / dNumB (signed)

eax, dword [dNumA]

; eax → edx:eax

; eax = edx:eax/dNumB

idiv dword [dNumB]

mov

mov

dword [dAns2], eax

dword [dRem2], edx

; edx = edx:eax%dNumB

Page 102

Chapter 7.0 ◄ Instruction Set Overview

;

dAns3 = (dNumA * dNumC) / dNumB (signed)

mov eax, dword [dNumA]

imul dword [dNumC]

idiv dword [dNumB]

; result in edx:eax

; eax = edx:eax/dNumB

mov

dword [dAns3], eax

;

;

-----

example quadword operations, signed

; qAns1 = qNumA / 9 (signed)

mov

cqo

mov

rax, qword [qNumA]

; rax → rdx:rax

rbx, 9

idiv rbx

mov qword [qAns1], rax

; eax = edx:eax / 9

;

mov

cqo

qAns2 = qNumA / qNumB (signed)

rax, qword [qNumA]

; rax → rdx:rax

; rax = rdx:rax/qNumB

idiv qword [qNumB]

mov

mov

qword [qAns2], rax

qword [qRem2], rdx

; rdx = rdx:rax%qNumB

; qAns3 = (qNumA * qNumC) / qNumB (signed)

mov rax, qword [qNumA]

imul qword [qNumC]

idiv qword [qNumB]

; result in rdx:rax

; rax = rdx:rax/qNumB

mov

qword [qAns3], rax

For some instructions, including those above, the explicit type specification (e.g., byte,

word, dword, qword) is required to clearly define the size.

Page 103

Chapter 7.0 ◄ Instruction Set Overview

The integer division instructions are summarized as follows:

Instruction

Explanation

div <src>

Unsigned divide A/D register (ax, dx:ax,

edx:eax, or rdx:rax) by the <src> operand.

Byte: al = ax / <src>, rem in ah

Word: ax = dx:ax / <src>, rem in dx

Double: eax = eax / <src>, rem in edx

Quad: rax = rax / <src>, rem in rdx

Note, <src> operand cannot be an immediate.

div <op8>

div <op16>

div <op32>

div <op64>

Examples:

div   word [wVvar]

div   bl

div   dword [dVar]

div   qword [qVar]

idiv <src>

Signed divide A/D register (ax, dx:ax,

edx:eax, or rdx:rax) by the <src> operand.

Byte: al = ax / <src>, rem in ah

Word: ax = dx:ax / <src>, rem in dx

Double: eax = eax / <src>, rem in edx

Quad: rax = rax / <src>, rem in rdx

Note, <src> operand cannot be an immediate.

idiv <op8>

idiv <op16>

idiv <op32>

idiv <op64>

Examples:

idiv  word [wVvar]

idiv  bl

idiv  dword [dVar]

idiv  qword [qVar]

A more complete list of the instructions is located in Appendix B.

7

.6 Logical Instructions

This section summarizes some of the more common logical instructions that may be

useful when programming.

Page 104

Chapter 7.0 ◄ Instruction Set Overview

7

.6.1 Logical Operations

As you should recall, below are the truth tables for the basic logical operations;

0

1 0 1

and 0 0 1 1 or 0 0 1 1 xor 0 0 1 1

0 0 1 0 1 1 1 0 1 1 0

0 1 0 1

0 1 0 1

0

Illustration 17: Logical Operations

The logical instructions are summarized as follows:

Instruction

Explanation

and   <dest>, <src>

Perform logical AND operation on two

operands, (<dest> and <src>) and place the

result in <dest> (over-writing previous value).

Note 1, both operands cannot be memory.

Note 2, destination operand cannot be an

immediate.

Examples:

and   ax, bx

and   rcx, rdx

and   eax, dword [dNum]

and   qword [qNum], rdx

or   <dest>, <src>

Perform logical OR operation on two operands,

(<dest> || <src>) and place the result in <dest>

(over-writing previous value).

Note 1, both operands cannot be memory.

Note 2, destination operand cannot be an

immediate.

Examples:

or   ax, bx

or   rcx, rdx

or   eax, dword [dNum]

or   qword [qNum], rdx

Page 105

Chapter 7.0 ◄ Instruction Set Overview

Instruction

Explanation

xor   <dest>, <src>

Perform logical XOR operation on two

operands, (<dest> ^ <src>) and place the

result in <dest> (over-writing previous value).

Note 1, both operands cannot be memory.

Note 2, destination operand cannot be an

immediate.

Examples:

xor   ax, bx

xor   rcx, rdx

xor   eax, dword [dNum]

xor   qword [qNum], rdx

not   <op>

Perform a logical not operation (one's

complement on the operand 1's→0's and

0

's→1's).

Note, operand cannot be an immediate.

Examples:

not   bx

not   rdx

not   dword [dNum]

not   qword [qNum]

The & refers to the logical AND operation, the || refers to the logical OR operation, and

the ^ refers to the logical XOR operation as per C/C++ conventions. The ¬ refers to the

logical NOT operation.

A more complete list of the instructions is located in Appendix B.

7

.6.2 Shift Operations

The shift operation shifts bits within an operand, either left or right. Two typical

reasons for shifting bits include isolating a subset of the bits within an operand for some

specific purpose or possibly for performing multiplication or division by powers of two.

All bits are shifted one position. The bit that is shifted outside the operand is lost and a

0

-bit added at the other side.

7.6.2.1 Logical Shift

The logical shift is a bitwise operation that shifts all the bits of its source register by the

specified number of bits and places the result into the destination register. The bits can

Page 106

Chapter 7.0 ◄ Instruction Set Overview

be shifted left or right as needed. Every bit in the source operand is moved the specified

number of bit positions and the newly vacant bit positions are filled in with zeros.

The following diagram shows how the right and left shift operations work for byte sized

operands.

Shift Right Logical

Shift Left Logical

7

1

6

0

5

1

4

1

3

0

2

0

1

1

0

1

7

1

6

0

5

1

4

1

3

0

2

0

1

1

0

1

0

0

1

0

1

1

0

0

1

0

1

1

0

0

1

1

0

0

Illustration 18: Logical Shift Overview

The logical shift treats the operand as a sequence of bits rather than as a number.

The shift instructions may be used to perform unsigned integer multiplication and

division operations for powers of 2. Powers of two would be 2, 4, 8, etc. up to the limit

of the operand size (32-bits for register operands).

In the examples below, 23 is divided by 2 by performing a shift right logical one bit.

The resulting 11 is shown in binary. Next, 13 is multiplied by 4 by performing a shift

left logical two bits. The resulting 52 is shown in binary.

Shift Right Logical

Unsigned Division

Shift Left Logical

Unsigned Multiplication

0

0

0 0 1 0 1 1 1 = 23

0 0 0 1 0 1 1 = 11

0 0 0 0 1 1 0 1 = 13

0 0 1 1 0 1 0 0 = 52

Illustration 19: Logical Shift Operations

Page 107

Chapter 7.0 ◄ Instruction Set Overview

As can be seen in the examples, a 0 was entered in the newly vacated bit locations on

either the right or left (depending on the operation).

The logical shift instructions are summarized as follows:

Instruction

Explanation

shl   <dest>, <imm>

shl   <dest>, cl

Perform logical shift left operation on

destination operand. Zero fills from right (as

needed).

The <imm> or the value in cl register must be

between 1 and 64.

Note, destination operand cannot be an

immediate.

Examples:

shl   ax, 8

shl   rcx, 32

shl   eax, cl

shl   qword [qNum], cl

shr   <dest>, <imm>

shr   <dest>, cl

Perform logical shift right operation on

destination operand. Zero fills from left (as

needed).

The <imm> or the value in cl register must be

between 1 and 64.

Note, destination operand cannot be an

immediate.

Examples:

shr   ax, 8

shr   rcx, 32

shr   eax, cl

shr   qword [qNum], cl

A more complete list of the instructions is located in Appendix B.

.6.2.2 Arithmetic Shift

7

The arithmetic shift right is also a bitwise operation that shifts all the bits of its source

register by the specified number of bits and places the result into the destination register.

Every bit in the source operand is moved the specified number of bit positions, and the

newly vacant bit positions are filled in. For an arithmetic left shift, the original leftmost

bit (the sign bit) is replicated to fill in all the vacant positions. This is referred to as sign

extension.

Page 108

Chapter 7.0 ◄ Instruction Set Overview

The following diagrams show how the shift left and shift right arithmetic operations

works for a byte sized operand.

Shift Left Arithmetic

7

1

6

0

5

1

4

1

3

0

2

0

1

1

0

1

0

1

1

0

0

1

1

0

0

Illustration 20: Arithmetic Left Shift

The arithmetic left shift moves bits the number of specified places to the left and zero

fills the from the least significant bit position (left). The leading sign bit is not

preserved. The arithmetic left shift can be useful to perform an efficient multiplication

by a power of two. If the resulting value does not fit an overflow is generated.

Shift Right Arithmetic

7

1

6

0

5

1

4

1

3

0

2

0

1

1

0

1

1

1

0

1

1

0

0

1

Illustration 21: Arithmetic Right Shift

The arithmetic right shift moves bits the number of specified places to the right and

treats the operand as a signed number which extends the sign (negative in this example).

Page 109

Chapter 7.0 ◄ Instruction Set Overview

The arithmetic shift rounds always rounds down (towards negative infinity) and the

standard divide instruction truncates (rounds toward 0). As such, the arithmetic shift is

not typically used to replace the signed divide instruction.

The arithmetic shift instructions are summarized as follows:

Instruction

Explanation

sal   <dest>, <imm>

sal   <dest>, cl

Perform arithmetic shift left operation on

destination operand. Zero fills from right (as

needed).

The <imm> or the value in cl register must be

between 1 and 64.

Note, destination operand cannot be an

immediate.

Examples:

sal   ax, 8

sal   rcx, 32

sal   eax, cl

sal   qword [qNum], cl

sar   <dest>, <imm>

sar   <dest>, cl

Perform arithmetic shift right operation on

destination operand. Sign fills from left (as

needed).

The <imm> or the value in cl register must be

between 1 and 64.

Note, destination operand cannot be an

immediate.

Examples:

sar   ax, 8

sar   rcx, 32

sar   eax, cl

sar   qword [qNum], cl

A more complete list of the instructions is located in Appendix B.

7

.6.3 Rotate Operations

The rotate operation shifts bits within an operand, either left or right, with the bit that is

shifted outside the operand is rotated around and placed at the other end.

For example, if a byte operand, 100101102, is rotated to the right 1 place, the result

would be 01001011 . If a byte operand, 10010110 , is rotated to the left 1 place, the

2

2

result would be 001011012.

Page 110

Chapter 7.0 ◄ Instruction Set Overview

The logical shift instructions are summarized as follows:

Instruction

Explanation

rol   <dest>, <imm>

rol   <dest>, cl

Perform rotate left operation on destination

operand.

The <imm> or the value in cl register must be

between 1 and 64.

Note, destination operand cannot be an

immediate.

Examples:

rol   ax, 8

rol   rcx, 32

rol   eax, cl

rol   qword [qNum], cl

ror   <dest>, <imm>

ror   <dest>, cl

Perform rotate right operation on destination

operand.

The <imm> or the value in cl register must be

between 1 and 64.

Note, destination operand cannot be an

immediate.

Examples:

ror   ax, 8

ror   rcx, 32

ror   eax, cl

ror   qword [qNum], cl

A more complete list of the instructions is located in Appendix B.

7

.7 Control Instructions

Program control refers to basic programming structures such as IF statements and

looping.

All of the high-level language control structures must be performed with the limited

assembly language control structures. For example, an IF-THEN-ELSE statement does

not exist at the assembly language level. Assembly language provides an unconditional

branch (or jump) and a conditional branch or an IF statement that will jump to a target

label or not jump.

The control instructions refer to unconditional and conditional jumping. Jumping is

required for basic conditional statements (i.e., IF statements) and looping.

Page 111

Chapter 7.0 ◄ Instruction Set Overview

7

.7.1 Labels

A program label is the target, or a location to jump to, for control statements. For

example, the start of a loop might be marked with a label such as “loopStart”. The code

may be re-executed by jumping to the label.

Generally, a label starts with a letter, followed by letters, numbers, or symbols (limited

to “_”), terminated with a colon (“:”). It is possible to start labels with non-letter

characters (i.e., digits, “_”, “$”, “#”, “@”, “~” or “?”) . However, these typically

convey special meaning and, in general, should not be used by programmers. Labels in

yasm are case sensitive.

For example,

loopStart:

last:

are valid labels. Program labels may be defined only once.

The following sections describe how labels are used.

7

.7.2 Unconditional Control Instructions

The unconditional instruction provides an unconditional jump to a specific location in

the program denoted with a program label. The target label must be defined exactly

once and accessible and within scope from the originating jump instruction.

The unconditional jump instruction is summarized as follows:

Instruction

Explanation

jmp   <label>

Jump to specified label.

Note, label must be defined exactly once.

Examples:

jmp   startLoop

jmp   ifDone

jmp   last

A more complete list of the instructions is located in Appendix B.

7

.7.3 Conditional Control Instructions

The conditional control instructions provide a conditional jump based on a comparison.

This provides the functionality of a basic IF statement.

Page 112

Chapter 7.0 ◄ Instruction Set Overview

Two steps are required for a comparison; the compare instruction and the conditional

jump instruction. The conditional jump instruction will jump or not jump to the

provided label based on the result of the previous comparison operation. The compare

instruction will compare two operands and store the results of the comparison in the

rFlag registers. The conditional jump instruction will act (jump or not jump) based on

the contents of the rFlag register. This requires that the compare instruction is

immediately followed by the conditional jump instruction. If other instructions are

placed between the compare and conditional jump, the rFlag register will be altered and

the conditional jump may not reflect the correct condition.

The general form of the compare instruction is:

cmp  <op1>, <op2>

Where <op1> and <op2> are not changed and must be of the same size. Either, but not

both, may be a memory operand. The <op1> operand cannot be an immediate, but the

<op2> operand may be an immediate value.

The conditional control instructions include the jump equal (je) and jump not equal

(jne) which work the same for both signed and unsigned data.

The signed conditional control instructions include the basic set of comparison

operations; jump less than (jl), jump less than or equal (jle), jump greater than (jg), and

jump greater than or equal (jge).

The unsigned conditional control instructions include the basic set of comparison

operations; jump below than (jb), jump below or equal (jbe), jump above than (ja), and

jump above or equal (jae).

The general form of the signed conditional instructions along with an explanatory

comment are as follows:

je    <label>

jne   <label>

; if <op1> == <op2>

; if <op1> != <op2>

jl    <label>

jle   <label>

jg    <label>

jge   <label>

; signed, if <op1> < <op2>

; signed, if <op1> <= <op2>

; signed, if <op1> > <op2>

; signed; if <op1> >= <op2>

jb    <label>

jbe   <label>

ja    <label>

jae   <label>

; unsigned, if <op1> < <op2>

; unsigned, if <op1> <= <op2>

; unsigned, if <op1> > <op2>

; unsigned, if <op1> >= <op2>

Page 113

Chapter 7.0 ◄ Instruction Set Overview

For example, given the following pseudo-code for signed data:

if (currNum > myMax)

myMax = currNum;

And, assuming the following data declarations:

currNum dq

myMax dq

0

0

Assuming that the values are updating appropriately within the program (not shown),

the following instructions could be used:

mov

cmp

jle

rax, qword [currNum]

rax, qword [myMax]

notNewMax

; if currNum <= myMax

;   skip set new max

mov

qword [myMax], rax

notNewMax:

Note that the logic for the IF statement has been reversed. The compare and conditional

jump provide functionality for jump or not jump. As such, if the condition from the

original IF statement is false, the code must not be executed. Thus, when false, in order

to skip the execution, the conditional jump will jump to the target label immediately

following the code to be skipped (not executed). While there is only one line in this

example, there can be many lines of code.

A more complex example might be as follows:

if (x != 0) {

ans = x / y;

errFlg = FALSE;

}

else {

ans = 0;

errFlg = TRUE;

}

This basic compare and conditional jump do not provide a typical IF-ELSE structure. It

must be created. Assuming the x and y variables are signed double-words that will be

set during the program execution, and the following declarations:

TRUE

FALSE

x

equ

equ

dd

1

0

0

0

y

dd

Page 114

Chapter 7.0 ◄ Instruction Set Overview

ans

errFlg

dd

db

0

FALSE

The following code could be used to implement the above IF-ELSE statement.

cmp

je

mov

cdq

dword [x], 0

doElse

eax, dword [x]

; if statement

idiv dword [y]

mov

mov

jmp

dword [ans], eax

byte [errFlg], FALSE

skpElse

doElse:

mov

dword [ans], 0

byte [errFlg], TRUE

mov

skpElse:

In this example, since the data was signed, a signed division (idiv) and the appropriate

conversion (cdq in this case) were required. It should also be noted that the edx register

was overwritten even though it did not appear explicitly. If a value was previously

placed in edx (or rdx), it has been altered.

7.7.3.1 Jump Out of Range

The target label is referred to as a short-jump. Specifically, that means the target label

must be within 128 bytes from the conditional jump instruction. While this limit is

not typically a problem, for very large loops, the assembler may generate an error

referring to “jump out-of-range”. The unconditional jump (jmp) is not limited in range.

If a “jump out-of-range” is generated, it can be eliminated by reversing the logic and

using an unconditional jump for the long jump. For example, the following code:

cmp

jne

rcx, 0

startOfLoop

might generate a “jump out-of-range” assembler error if the label, startOfLoop, is a long

distance away. The error can be eliminated with the following code:

cmp

je

jmp

rcx, 0

endOfLoop

startOfLoop

endOfLoop:

Page 115

Chapter 7.0 ◄ Instruction Set Overview

Which accomplishes the same thing using an unconditional jump for the long jump and

adding a conditional jump to a very close label.

The conditional jump instructions are summarized as follows:

Instruction

Explanation

cmp   <op1>, <op2>

Compare <op1> with <op2>.

Results are stored in the rFlag register.

Note 1, operands are not changed.

Note 2, both operands cannot be memory.

Note 3, <op1> operand cannot be an

immediate.

Examples:

cmp   rax, 5

cmp   ecx, edx

cmp   ax, word [wNum]

je   <label>

jne   <label>

jl   <label>

Based on preceding comparison instruction,

jump to <label> if <op1> == <op2>.

Label must be defined exactly once.

Examples:

Examples:

cmp   rax, 5

je    wasEqual

Based on preceding comparison instruction,

jump to <label> if <op1> != <op2>.

Label must be defined exactly once.

cmp   rax, 5

jne   wasNotEqual

For signed data, based on preceding

comparison instruction, jump to <label> if

<op1> < <op2>.

Label must be defined exactly once.

Examples:

cmp   rax, 5

jl    wasLess

jle   <label>

For signed data, based on preceding

comparison instruction, jump to <label> if

<op1> <op2>.

Label must be defined exactly once.

Page 116

Chapter 7.0 ◄ Instruction Set Overview

Instruction

Explanation

Examples:

Examples:

Examples:

Examples:

Examples:

cmp   rax, 5

jle    wasLessOrEqual

jg   <label>

jge   <label>

jb   <label>

jbe   <label>

ja   <label>

For signed data, based on preceding

comparison instruction, jump to <label> if

<op1> > <op2>.

Label must be defined exactly once.

cmp   rax, 5

jg    wasGreater

For signed data, based on preceding

comparison instruction, jump to <label> if

<op1> <op2>.

Label must be defined exactly once.

cmp   rax, 5

jge    wasGreaterOrEqual

For unsigned data, based on preceding

comparison instruction, jump to <label> if

<op1> < <op2>.

Label must be defined exactly once.

cmp   rax, 5

jb    wasLess

For unsigned data, based on preceding

comparison instruction, jump to <label> if

<op1> <op2>.

Label must be defined exactly once.

cmp   rax, 5

jbe    wasLessOrEqual

For unsigned data, based on preceding

comparison instruction, jump to <label> if

<op1> > <op2>.

Label must be defined exactly once.

Page 117

Chapter 7.0 ◄ Instruction Set Overview

Instruction

Explanation

Examples:

cmp   rax, 5

ja    wasGreater

jae   <label>

For unsigned data, based on preceding

comparison instruction, jump to <label> if

<op1> <op2>.

Label must be defined exactly once.

Examples:

cmp   rax, 5

jae   wasGreaterOrEqual

A more complete list of the instructions is located in Appendix B.

7

.7.4 Iteration

The basic control instructions outlined provide a means to iterate or loop.

A basic loop can be implemented consisting of a counter which is checked at either the

bottom or top of a loop with a compare and conditional jump.

For example, assuming the following declarations:

lpCnt dq   15

sum

dq   0

The following code would sum the odd integers from 1 to 30:

mov

mov

rcx, qword [lpCnt] ; loop counter

rax, 1

; odd integer counter

sumLoop:

add

qword [sum], rax

rax, 2

rcx

; sum current odd integer

; set next odd integer

; decrement loop counter

add

dec

cmp

jne

rcx, 0

sumLoop

This is just one of many different ways to accomplish the odd integer summation task.

In this example, rcx was used as a loop counter and rax was used for the current odd

integer (appropriately initialized to 1 and incremented by 2).

Page 118

Chapter 7.0 ◄ Instruction Set Overview

The process shown using rcx as a counter is useful when looping a predetermined

number of times. There is a special instruction, loop, provides looping support.

The general format is as follows:

loop   <label>

Which will perform the decrement of the rcx register, comparison to 0, and jump to the

specified label if rcx ≠ 0. The label must be defined exactly once.

As such, the loop instruction provides the same functionality as the three lines of code

from the previous example program. The following sets of code are equivalent:

Code Set 1

Code Set 2

loop  <label>

dec   rcx

cmp   rcx, 0

jne   <label>

For example, the previous program can be written as follows:

mov

mov

rcx, qword [maxN]

rax, 1

; loop counter

; odd integer counter

sumLoop:

add

qword [sum], rax

rax, 2

; sum current odd integer

; set next odd integer

add

loop sumLoop

Both code examples produce the exact same result in the same manner.

Since the rcx register is decremented and then checked, forgetting to set the rcx register

could result in looping an unknown number of times. This is likely to generate an error

during the loop execution, which can be very misleading when debugging.

The loop instruction can be useful when coding, but it is limited to the rcx register and

to counting down. If nesting loops are required, the use of a loop instruction for both

the inner and outer loop can cause a conflict unless additional actions are taken (i.e.,

save/restore rcx register as required for inner loop).

While some of the programming examples in this text will use the loop instruction, it is

not required.

The loop instruction is summarized as follows:

Page 119

Chapter 7.0 ◄ Instruction Set Overview

Instruction

Explanation

loop   <label>

Decrement rcx register and jump to <label> if

rcx is ≠ 0.

Note, label must be defined exactly once.

Examples:

loop  startLoop

loop  ifDone

loop  sumLoop

A more complete list of the instructions is located in Appendix B.

7

.8 Example Program, Sum of Squares

The following is a complete example program to find the sum of squares from 1 to n.

For example, the sum of squares for 10 is as follows:

12

+2 + ⋯ + 10 = 385

2

2

This example main initializes the n value to 10 to match the above example.

;

;

;

;

Simple example program to compute the

sum of squares from 1 to n.

**********************************************

Data declarations

section .data

;

-----

;  Define constants

SUCCESS

SYS_exit

equ

equ

0

60

; Successful operation

; call code for terminate

;

Define Data.

dd

n

10

0

sumOfSquares dq

; *******************************************************

section .text

Page 120

Chapter 7.0 ◄ Instruction Set Overview

global _start

_start:

;

;

;

;

;

-----

Compute sum of squares from 1 to n (inclusive).

Approach:

for (i=1; i<=n; i++)

sumOfSquares += i^2;

mov

mov

rbx, 1

ecx, dword [n]

; i

sumLoop:

mov

rax, rbx

rax

; get i

; i^2

mul

add

inc

qword [sumOfSquares], rax

rbx

loop sumLoop

;

;

-----

Done, terminate program.

last:

mov

rax, SYS_exit

rdi, SUCCESS

; call code for exit

; exit with success

mov

syscall

The debugger can be used to examine the results and verify correct execution of the

program.

7

.9 Exercises

Below are some quiz questions and suggested projects based on this chapter.

7

.9.1 Quiz Questions

Below are some quiz questions based on this chapter.

1

) Which of the following instructions is legal / illegal? As appropriate, provide an

explanation.

1

2

.

.

mov   rax, 54

mov   ax, 54

Page 121

Chapter 7.0 ◄ Instruction Set Overview

3

4

5

6

7

8

9

1

1

1

1

1

1

1

.

.

.

.

.

.

.

mov   al, 354

mov   rax, r11

mov   rax, r11d

mov   54, ecx

mov   rax, qword [qVar]

mov   rax, qword [bVar]

mov   rax, [qVar]

0. mov rax, qVar

1. mov   eax, dword [bVar]

2. mov   qword [qVar2], qword [qVar1]

3. mov   qword [bVar2], qword [qVar1]

4. mov   r15, 54

5. mov   r16, 54

6. mov   r11b, 54

2

3

4

5

) Explain what each of the following instructions does.

1

2

.

.

movzx   rsi, byte [bVar1]

movsx   rsi, byte [bVar1]

) What instruction is used to:

1

2

. convert an unsigned byte in al into a word in ax.

. convert a signed byte in al into a word in ax.

) What instruction is used to:

1

2

. convert an unsigned word in ax into a double-word in eax.

. convert a signed word in ax into a double-word in eax.

) What instruction is used to:

1

2

. convert an unsigned word in ax into a double-word in dx:ax.

. convert a signed word in ax into a double-word in dx:ax.

Page 122

Chapter 7.0 ◄ Instruction Set Overview

6

7

) Explain the difference between the cwd instruction and the movsx instructions.

) Explain why the explicit specification (dword in this example) is required on the

first instruction and is not required on the second instruction.

1

2

.

.

add   dword [dVar], 1

add [dVar], eax

8

9

1

1

1

) Given the following code fragment:

mov rax, 9

mov rbx, 2

add rbx, rax

What would be in the rax and rbx registers after execution? Show answer in

hex, full register size.

) Given the following code fragment:

mov rax, 9

mov rbx, 2

sub rax, rbx

What would be in the rax and rbx registers after execution? Show answer in

hex, full register size.

0) Given the following code fragment:

mov rax, 9

mov rbx, 2

sub rbx, rax

What would be in the rax and rbx registers after execution? Show answer in

hex, full register size.

1) Given the following code fragment:

mov rax, 4

mov rbx, 3

imul rbx

What would be in the rax and rdx registers after execution? Show answer in

hex, full register size.

2) Given the following code fragment:

mov rax, 5

cqo

Page 123

Chapter 7.0 ◄ Instruction Set Overview

mov rbx, 3

idiv rbx

What would be in the rax and rdx registers after execution? Show answer in

hex, full register size.

1

1

3) Given the following code fragment:

mov   rax, 11

cqo

mov   rbx, 4

idiv  rbx

What would be in the rax and rdx registers after execution? Show answer in

hex, full register size.

4) Explain why each of the following statements will not work.

1

2

3

4

.

.

.

.

mov   42, eax

div   3

mov dword [num1], dword [num2]

mov dword [ax], 800

1

1

5) Explain why the following code fragment will not work correctly.

mov eax, 500

mov ebx, 10

idiv ebx

6) Explain why the following code fragment will not work correctly.

mov eax, -500

cdq

mov ebx, 10

div ebx

1

7) Explain why the following code fragment will not work correctly.

mov ax, -500

cwd

mov bx, 10

idiv bx

mov dword [ans], eax

1

8) Under what circumstances can the three-operand multiple be used?

Page 124

Chapter 7.0 ◄ Instruction Set Overview

7

.9.2 Suggested Projects

Below are some suggested projects based on this chapter.

1

) Create a program to compute the following expressions using unsigned byte

variables and unsigned operations. Note, the first letter of the variable name

denotes the size (b → byte and w → word).

1

2

3

4

5

6

7

8

9

1

1

1

1

.

.

.

.

.

.

.

.

.

bAns1 = bNum1 + bNum2

bAns2 = bNum1 + bNum3

bAns3 = bNum3 + bNum4

bAns6 = bNum1 – bNum2

bAns7 = bNum1 – bNum3

bAns8 = bNum2 – bNum4

wAns11 = bNum1 * bNum3

wAns12 = bNum2 * bNum2

wAns13 = bNum2 * bNum4

0. bAns16 = bNum1 / bNum2

1. bAns17 = bNum3 / bNum4

2. bAns18 = wNum1 / bNum4

3. bRem18 = wNum1 % bNum4

Use the debugger to execute the program and display the final results. Create a

debugger input file to show the results in both decimal and hexadecimal.

2

3

) Repeat the previous program using signed values and signed operations. Use the

debugger to execute the program and display the final results. Create a debugger

input file to show the results in both decimal and hexadecimal.

) Create a program to complete the following expressions using unsigned word

sized variables. Note, the first letter of the variable name denotes the size (w →

word and d → double-word).

1

2

.

.

wAns1 = wNum1 + wNum2

wAns2 = wNum1 + wNum3

Page 125

Chapter 7.0 ◄ Instruction Set Overview

3

4

5

6

7

8

9

1

1

1

1

.

.

.

.

.

.

.

wAns3 = wNum3 + wNum4

wAns6 = wNum1 – wNum2

wAns7 = wNum1 – wNum3

wAns8 = wNum2 – wNum4

dAns11 = wNum1 * wNum3

dAns12 = wNum2 * wNum2

dAns13 = wNum2 * wNum4

0. wAns16 = wNum1 / wNum2

1. wAns17 = wNum3 / wNum4

2. wAns18 = dNum1 / wNum4

3. wRem18 = dNum1 % wNum4

Use the debugger to execute the program and display the final results. Create a

debugger input file to show the results in both decimal and hexadecimal.

4

5

) Repeat the previous program using signed values and signed operations. Use the

debugger to execute the program and display the final results. Create a debugger

input file to show the results in both decimal and hexadecimal.

) Create a program to complete the following expressions using unsigned double-

word sized variables. Note, the first letter of the variable name denotes the size

(d → double-word and q → quadword).

1

2

3

4

5

6

7

8

9

.

.

.

.

.

.

.

.

.

dAns1 = dNum1 + dNum2

dAns2 = dNum1 + dNum3

dAns3 = dNum3 + dNum4

dAns6 = dNum1 – dNum2

dAns7 = dNum1 – dNum3

dAns8 = dNum2 – dNum4

qAns11 = dNum1 * dNum3

qAns12 = dNum2 * dNum2

qAns13 = dNum2 * dNum4

Page 126

Chapter 7.0 ◄ Instruction Set Overview

1

1

1

1

0. dAns16 = dNum1 / dNum2

1. dAns17 = dNum3 / dNum4

2. dAns18 = qNum1 / dNum4

3. dRem18 = qNum1 % dNum4

Use the debugger to execute the program and display the final results. Create a

debugger input file to show the results in both decimal and hexadecimal.

6

7

8

) Repeat the previous program using signed values and signed operations. Use the

debugger to execute the program and display the final results. Create a debugger

input file to show the results in both decimal and hexadecimal.

) Implement the example program to compute the sum of squares from 1 to n.

Use the debugger to execute the program and display the final results. Create a

debugger input file to show the results in both decimal and hexadecimal.

) Create a program to compute the square of the sum from 1 to n. Specifically,

compute the sum of integers from 1 to n and then square the value. Use the

debugger to execute the program and display the final results. Create a debugger

input file to show the results in both decimal and hexadecimal.

9

) Create a program to iteratively find the nth Fibonacci number37. The value for n

should be set as a parameter (e.g., a programmer defined constant). The formula

for computing Fibonacci is as follows:

n

if n=0 or n=1

if n≥2

fibonacci(n) =

fibonacci(n−2)+

{

fibonacci (n−1)

Use the debugger to execute the program and display the final results. Test the

program for various values of n. Create a debugger input file to show the results

in both decimal and hexadecimal.

3

7 For more information, refer to: http://en.wikipedia.org/wiki/Fibonacci_number

Page 127

Chapter 7.0 ◄ Instruction Set Overview

Page 128

Chapter

8

Why did the programmer quit his job?

Because he didn't get arrays.

8.0 Addressing Modes

This chapter provides some basic information regarding addressing modes and the

associated address manipulations on the x86-64 architecture.

The addressing modes are the supported methods for accessing a value in memory using

the address of a data item being accessed (read or written). This might include the name

of a variable or the location in an array.

The basic addressing modes are:

•

•

•

Register

Immediate

Memory

Each of these modes is described with examples in the following sections. Additionally,

a simple example for accessing an array is presented.

8

.1 Addresses and Values

On a 64-bit architecture, addresses require 64-bits.

As noted in the previous chapter, the only way to access memory is with the brackets

([]'s). Omitting the brackets will not access memory and instead obtain the address of

the item. For example:

mov

mov

rax, qword [var1]

rax, var1

; value of var1 in rax

; address of var1 in rax

Since omitting the brackets is not an error, the assembler will not generate error

messages or warnings.

Page 129

Chapter 8.0 ◄ Addressing Modes

When accessing memory, in many cases the operand size is clear. For example, the

instruction

mov

eax, [rbx]

moves a double-word from memory. However, for some instructions the size can be

ambiguous. For example,

inc [rbx]

; error

is ambiguous since it is not clear if the memory being accessed is a byte, word, or

double-word. In such a case, operand size must be specified with either the byte, word,

or dword, qword size qualifier. For example,

inc

inc

inc

byte [rbx]

word [rbx]

dword [rbx]

each instruction requires the size specification in order to be clear and legal.

8

.1.1 Register Mode Addressing

Register mode addressing means that the operand is a CPU register (eax, ebx, etc.). For

example:

mov   eax, ebx

Both eax and ebx are in register mode addressing.

8

.1.2 Immediate Mode Addressing

Immediate mode addressing means that the operand is an immediate value. For

example:

mov

eax, 123

The destination operand, eax , is register mode addressing. The 123 is immediate mode

addressing. It should be clear that the destination operand in this example cannot be

immediate mode.

8

.1.3 Memory Mode Addressing

Memory mode addressing means that the operand is a location in memory (accessed via

an address). This is referred to as indirection or dereferencing.

Page 130

Chapter 8.0 ◄ Addressing Modes

The most basic form of memory mode addressing has been used extensively in the

previous chapter. Specifically, the instruction:

mov rax, qword [qNum]

Will access the memory location of the variable qNum and retrieve the value stored

there. This requires that the CPU wait until the value is retrieved before completing the

operation and thus might take slightly longer to complete than a similar operation using

an immediate value.

When accessing arrays, a more generalized method is required. Specifically, an address

can be placed in a register and indirection performed using the register (instead of the

variable name).

For example, assuming the following declaration:

lst

dd

101, 103, 105, 107

The decimal value of 101 is 0x00000065 in hex. The memory picture would be as

follows:

Value

Address

0x6000ef

0x6000ee

0x6000ed

0x6000ec

0x6000eb

0x6000ea

0x6000e9

0x6000e8

0x6000e7

0x6000e6

0x6000e5

0x6000e4

0x6000e3

0x6000e2

0x6000e1

0x6000e0

Offset

lst + 15

lst + 14

lst + 13

lst + 12

lst + 11

lst + 10

lst + 9

lst + 8

lst + 7

lst + 6

lst + 5

lst + 4

lst + 3

lst + 2

lst + 1

lst + 0

Index

lst[3]

lst[2]

lst[1]

lst[0]

0

0

0

6

0

0

0

6

0

0

0

6

0

0

0

0

0

0

b

0

0

0

9

0

0

0

7

0

0

0

lst →

65

Page 131

Chapter 8.0 ◄ Addressing Modes

The first element of the array could be accessed as follows:

mov

eax, dword [lst]

Another way to access the first element is as follows:

mov

mov

rbx, list

eax, dword [rbx]

In this example, the starting address, or base address, of the list is placed in rbx (first

line) and then the value at that address is accessed and placed in the rax register (second

line). This allows us to easily access other elements in the array.

Recall that memory is “byte addressable”, which means that each address is one byte of

information. A double-word variable is 32-bits or 4 bytes so each array element uses 4

bytes of memory. As such, the next element (103) is the starting address (lst) plus 4,

and the next element (105) is the starting address (lst) 8.

Increasing the offset by 4 for each successive element. A list of bytes would increase by

1

, a list of words would increase by 2, a list of double-words would increase by 4, and a

list of quadwords would increase by 8.

The offset is the amount added to the base address. The index is the array element

number as used in a high-level language.

There are several ways to access the array elements. One is to use a base address and

add a displacement. For example, given the initializations:

mov

mov

rbx, lst

rsi, 8

Each of the following instructions access the third element (105 in the above list).

mov

mov

mov

eax, dword [lst+8]

eax, dword [rbx+8]

eax, dword [rbx+rsi]

In each case, the starting address plus 8 was accessed and the value of 105 placed in the

eax register. The displacement is added and the memory location accessed while none

of the source operand registers (rbx, rsi) are altered. The specific method used is up to

the programmer.

In addition, the displacement may be computed in more complex ways.

Page 132

Chapter 8.0 ◄ Addressing Modes

The general format of memory addressing is as follows:

[ baseAddr + (indexReg * scaleValue ) + displacement ]

Where baseAddr is a register or a variable name. The indexReg must be a register. The

scaleValue is an immediate value of 1, 2, 4, 8 (1 is legal, but not useful). The

displacement must be an immediate value. The total represents a 64-bit address.

Elements may be used in any combination, but must be legal and result in a valid

address.

Some example of memory addressing for the source operand are as follows:

mov

mov

mov

mov

mov

mov

mov

eax, dword [var1]

rax, qword [rbx+rsi]

ax, word [lst+4]

bx, word [lst+rdx+2]

rcx, qword [lst+(rsi*8)]

al, byte [buff-1+rcx]

eax, dword [rbx+(rsi*4)+16]

For example, to access the 3rd element of the previously defined double-word array

(which is index 2 since index's start at 0):

mov

mov

rsi, 2

eax, dword [lst+rsi*4]

; index=2

; get lst[2]

Since addresses are always qword (on a 64-bit architecture), a 64-bit register is used for

the memory mode addressing (even when accessing double-word values). This allows a

register to be used more like an array index (from a high-level language).

For example, the memory operand, [lst+rsi*4], is analogous to lst[rsi] from a high-level

language. The rsi register is multiplied by the data size (4 in this example since each

element is 4 bytes).

8

.2 Example Program, List Summation

The following example program will sum the numbers in a list.

;

;

Simple example to the sum and average for

a list of numbers.

;

;

*****************************************************

Data declarations

Page 133

Chapter 8.0 ◄ Addressing Modes

section .data

; -----

;  Define constants

EXIT_SUCCESS equ 0

SYS_exit equ 60

; successful operation

; call code for terminate

;

;

-----

Define Data.

section .data

lst

len

sum

dd

dd

dd

1002, 1004, 1006, 1008, 10010

5

0

; ********************************************************

section .text

global _start

_start:

;

-----

;  Summation loop.

mov

mov

ecx, dword [len]

rsi, 0

; get length value

; index=0

sumLoop:

mov

eax, dword [lst+(rsi*4)]

dword [sum], eax

rsi

; get lst[rsi]

; update sum

; next item

add

inc

loop sumLoop

;

;

-----

Done, terminate program.

last:

mov

rax, SYS_exit

rdi, EXIT_SUCCESS

; call code for exit

; exit with success

mov

syscall

The ()'s within the [ ]'s are not required and added only for clarity. As such, the [lst+

(rsi*4)], is exactly the same as [lst+rsi*4].

Page 134

Chapter 8.0 ◄ Addressing Modes

8

.3 Example Program, Pyramid Areas and Volumes

This example is a simple assembly language program to calculate some geometric

information for each square pyramid in a series of square

pyramids. Specifically, the program will find the lateral total

surface area (including the base) and volume of each square

pyramid in a set of square pyramids.

Once the values are computed, the program finds the

minimum, maximum, sum, and average for the total surface

areas and volumes.

All data are unsigned values (i.e., uses mul and div, not imul

or idiv).

This basic approach used in this example is the loop to calculate the surface areas and

volumes arrays. A second loop is used to find the sum, minimum, and maximum for

each array. To find the minimum and maximum values, the minimum and maximum

variables are each initialized to the first value in the list. Then, every element in the list

is compared to the current minimum and maximum. If the current value from the list is

less than the current minimum, the minimum is set to the current value (over-writing the

previous value). When all values have been checked, the minimum will represent the

true minimum from the list. If the current value from the list is more than the current

maximum, the maximum is set to the current value (over-writing the previous value).

When all values have been checked, the maximum will represent the true maximum

from the list.

;  Example assembly language program to calculate the

;  geometric information for each square pyramid in

;  a series of square pyramids.

;

;

The program calculates the total surface area

and volume of each square pyramid.

;  Once the values are computed, the program finds

;  the minimum, maximum, sum, and average for the

;  total surface areas and volumes.

;

;

;

;

-----

Formulas:

totalSurfaceAreas(n) = aSides(n) *

(2*aSides(n)*sSides(n))

Page 135

Chapter 8.0 ◄ Addressing Modes

;

;

volumes(n) = (aSides(n)^2 * heights(n)) / 3

*************************************************

section .data

;

-----

;  Define constants

EXIT_SUCCESS equ

SYS_exit equ

0

60

; successful operation

; call code for terminate

;

;

-----

Provided Data

aSides

db    10,    14,    13,    37,    54

db    31,    13,    20,    61,    36

db    14,    53,    44,    19,    42

db    27,    41,    53,    62,    10

db    19,    18,    14,    10,    15

db    15,    11,    22,    33,    70

db    15,    23,    15,    63,    26

db    24,    33,    10,    61,    15

db    14,    34,    13,    71,    81

db    38,    13,    29,    17,    93

sSides

dw  1233,  1114,  1773,  1131,  1675

dw  1164,  1973,  1974,  1123,  1156

dw  1344,  1752,  1973,  1142,  1456

dw  1165,  1754,  1273,  1175,  1546

dw  1153,  1673,  1453,  1567,  1535

dw  1144,  1579,  1764,  1567,  1334

dw  1456,  1563,  1564,  1753,  1165

dw  1646,  1862,  1457,  1167,  1534

dw  1867,  1864,  1757,  1755,  1453

dw  1863,  1673,  1275,  1756,  1353

heights dd 14145, 11134, 15123, 15123, 14123

dd 18454, 15454, 12156, 12164, 12542

dd 18453, 18453, 11184, 15142, 12354

dd 14564, 14134, 12156, 12344, 13142

dd 11153, 18543, 17156, 12352, 15434

Page 136

Chapter 8.0 ◄ Addressing Modes

dd 18455, 14134, 12123, 15324, 13453

dd 11134, 14134, 15156, 15234, 17142

dd 19567, 14134, 12134, 17546, 16123

dd 11134, 14134, 14576, 15457, 17142

dd 13153, 11153, 12184, 14142, 17134

length

dd 50

taMin

taMax

taSum

taAve

dd 0

dd 0

dd 0

dd 0

volMin

volMax

volSum

volAve

dd 0

dd 0

dd 0

dd 0

;

;

-----

Additional variables

ddTwo

dd 2

ddThree dd 3

;

;

-------------------------------------------------

Uninitialized data

section .bss

totalAreas

volumes

resd 50

resd 50

; *************************************************

section .text

global _start

_start:

;  Calculate volume, lateral and total surface areas

mov

mov

ecx, dword [length]

rsi, 0

; length counter

; index

calculationLoop:

Page 137

Chapter 8.0 ◄ Addressing Modes

;

totalAreas(n) = aSides(n) * (2*aSides(n)*sSides(n))

movzx r8d, byte [aSides+rsi]

movzx r9d, word [sSides+rsi*2]

; aSides[i]

; sSides[i]

mov

mul

mul

mul

mov

eax, r8d

dword [ddTwo]

r9d

r8d

dword [totalAreas+rsi*4], eax

;

volumes(n) = (aSides(n)^2 * heights(n)) / 3

movzx eax, byte [aSides+rsi]

mul

mul

div

mov

eax

dword [heights+rsi*4]

dword [ddThree]

dword [volumes+rsi*4], eax

inc

rsi

loop calculationLoop

;

;

;

-----

Find min, max, sum, and average for the total

areas and volumes.

mov

mov

mov

eax, dword [totalAreas]

dword [taMin], eax

dword [taMax], eax

mov

mov

mov

eax, dword [volumes]

dword [volMin], eax

dword [volMax], eax

mov

mov

dword [taSum], 0

dword [volSum], 0

mov

mov

ecx, dword [length]

rsi, 0

statsLoop:

mov

eax, dword [totalAreas+rsi*4]

dword [taSum], eax

add

Page 138

Chapter 8.0 ◄ Addressing Modes

cmp

jae

mov

eax, dword [taMin]

notNewTaMin

dword [taMin], eax

notNewTaMin:

cmp

jbe

mov

eax, dword [taMax]

notNewTaMax

dword [taMax], eax

notNewTaMax:

mov

add

cmp

jae

mov

eax, dword [volumes+rsi*4]

dword [volSum], eax

eax, dword [volMin]

notNewVolMin

dword [volMin], eax

notNewVolMin:

cmp

jbe

mov

eax, dword [volMax]

notNewVolMax

dword [volMax], eax

notNewVolMax:

inc

rsi

loop statsLoop

;

;

-----

Calculate averages.

mov

mov

div

mov

eax, dword [taSum]

edx, 0

dword [length]

dword [taAve], eax

mov

mov

div

mov

eax, dword [volSum]

edx, 0

dword [length]

dword [volAve], eax

; -----

;  Done, terminate program.

last:

mov

rax, SYS_exit

; call code for exit

Page 139

Chapter 8.0 ◄ Addressing Modes

mov

rdi, EXIT_SUCCESS

; exit with success

syscall

This is one example. There are multiple other valid approaches to solving this problem.

8

.4 Exercises

Below are some quiz questions and suggested projects based on this chapter.

8

.4.1 Quiz Questions

Below are some quiz questions based on this chapter.

1

) Explain the difference between the following two instructions:

1

.

mov   rdx, qword [qVar1]

2. mov   rdx, qVar1

2

) What is the address mode of the source operand for each of the instructions listed

below. Respond with Register, Immediate, Memory, or Illegal Instruction.

Note, mov <dest>, <source>

mov

mov

mov

mov

mov

mov

mov

mov

ebx, 14

ecx, dword [rbx]

byte [rbx+4], 10

10, rcx

dl, ah

ax, word [rsi+4]

cx, word [rbx+rsi]

ax, byte [rbx]

3

) Given the following variable declarations and code fragment:

ans1   dd   7

mov

mov

add

rax, 3

rbx, ans1

eax, dword [rbx]

Page 140

Chapter 8.0 ◄ Addressing Modes

What would be in the eax register after execution? Show answer in hex, full

register size.

4

) Given the following variable declarations and code fragment:

list1    dd   2, 3, 4, 5, 6, 7

mov rbx, list1

add rbx, 4

mov eax, dword [rbx]

mov edx, dword [list1]

What would be in the eax and edx registers after execution? Show answer in

hex, full register size.

5

) Given the following variable declarations and code fragment:

lst

dd   2, 3, 5, 7, 9

mov

mov

mov

rsi, 4

eax, 1

rcx, 2

lp:   add

eax, dword [lst+rsi]

rsi, 4

lp

add

loop

mov

ebx, dword [lst]

What would be in the eax, ebx, rcx, and rsi registers after execution? Show

answer in hex, full register size. Note, pay close attention to the register sizes

(32-bit vs. 64-bit).

6

) Given the following variable declarations and code fragment:

list    dd   8, 6, 4, 2, 1, 0

mov

mov

mov

mov

rbx, list

rsi, 1

rcx, 3

edx, dword [rbx]

lp:   mov

eax, dword [list+rsi*4]

rsi

lp

inc

loop

imul

dword [list]

Page 141

Chapter 8.0 ◄ Addressing Modes

What would be in the eax, edx, rcx, and rsi registers after execution? Show

answer in hex, full register size. Note, pay close attention to the register sizes

(32-bit vs. 64-bit).

7

) Given the following variable declarations and code fragment:

list    dd   8, 7, 6, 5, 4, 3, 2, 1, 0

mov   rbx, list

mov   rsi, 0

mov   rcx, 3

mov   edx, dword [rbx]

lp: add   eax, dword [list+rsi*4]

inc   rsi

loop lp

cdq

idiv dword [list]

What would be in the eax, edx, rcx, and rsi registers after execution? Show

answer in hex, full register size. Note, pay close attention to the register sizes

(32-bit vs. 64-bit).

8

) Given the following variable declarations and code fragment:

list    dd  2, 7, 4, 5, 6, 3

mov

mov

mov

mov

mov

add

add

rbx, list

rsi, 1

rcx, 2

eax, 0

edx, dword [rbx+4]

eax, dword [rbx+rsi*4]

rsi, 2

lp:

loop lp

imul dword [rbx]

What would be in the eax, edx, rcx, and rsi registers after execution? Show

answer in hex, full register size. Note, pay close attention to the register sizes

(32-bit vs. 64-bit).

Page 142

Chapter 8.0 ◄ Addressing Modes

8

.4.2 Suggested Projects

Below are some suggested projects based on this chapter.

1

) Implement the example program to sum a list of numbers. Use the debugger to

execute the program and display the final results. Create a debugger input file to

show the results.

2

) Update the example program from the previous question to find the maximum,

minimum, and average for the list of numbers. Use the debugger to execute the

program and display the final results. Create a debugger input file to show the

results.

3

4

) Implement the example program to compute the lateral total surface area

(including the base) and volume of each square pyramid in a set of square

pyramids. Once the values are computed, the program finds the minimum,

maximum, sum, and average for the total surface areas and volumes. Use the

debugger to execute the program and display the final results. Create a debugger

input file to show the results.

) Write an assembly language program to find the minimum, middle value,

maximum, sum, and integer average of a list of numbers. Additionally, the

program should also find the sum, count, and integer average for the negative

numbers. The program should also find the sum, count, and integer average for

the numbers that are evenly divisible by 3. Unlike the median, the 'middle value'

does not require the numbers to be sorted. Note, for an odd number of items, the

middle value is defined as the middle value. For an even number of values, it is

the integer average of the two middle values. Assume all data is unsigned. Use

the debugger to execute the program and display the final results. Create a

debugger input file to show the results.

5

) Repeat the previous program using signed values and signed operations. Use the

debugger to execute the program and display the final results. Create a debugger

input file to show the results.

Page 143

Chapter 8.0 ◄ Addressing Modes

6

) Create a program to sort a list of numbers. Use the following bubble sort38

algorithm:

for ( i = (len-1) to 0 ) {

}

swapped = false

for ( j = 0 to i-1 )

if ( lst(j) > lst(j+1) ) {

tmp = lst(j)

lst(j) = lst(j+1)

lst(j+1) = tmp

swapped = true

}

if ( swapped = false ) exit

Use the debugger to execute the program and display the final results. Create a

debugger input file to show the results.

3

8 For more information, refer to: http://en.wikipedia.org/wiki/Bubble_sort

Page 144

Chapter

9

A programmer is heading out to the grocery

store, and is asked to "get a gallon of milk,

and if they have eggs, get a dozen."

He returns with 12 gallons of milk.

9.0 Process Stack

In a computer, a stack is a type of data structure where items are added and then

removed from the stack in reverse order. That is, the most recently added item is the

very first one that is removed. This is often referred to as Last-In, First-Out (LIFO).

A stack is heavily used in programming for the storage of information during procedure

or function calls. The following chapter provides information and examples regarding

the stack.

Adding an item to a stack is referred to as a push or push operation. Removing an item

from a stack is referred to as a pop or pop operation.

It is expected that the reader will be familiar with the general concept of a stack.

9

.1 Stack Example

To demonstrate the general usage of the stack, given an array, a = {7, 19, 37},

consider the operations:

push a[0]

push a[1]

push a[2]

Followed by the operations:

pop

pop

pop

a[0]

a[1]

a[2]

The initial push will push the 7, followed by the 19, and finally the 37. Since the stack

is last-in, first-out, the first item popped off the stack will be the last item pushed, or 37

in this example. The 37 is placed in the first element of the array (over-writing the 7).

As this continues, the order of the array elements is reversed.

Page 145

Chapter 9.0 ◄ Process Stack

The following diagram shows the progress and the results.

stack

stack

stack

stack

stack

stack

empty

37

19

19

7

19

7

7

7

7

push

a[0]

push

a[1]

push

a[2]

pop

a[0]

pop

a[1]

pop

a[2]

a = {7, a = {7, a = {7, a =

9, 37} 19, 37} 19, 37} {37,

9, 37} 19, 37} 19, 7}

a =

{37,

a =

{37,

1

1

The following sections provide more detail regarding the stack implementation and

applicable stack operations and instructions.

9

.2 Stack Instructions

A push operation puts things onto the stack, and a pop operation takes things off the

stack. The format for these commands is:

push <operand64>

pop

<operand64>

The operand can be a register or memory, but an immediate is not allowed. In general,

push and pop operations will push the architecture size. Since the architecture is 64-bit,

we will push and pop quadwords.

The stack is implemented in reverse in memory. Refer to the following sections for a

detailed explanation of why.

Page 146

Chapter 9.0 ◄ Process Stack

The stack instructions are summarized as follows:

Instruction

Explanation

Push the 64-bit operand on the stack.

push   <op64>

First, adjusts rsp accordingly (rsp-8) and then

copy the operand to [rsp]. The operand may

not be an immediate value. Operand is not

changed.

Examples:

Examples:

push  rax

push  qword [qVal] ; value

push  qVal ; address

pop   <op64>

Pop the 64-bit operand from the stack. Adjusts

rsp accordingly (rsp+8). The operand may not

be an immediate value. Operand is

overwritten.

pop  rax

pop  qword [qVal]

pop  rsi

If more than 64-bits must be pushed, multiple push operations would be required. While

it is possible to push and pop operands less than 64-bits, it is not recommended.

A more complete list of the instructions is located in Appendix B.

9

.3 Stack Implementation

The rsp register is used to point to the current top of stack in memory. In this

architecture, as with most, the stack is implemented growing downward in memory.

9

.3.1 Stack Layout

As noted in Chapter 2, Architecture, the general memory layout for a program is as

follows:

Page 147

Chapter 9.0 ◄ Process Stack

high memory

stack

.

.

.

. . available memory . . .

.

.

heap

uninitialized data

data

text (code)

reserved

low memory

Illustration 22: Process Memory Layout

The heap is where dynamically allocated data will be placed (if requested). For

example, items allocated with the C++ new operator or the C malloc() system call. As

dynamically allocated data is created (at run-time), the heap typically grows upward.

However, the stack starts in high memory and grows downward. The stack is used to

temporarily store information such as call frames for function calls. A large program or

a recursive function may use a significant amount of stack space.

As the heap and stack expand, they grow toward each other. This is done to ensure the

most effective overall use of memory.

A program (Process A) that uses a significant amount of stack space and a minimal

amount of heap space will function. A program (Process B) that uses a minimal amount

of stack space and a very large amount of heap space will also function.

Page 148

Chapter 9.0 ◄ Process Stack

For example:

Process A

stack

Process B

stack

heap

bss

heap

bss

data

data

text (code)

reserved

text (code)

reserved

Illustration 23: Process Memory Layout Example

Of course, if the stack and heap meet, the program will crash. If that occurs, there is no

memory available.

9

.3.2 Stack Operations

The basic stack operations of push and pop adjust the stack pointer register, rsp, during

their operation.

For a push operation:

1

2

. The rsp register is decreased by 8 (1 quadword).

. The operand is copied to the stack at [rsp].

The operand is not altered. The order of these operations is important.

For a pop operation:

1

2

. The current top of the stack, at [rsp], is copied into the operand.

. The rsp register is increased by 8 (1 quadword).

The order of these operations is the exact reverse of the push. The item popped is not

actually deleted. However, the programmer cannot count on the item remaining on the

Page 149

Chapter 9.0 ◄ Process Stack

stack after the pop operation. Previously pushed, but not popped, items can be accessed.

For example:

mov

rax, 6700

; 6700  = 00001A2C

10

16

push rax

mov

rax, 31

;   31  = 0000001F

10

16

push rax

Would produce the following stack configuration (where each box is a byte):

.

. .

0

0

0

0

0

0

0

0

0

0

0

0

1

A

C

2

00

00

00

00

00

00

00

rsp → 1F

.

. .

.

.

. .

. .

The layout shows the architecture is little-endian in that the least significant byte is

placed into the lowest memory location.

Page 150

Chapter 9.0 ◄ Process Stack

9

.4 Stack Example

The following is an example program to use the stack to reverse a list of quadwords in

place. Specifically, each value in a quadword array is placed on the stack in the first

loop. In the second loop, each element is removed from the stack and placed back into

the array (over-writing) the previous value.

; Simple example demonstrating basic stack operations.

;

;

;

Reverse a list of numbers - in place.

Method: Put each number on stack, then pop each number

back off, and then put back into memory.

;

;

*****************************************************

Data declarations

section .data

; -----

;  Define constants

EXIT_SUCCESS equ

SYS_exit equ

0

60

; successful operation

; call code for terminate

;

;

-----

Define Data.

numbers

len

dq

dq

121, 122, 123, 124, 125

5

; ****************************************************

section .text

global _start

_start:

;  Loop to put numbers on stack.

mov

mov

mov

mov

rcx, qword [len]

rbx, numbers

r12, 0

rax, 0

Page 151

Chapter 9.0 ◄ Process Stack

pushLoop:

push qword [rbx+r12*8]

inc

r12

loop pushLoop

;

;

;

;

-----

All the numbers are on stack (in reverse order).

Loop to get them back off.   Put them back into

the original list...

mov

mov

mov

rcx, qword [len]

rbx, numbers

r12, 0

popLoop:

pop

rax

mov

inc

qword [rbx+r12*8], rax

r12

loop popLoop

;

;

-----

Done, terminate program.

last:

mov

rax, SYS_exit

rdi, EXIT_SUCCESS

; call code for exit

; exit with success

mov

syscall

There are other ways to accomplish this function (reversing a list), however this is

meant to demonstrate the stack operations.

9

.5 Exercises

Below are some quiz questions and suggested projects based on this chapter.

9

.5.1 Quiz Questions

Below are some quiz questions based on this chapter.

1

2

3

) Which register refers to the top of the stack?

) What happens as a result of a push rax instruction (two things)?

) How many bytes of data does the pop rax instruction remove from the stack?

Page 152

Chapter 9.0 ◄ Process Stack

4

) Given the following code fragment:

mov

mov

mov

r10, 1

r11, 2

r12, 3

push r10

push r11

push r12

pop

pop

pop

r10

r11

r12

What would be in the r10 , r11, and r12 registers after execution? Show answer

in hex, full register size.

5

) Given the following variable declarations and code fragment:

lst

dq   1, 3, 5, 7, 9

mov

mov

rsi, 0

rcx, 5

lp1: push qword [lst+rsi*8]

inc rsi

loop lp1

mov

mov

lp2: pop

inc

rsi, 0

rcx, 5

qword [lst+rsi*8]

rsi

loop lp2

mov

rbx, qword [lst]

Explain what would be the result of the code (after execution)?

6

) Provide one advantage to the stack growing downward in memory.

9

.5.2 Suggested Projects

Below are some suggested projects based on this chapter.

1

) Implement the example program to reverse a list of numbers. Use the debugger

to execute the program and display the final results. Create a debugger input file

to show the results.

Page 153

Chapter 9.0 ◄ Process Stack

2

) Create a program to determine if a NULL terminated string representing a word

is a palindrome39. A palindrome is a word that reads the same forward or

backwards. For example, “anna”, “civic”, “hannah”, “kayak”, and “madam” are

palindromes. This can be accomplished by pushing the characters on the stack

one at a time and then comparing the stack items to the string starting from the

beginning. Use the debugger to execute the program and display the final

results. Create a debugger input file to show the results.

3

) Update the previous program to test if a phrase is a palindrome. The general

approach using the stack is the same, however, spaces and punctuation must be

skipped. For example, “A man, a plan, a canal – Panama!” is a palindrome. The

program must ignore the comma, dash, and exclamation point. Use the debugger

to execute the program and display the final results. Create a debugger input file

to show the results.

3

9 For more information, refer to: http://en.wikipedia.org/wiki/Palindrome

Page 154

Chapter

CAPS LOCK – Preventing login since 1980.

1

0

10.0 Program Development

Writing or developing programs is easier when following a clear methodology. The

main steps in the methodology are:

●

●

●

●

Understand the Problem

Create the Algorithm

Implement the Program

Test/Debug the Program

To help demonstrate this process in detail, these steps will be applied to a simple

example problem in the following sections.

1

0.1 Understand the Problem

Before attempting to create a solution, it is important to fully understand the problem.

Ensuring a complete understanding of the problem can help reduce errors and save time

and effort. The first step is to understand what is required, especially the applicable

input information and expected results or output.

Consider the problem of converting a single integer number into a string or series of

characters representing that integer. To be clear, an integer can be used for numeric

calculations, but cannot be displayed to the console (as it is). A string can be displayed

to the console but not used in numeric calculations.

For this example, only unsigned (positive only) values will be considered. The small

extra effort to address signed values is left to the reader as an exercise.

As an unsigned double-word integer, the numeric value 1498 would be represented as

10

0

x000005DA in hex (double-word sized). The integer number 1498 (0x000005DA)

10

would be represented by the string “1”, “4”, “9”, “8” with a NULL termination. This

would require a total of 5 bytes since there is no sign or leading spaces required for this

specific example. As such, the string “1498” would be represented as follows:

Page 155

Chapter 10.0 ◄ Program Development

Character

“1” “4” “9” “8” NULL

ASCII Value (decimal) 49

52

57

56

0

ASCII Value (hex)

0x31 0x34 0x39 0x38 0x0

The goal is to convert the single integer number into the appropriate series of characters

to form a NULL terminated string.

1

0.2 Create the Algorithm

The algorithm is the name for the unambiguous, ordered sequence of steps involved in

solving the problem. Once the program is understood, a series of steps can be

developed to solve that problem. There can be, and usually are, multiple correct

solutions to a given problem.

The process for creating an algorithm can be different for different people. In general,

some time should be devoted to thinking about possible solutions. This may involve

working on some possible solutions using a scratch piece of paper. Once an approach is

selected, that solution can be developed into an algorithm. The algorithm should be

written down, reviewed, and refined. The algorithm is then used as the outline of the

program.

For example, we will consider the integer to ASCII conversion problem outlined in the

previous section. To convert a single digit integer (0-9) into a character, 48 (or “0” or

10

0

x30) can be added to the integer. For example, 0x01 + 0x30 is 0x31 which is the

ASCII value of “1”. It should be obvious that this trick will only work for single digit

numbers (0-9).

In order to convert a larger integer ( 10) into a string, the integer must be broken into

its component digits. For example, 123 (0x7B) would be 1, 2, and 3. This can be

10

accomplished by repeatedly performing integer division by 10 until a 0 result is

obtained.

For example;

Page 156

Chapter 10.0 ◄ Program Development

1

1

23

0

=

12 remainder3

1

1

2

0

=

1

remainder 2

remainder 1

1

1

=

0

0

As can be seen, the remainder represents the individual digits. However, they are

obtained in reverse order. To address this, the program can push the remainder and,

when done dividing, pop the remainders and convert to ASCII and store in a string

(which is an array of bytes).

This process forms the basis for the algorithm. It should be noted, that there are many

ways to develop this algorithm. One such approach is shown as follows:

;

;

;

;

;

;

;

;

Part A - Successive division

digitCount = 0

get integer

divideLoop:

divide number by 10

push remainder

increment digitCount

if (result > 0) goto divideLoop

;

;

;

;

;

;

;

;

;

;

;

Part B – Convert remainders and store

get starting address of string (array of bytes)

idx = 0

popLoop:

pop intDigit

charDigit = intDigit + “0” (0x030)

string[idx] = charDigit

increment idx

decrement digitCount

if (digitCount > 0) goto popLoop

string[idx] = NULL

The algorithm steps are shown as program comments for convenience. The algorithm is

typically started on paper and then more formally written in pseudo-code as shown

Page 157

Chapter 10.0 ◄ Program Development

above. In the unlikely event the program does not work the first time, the comments are

the primary debugging checklist.

Some programmers skip the comments and will end up spending much more time

debugging. The commenting represents the algorithm and the code is the

implementation of that algorithm.

1

0.3 Implement the Program

Based on the algorithm, a program can be developed and implemented. The algorithm

is expanded and the code added based on the steps outlined in the algorithm. This

allows the programmer to focus on the specific issues for the current section being

coded including the data types and data sizes. This example addresses only unsigned

data so the unsigned divide (DIV, not IDIV) is used. Since the integer is a double-word,

it must be converted into a quadword for the division. However, the result and the

remainder after division will also be a double-words. Since the stack is quadwords, the

entire quadword register will be pushed. The upper-order portion of the register will not

be accessed, so its contents are not relevant.

One possible implementation of the algorithm is as follows:

;

;

Simple example program to convert an

integer into an ASCII string.

;

;

*********************************************************

Data declarations

section .data

; -----

;  Define constants

NULL

EXIT_SUCCESS

SYS_exit

equ

equ

equ

0

0

60

; successful operation

; code for terminate

;

-----

;  Define Data.

intNum

dd

1498

section .bss

Page 158

Chapter 10.0 ◄ Program Development

strNum

resb 10

; *********************************************************

section .text

global _start

_start:

; Convert an integer to an ASCII string.

; -----

;  Part A - Successive division

mov

mov

mov

eax, dword [intNum]

rcx, 0

ebx, 10

; get integer

; digitCount = 0

; set for dividing by 10

divideLoop:

mov

div

edx, 0

ebx

; divide number by 10

push rdx

; push remainder

; increment digitCount

inc

rcx

cmp

jne

eax, 0

divideLoop

; if (result > 0)

;

goto divideLoop

;

;

-----

Part B - Convert remainders and store

mov

mov

rbx, strNum

rdi, 0

; get addr of string

; idx = 0

popLoop:

pop

rax

; pop intDigit

add

al, "0"

; char = int + "0"

mov

inc

byte [rbx+rdi], al

rdi

; string[idx] = char

; increment idx

; if (digitCount > 0)

loop popLoop

;

goto popLoop

mov

byte [rbx+rdi], NULL

; string[idx] = NULL

Page 159

Chapter 10.0 ◄ Program Development

;

-----

;  Done, terminate program.

last:

mov

rax, SYS_exit

rdi, EXIT_SUCCESS

; call code for exit

; exit with success

mov

syscall

There are many different valid implementations for this algorithm. The program should

be assembled to address any typos or syntax errors.

1

0.4 Test/Debug the Program

Once the program is written, testing should be performed to ensure that the program

works. The testing will be based on the specific parameters of the program.

In this case, the program can be executed using the debugger and stopped near the end

of the program (e.g., at the label “last” in this example). After starting the debugger

with ddd, the command b last and run can be entered which will run the program

up to, but not executing the line referenced by the label “last”. The resulting string,

strNum can be viewed in the debugger with x/s &strNum will display the string

address and the contents which should be “1498”. For example;

(gdb) x/s &strNum

0x600104: "1498"

If the string is not displayed properly, it might be worth checking each character of the

five (5) byte array with the x/5cb &strNum debugger command. The output will

show the address of the string followed by both the decimal and ASCII representation.

For example;

(gdb) x/5cb &strNum

0x600104: 49 '1' 52 '4' 57 '9' 56 '8' 0 '\000'

The format of this output can be confusing initially.

If the correct output is not provided, the programmer will need to debug the code. For

this example, there are two main steps; successive division and conversion/storing the

remainders. The second step requires the first step to work, so the first step should be

verified. This can be done by using the debugger to focus only on the first section. In

this example, the first step should iterate exactly 4 times, so rcx should be 4.

Page 160

Chapter 10.0 ◄ Program Development

Additionally, 8, 9, 4, and 1 should be pushed on the stack in that order. This is easily

verified in the debugger by looking at the register contents of rdx when it is pushed or

by viewing the top 4 entries in the stack.

If that section works, the second section can be verified. Here, the values 1, 4, 9, and 8

should be coming off the stack (in that order). If so, the integer is converted into a

character by adding “0” (0x30) and that stored in the string, one character at a time. The

string can be viewed character by character to see if they are being entered into the

string correctly.

In this manner, the problem can be narrowed down fairly quickly. Efficient debugging

is a critical skill and must be honed by practice.

Refer to Chapter 6, DDD Debugger for additional information on specific debugger

commands.

1

0.5 Error Terminology

In case the program does not work, it helps to understand some basic terminology about

where or what the error might be. Using the correct terminology ensures that you can

communicate effectively about the problem with others.

1

0.5.1 Assembler Error

Assembler errors are generated when the program is assembled. This means that the

assembler does not understand one or more of the instructions. The assembler will

provide a list of errors and the line number of each error. It is recommended to address

the errors from the top down. Resolving an error at the top can clear multiple errors

further down.

Typical assembler errors include misspelling an instruction and/or omitting a variable

declaration.

1

0.5.2 Run-time Error

A run-time error is something that causes the program to crash.

1

0.5.3 Logic Error

A logic error is when the program executes, but does not produce the correct result. For

example, coding a provided formula incorrectly or attempting to compute the average of

a series of numbers before calculating the sum.

If the program has a logic error, one way to find the error is to display intermediate

Page 161

Chapter 10.0 ◄ Program Development

values. Further information will be provided in later chapters regarding advice on

finding logic errors.

1

0.6 Exercises

Below are some quiz questions and suggested projects based on this chapter.

1

0.6.1 Quiz Questions

Below are some quiz questions based on this chapter.

1

2

3

) What is an algorithm?

) What are the four main steps in algorithm development?

) Are the four main steps in algorithm development applicable only to assembly

language programming?

4

5

6

7

) What type of error, if any, occurs if the one operand multiply instruction uses an

immediate value operand? Respond with assemble-time or run-time.

) If an assembly language instruction is spelled incorrectly (e.g., “mv” instead of

“mov”), when will the error be found? Respond with assemble-time or run-time.

) If a label is referenced, but not defined, when will the error be found? Respond

with assemble-time or run-time.

) If a program performing a series of divides on values in an array divides by 0,

when will the error be found? Respond with assemble-time or run-time.

1

0.6.2 Suggested Projects

Below are some suggested projects based on this chapter.

1

) Implement the example program to convert an integer into a string. Change the

original integer to a different value. Use the debugger to execute the program

and display the final results. Create a debugger input file to show the results.

2

) Update the example program to address signed integers. This will require

including a preceding sign, “+” or “-” in the string. For example, -12310

(0xFFFFFF85) would be “-123” with a NULL termination (total of 5 bytes).

Additionally, the signed divide (IDIV, not DIV) and signed conversions (e.g.,

CDQ) must be used. Use the debugger to execute the program and display the

final results. Create a debugger input file to show the results.

3

) Create a program to convert a string representing a numeric value into an integer.

Page 162

Chapter 10.0 ◄ Program Development

For example, given the NULL terminated string “41275” (a total of 6 bytes),

convert the string into a double-word sized integer (0x0000A13B). You may

assume the string and resulting integer is unsigned. Use the debugger to execute

the program and display the final results. Create a debugger input file to show

the results.

4

5

) Update the previous program to address strings with a preceding sign (“+” or

“-”). This will require including a sign, “+” or “-” in the string. You must

ensure the final string is NULL terminated. You may assume the input strings

are valid. Use the debugger to execute the program and display the final results.

Create a debugger input file to show the results.

) Update the previous program to convert strings into integers to include error

checking on the input string. Specifically, the sign must be valid and be the first

character in the string, each digit must be between “0” and “9”, and the string

NULL terminated. For example, the string “-321” is valid while “1+32” and

“+1R3” are both invalid. Use the debugger to execute the program and display

the final results. Create a debugger input file to show the results.

Page 163

Chapter 10.0 ◄ Program Development

Page 164

Chapter

11

Why did C++ decide not to go out with C?

Because C has no class.

11.0 Macros

An assembly language macro is a predefined set of instructions that can easily be

inserted wherever needed. Once defined, the macro can be used as many times as

necessary. It is useful when the same set of code must be utilized numerous times. A

macro can be useful to reduce the amount of coding, streamline programs, and reduce

errors from repetitive coding.

The assembler contains a powerful macro processor, which supports conditional

assembly, multi-level file inclusion, and two forms of macros (single-line and multi-

line), and a 'context stack' mechanism for extra macro power.

Before using a macro, it must be defined. Macro definitions should be placed in the

source file before the data and code sections. The macro is used in the text (code)

section. The following sections will present a detailed example with the definition and

use.

1

1.1 Single-Line Macros

There are two key types of macros; single-line macros and multi-line macros. Each of

these is described in the following sections.

Single-line macros are defined using the %define directive. The definitions work in a

similar way to C/C++; so you can do things like:

%define   mulby4(x)   shl x, 2

And, then use the macro by entering:

mulby4 (rax)

in the source, which will multiply the contents to the rax register by 4 (via shifting two

bits).

Page 165

Chapter 11.0 ◄ Macros

1

1.2 Multi-Line Macros

Multi-line macros can include a varying number of lines (including one). The multi-line

macros are more useful and the following sections will focus primarily on multi-line

macros.

1

1.2.1 Macro Definition

Before using a multi-line macro, it must first be defined. The general format is as

follows:

%

macro  <name>  <number of arguments>

[body of macro]

endmacro

;

%

The arguments can be referenced within the macro by %<number>, with %1 being the

first argument, and %2 the second argument, and so forth.

In order to use labels, the labels within the macro must be prefixing the label name with

a %%.

This will ensure that calling the same macro multiple times will use a different label

each time. For example, a macro definition for the absolute value function would be as

follows:

%macro  abs  1

cmp  %1, 0

jge  %%done

neg  %1

%%done:

%endmacro

Refer to the sample macro program for a complete example.

1

1.2.2 Using a Macro

In order to use or “invoke” a macro, it must be placed in the code segment and referred

to by name with the appropriate number of arguments.

Page 166

Chapter 11.0 ◄ Macros

Given a data declaration as follows:

qVar    dq    4

Then, to invoke the “abs” macro (twice):

mov eax, -3

abs eax

abs qword [qVar]

The list file will display the code as follows (for the first invocation):

27 00000000 B8FDFFFFFF         mov  eax, -3

28                             abs  eax

29 00000005 3D00000000  <1>  cmp %1, 0

30 0000000A 7D02        <1>  jge %%done

31 0000000C F7D8        <1>  neg %1

32                      <1> %%done:

The macro will be copied from the definition into the code, with the appropriate

arguments replaced in the body of the macro, each time it is used. The <1> indicates

code copied from a macro definition. In both cases, the %1 argument was replaced with

the given argument; eax in this example.

Macros use more memory, but do not require overhead for transfer of control (like

functions).

1

1.3 Macro Example

The following example program demonstrates the definition and use of a simple macro.

; Example Program to demonstrate a simple macro

;

;

;

;

**************************************************

Define the macro

called with three arguments:

aver  <lst>, <len>, <ave>

%

macro

mov

aver   3

eax, 0

mov

ecx, dword [%2]

; length

Page 167

Chapter 11.0 ◄ Macros

mov

lea

r12, 0

rbx, [%1]

%%sumLoop:

add

eax, dword [rbx+r12*4] ; get list[n]

r12

inc

loop %%sumLoop

cdq

idiv dword [%2]

mov   dword [%3], eax

%endmacro

;

;

**************************************************

Data declarations

section .data

; -----

;  Define constants

EXIT_SUCCESS equ

SYS_exit equ

0

60

; success code

; code for terminate

;

Define Data.

section .data

list1

len1

ave1

dd   4, 5, 2, -3, 1

dd   5

dd   0

list2

len2

ave2

dd   2, 6, 3, -2, 1, 8, 19

dd   7

dd   0

; **************************************************

section .text

global _start

_start:

Page 168

Chapter 11.0 ◄ Macros

; -----

;  Use the macro in the program

aver list1, len1, ave1

aver list2, len2, ave2

; 1st, data set 1

; 2nd, data set 2

;

-----

;  Done, terminate program.

last:

mov rax, SYS_exit

; exit

; success

mov rdi, EXIT_SUCCESS

syscall

In this example, the macro is invoked twice. Each time the macro is used, it is copied

from the definition into the text section. As such, macros typically use more memory.

1

1.4 Debugging Macros

The code for a macro will not be displayed in the debugger source window. When a

macro is working correctly, this is very convenient. However, when debugging macros,

the code must be viewable.

In order to see the macro code, display the machine code window (View → Machine

Code Window). In the window, the machine code for the instructions are displayed.

The step and next instructions will execute the entire macro. In order to execute the

macro instructions, the stepi and nexti commands must be used.

The code, when viewed, will be the expanded code (as opposed to the original macro’s

definition).

1

1.5 Exercises

Below are some quiz questions and suggested projects based on this chapter.

1

1.5.1 Quiz Questions

Below are some quiz questions based on this chapter.

1

2

) Where is the macro definition placed in the assembly language source file?

) When a macro is invoked, how many times is the code placed in the code

segment?

Page 169

Chapter 11.0 ◄ Macros

3

) Explain why, in a macro, labels are typically preceded by a %% (double percent

sign).

4

5

) Explain what might happen if the %% is not included on a label?

) Is it legal to jump to a label that does not include the %%? If not legal, explain

why. If legal, explain under what circumstances that might be useful.

6

) When does the macro argument substitution occur?

1

1.5.2 Suggested Projects

Below are some suggested projects based on this chapter.

1

2

3

) Implement the example program for a list average macro. Use the debugger to

execute the program and display the final results. Create a debugger input file to

show the results.

) Update the program from the previous question to include the minimum and

maximum values. Use the debugger to execute the program and display the final

results. Create a debugger input file to show the results.

) Create a macro to update an existing list by multiplying every element by 2.

Invoke the macro at least three times of three different data sets. Use the

debugger to execute the program and display the final results. Create a debugger

input file to show the results.

4

) Create a macro from the integer to ASCII conversion example from the previous

chapter. Invoke the macro at least three times of three different data sets. Use

the debugger to execute the program and display the final results. Create a

debugger input file to show the results.

Page 170

Chapter

12

Why do programmers mix up Halloween

and Christmas?

Because 31 Oct = 25 Dec.

12.0 Functions

Functions and procedures (i.e., void functions) help break-up a program into smaller

parts making it easier to code, debug, and maintain. Function calls involve two main

actions:

•

Linkage

Since the function can be called from multiple different places in the code,

◦

the function must be able to return to the correct place in which it was

originally called.

•

Argument Transmission

◦

The function must be able to access parameters to operate on or to return

results (i.e., access call-by-reference parameters).

The specifics of how each of these actions are accomplished is explained in the

following sections.

1

2.1 Updated Linking Instructions

When writing and debugging functions, it is easier for the C compiler (either gcc or

g++) to link the program as the C compiler is aware of the appropriate locations for the

various C/C++ libraries.

For example, assuming that the source file is named example.asm, the commands to

compile, assemble, link, and execute as follows:

yasm -g dwarf2 -f elf64 example.asm -l example.lst

gcc -g -o example example.o

Note, Ubuntu 18 and above will require the no-pie option on the gcc as shown:

gcc -g -no-pie -o example example.o

Page 171

Chapter 12.0 ◄ Functions

This will use the GCC compiler to call the linker, reading the example.o object file and

creating the example executable file. The “-g” option includes the debugging

information in the executable file in the usual manner. The file names can be changed

as desired.

1

2.2 Debugger Commands

When using the debugger to debug programs with functions, a review of the step and

next debugger commands may be helpful.

1

2.2.1 Debugger Command, next

With respect to a function call, the debugger next command will execute the entire

function and go to the next line. When debugging functions, this is useful to quickly

execute the entire function and then just verify the results. It will not display any of the

function code.

1

2.2.2 Debugger Command, step

With respect to a function call, the debugger step command will step into the function

and go to the first line of the function code. It will display the function code. When

debugging functions, this is useful to debug the function code.

1

2.3 Stack Dynamic Local Variables

In a high-level language, non-static local variables declared in a function are stack

dynamic local variables by default. Some C++ texts refer to such variables as

automatics. This means that the local variables are created by allocating space on the

stack and assigning these stack locations to the variables. When the function completes,

the space is recovered and reused for other purposes. This requires a small amount of

additional run-time overhead, but makes a more efficient overall use of memory. If a

function with a large number of local variables is never called, the memory for the local

variables is never allocated. This helps reduce the overall memory footprint of the

program which generally helps the overall performance of the program.

In contrast, statically declared variables are assigned memory locations for the entire

execution of the program. This uses memory even if the associated function is not being

executed. However, no additional run-time overhead is required to allocate the space

since the space allocation has already been performed (when the program was initially

loaded into memory).

Page 172

Chapter 12.0 ◄ Functions

1

2.4 Function Declaration

A function must be written before it can be used. Functions are located in the code

segment. The general format is:

global <procName>

<procName>:

; function body

ret

A function may be defined only once. There is no specific order required for how

functions are defined. However, functions cannot be nested. A function definition

should be started and ended before the next function’s definition can be started.

Refer to the sample functions for examples of function declarations and usage.

1

2.5 Standard Calling Convention

To write assembly programs, a standard process for passing parameters, returning

values, and allocating registers between functions is needed. If each function did these

operations differently, things would quickly get very confusing and require

programmers to attempt to remember for each function how to handle parameters and

which registers were used. To address this, a standard process is defined and used

which is typically referred to as a standard calling convention40. There are actually a

number of different standard calling conventions. The 64-bit C calling convention,

called System V AMD64 ABI41 42, is described in the remainder of this document.

This calling convention is also used for C/C++ programs by default. This means that

interfacing assembly language code and C/C++ code is easily accomplished since the

same calling convention is used.

It must be noted that the standard calling convention presented here applies to Linux-

based operating systems. The standard calling convention for Microsoft Windows is

slightly different and not presented in this text.

4

4

0 For more information, refer to: http://en.wikipedia.org/wiki/Calling_convention

1 For more information, refer to:

https://en.wikipedia.org/wiki/X86_calling_conventions#System_V_AMD64_ABI

2 For complete details, refer to: https://software.intel.com/sites/default/files/article/402129/mpx-

linux64-abi.pdf

4

Page 173

Chapter 12.0 ◄ Functions

1

2.6 Linkage

The linkage is about getting to and returning from a function call correctly. There are

two instructions that handle the linkage, call <funcName> and ret instructions.

The call transfers control to the named function, and ret returns control back to the

calling routine.

•

•

The call works by saving the address of where to return to when the function

completes (referred to as the return address). This is accomplished by placing

contents of the rip register on the stack. Recall that the rip register points to the

next instruction to be executed (which is the instruction immediately after the

call).

The ret instruction is used in a procedure to return. The ret instruction pops

the current top of the stack (rsp) into the rip register. Thus, the appropriate

return address is restored.

Since the stack is used to support the linkage, it is important that within the function the

stack must not be corrupted. Specifically, any items pushed must be popped. Pushing a

value and not popping would result in that value being popped off the stack and placed

in the rip register. This would cause the processor to attempt to execute code at that

location. Most likely the invalid location will cause the process to crash.

The function calling or linkage instruction is summarized as follows:

Instruction

Explanation

call   <funcName>

Calls a function. Push the 64-bit rip register

and jump to the <funcName>.

Examples:

Examples:

call  printString

ret

Return from a function. Pop the stack into the

rip register, effecting a jump to the line after

the call.

ret

A more complete list of the instructions is located in Appendix B.

Page 174

Chapter 12.0 ◄ Functions

1

2.7 Argument Transmission

Argument transmission refers to sending information (variables, etc.) to a function and

obtaining a result as appropriate for the specific function.

The standard terminology for transmitting values to a function is referred to as call-by-

value. The standard terminology for transmitting addresses to a function is referred to

as call-by-reference. This should be a familiar topic from a high-level language.

There are various ways to pass arguments to and/or from a function.

•

Placing values in register

◦

◦

◦

Easiest, but has limitations (i.e., the number of registers).

Used for first six integer arguments.

Used for system calls.

•

•

Globally defined variables

◦

Generally poor practice, potentially confusing, and will not work in many

cases.

◦

Occasionally useful in limited circumstances.

Putting values and/or addresses on stack

◦

◦

No specific limit to count of arguments that can be passed.

Incurs higher run-time overhead.

In general, the calling routine is referred to as the caller and the routine being called is

referred to as the callee.

1

2.8 Calling Convention

The function prologue is the code at the beginning of a function and the function

epilogue is the code at the end of a function. The operations performed by the prologue

and epilogue are generally specified by the standard calling convention and deal with

stack, registers, passed arguments (if any), and stack dynamic local variables (if any).

The general idea is that the program state (i.e., contents of specific registers and the

stack) are saved, the function executed, and then the state is restored. Of course, the

function will often require extensive use of the registers and the stack. The prologue

code helps save the state and the epilogue code restores the state.

Page 175

Chapter 12.0 ◄ Functions

1

2.8.1 Parameter Passing

As noted, a combination of registers and the stack is used to pass parameters to and/or

from a function.

The first six integer arguments are passed in registers as follows:

Argument

Number

Argument Size

6

4-bits

rdi

rsi

rdx

rcx

r8

32-bits

16-bits

di

8-bits

dil

sil

dl

1

2

3

4

5

6

edi

esi

edx

ecx

r8d

r9d

si

dx

cx

cl

r8w

r9w

r8b

r9b

r9

The seventh and any additional arguments are passed on the stack. The standard calling

convention requires that, when passing arguments (values or addresses) on the stack, the

arguments should be pushed in reverse order. That is “someFunc   (one,   two,

three, four, five, six, seven, eight, nine)” would imply a push order

of: nine, eight, and then seven.

For floating-point arguments, the floating-point registers xmm0 to xmm7 are used in

that order for the first eight float arguments.

Additionally, when the function is completed, the calling routine is responsible for

clearing the arguments from the stack. Instead of doing a series of pop instructions, the

stack pointer, rsp, is adjusted as necessary to clear the arguments off the stack. Since

each argument is 8 bytes, the adjustment would be adding [(number of arguments) * 8]

to the rsp.

For value returning functions, the result is placed in the A register based on the size of

the value being returned.

Specifically, the values are returned as follows:

Page 176

Chapter 12.0 ◄ Functions

Return Value

Size

Location

byte

al

ax

word

double-word

quadword

floating-point

eax

rax

xmm0

The rax register may be used in the function as needed as long as the return value is set

appropriately before returning.

1

2.8.2 Register Usage

The standard calling convention specifies the usage of registers when making function

calls. Specifically, some registers are expected to be preserved across a function call.

That means that if a value is placed in a preserved register or saved register, and the

function must use that register, the original value must be preserved by placing it on the

stack, altered as needed, and then restored to its original value before returning to the

calling routine. This register preservation is typically performed in the prologue and the

restoration is typically performed in the epilogue.

The following table summarizes the register usage.

Register

rax

rbx

rcx

rdx

rsi

rdi

rbp

rsp

r8

Usage

Return Value

Callee Saved

4th Argument

3rd Argument

2nd Argument

1st Argument

Callee Saved

Stack Pointer

5th Argument

6th Argument

r9

Page 177

Chapter 12.0 ◄ Functions

r10

r11

r12

r13

r14

r15

Temporary

Temporary

Callee Saved

Callee Saved

Callee Saved

Callee Saved

The temporary registers (rax, r10 and r11) and the argument registers (rdi, rsi, rdx,

rcx, r8, and r9) are not preserved across a function call. This means that any of these

registers may be used in the function without the need to preserve the original value.

Additionally, none of the floating-point registers are preserved across a function call.

Refer to Chapter 18 for more information regarding floating-point operations.

1

2.8.3 Call Frame

The items on the stack as part of a function call are referred to as a call frame (also

referred to as an activation record or stack frame). Based on the standard calling

convention, the items on the stack, if any, will be in a specific general format.

The possible items in the call frame include:

•

•

•

•

Return address (required).

Preserved registers (if any).

Passed arguments (if any).

Stack dynamic local variables (if any).

Other items may be placed in the call frame such as static links for dynamically scoped

languages. Such topics are outside the scope of this text and will not be discussed here.

For some functions, a full call frame may not be required. For example, if the function:

•

•

•

•

Is a leaf function (i.e., does not call another function).

Passes its arguments only in registers (i.e., does not use the stack).

Does not alter any of the saved registers.

Does not require stack-based local variables.

This can occur for simpler, smaller leaf functions.

Page 178

Chapter 12.0 ◄ Functions

However, if any of these conditions is not true, a full call frame is required.

For more non-leaf or more complex functions, a more complete call frame is required.

The standard calling convention does not explicitly require use of the frame pointer

register, rbp. Compilers are allowed to optimize the call frame and not use the frame

pointer. To simplify and clarify accessing stack-based arguments (if any) and stack

dynamic local variables, this text will utilize the frame pointer register. This is similar

to how many other architectures use a frame pointer register.

As such, if there are any stack-based arguments or any local variables needed within a

function, the frame pointer register, rbp, should be pushed and then set pointing to

itself. As additional pushes and pops are performed (thus changing rsp), the rbp

register will remain unchanged. This allows the rbp register to be used as a reference to

access arguments passed on the stack (if any) or stack dynamic local variables (if any).

For example, assuming a function call has eight (8) arguments and assuming the

function uses rbx, r12, and r13 registers (and thus must be pushed), the call frame

would be as follows:

...

<

8

7

th Argument>

← rbp + 24

← rbp + 16

(return address)

← rbp

<

th Argument>

rip

rbp

rbx

r12

r13

← rsp

...

Illustration 24: Stack Frame Layout

The stack-based arguments are accessed relative to the rbp. Each item push is a

quadword which uses 8 bytes. For example, [rbp+16] is the location of the first passed

argument (7th integer argument) and [rbp+24] is the location of the second passed

argument (8th integer argument).

Page 179

Chapter 12.0 ◄ Functions

In addition, the call frame would contain the assigned locations of local variables (if

any). The section on local variables details the specifics regarding allocating and using

local variables.

12.8.3.1 Red Zone

In the Linux standard calling convention, the first 128-bytes after the stack pointer, rsp,

are reserved. For example, extending the previous example, the call frame would be as

follows:

...

<

8th Argument> ← rbp + 24

7th Argument> ← rbp + 16

<

rip

rbp

rbx

r10

r12

(return address)

← rbp

← rsp

.

. .

128 bytes

.

. .

. .

Red Zone

.

Illustration 25: Stack Frame Layout with Red Zone

This red zone may be used by the function without any adjustment to the stack pointer.

The purpose is to allow compiler optimizations for the allocation of local variables.

This does not directly impact programs written directly in assembly language.

1

2.9 Example, Statistical Function 1 (leaf)

This simple example will demonstrate calling a simple void function to find the sum and

Page 180

Chapter 12.0 ◄ Functions

average of an array of numbers. The High-Level Language (HLL) call for C/C++ is as

follows:

stats1(arr, len, sum, ave);

As per the C/C++ convention, the array, arr, is call-by-reference and the length, len, is

call-by-value. The arguments for sum and ave are both call-by-reference (since there

are no values as yet). For this example, the array arr, sum, and ave variables are all

signed double-word integers. Of course, in context, the len must be unsigned.

1

2.9.1 Caller

In this case, there are 4 arguments, and all arguments are passed in registers in

accordance with the standard calling convention. The assembly language code in the

calling routine for the call to the stats function would be as follows:

;

stats1(arr, len, sum, ave);

mov

mov

mov

mov

rcx, ave

rdx, sum

esi, dword [len]

rdi, arr

; 4th arg, addr of ave

; 3rd arg, addr of sum

; 2nd arg, value of len

; 1st arg, addr of arr

call stats1

There is no specific required order for setting the argument registers. This example sets

them in reverse order in preparation for the next, extended example.

Note, the setting of the esi register also sets the upper-order double-word to zero, thus

ensuring the rsi register is set appropriately for this specific usage since length is

unsigned.

No return value is provided by this void routine. If the function was a value returning

function, the value returned would be in the A register (of appropriate size).

1

2.9.2 Callee

The function being called, the callee, must perform the prologue and epilogue operations

(as specified by the standard calling convention) before and after the code to perform the

function goal. For this example, the function must perform the summation of values in

the array, compute the integer average, return the sum and average values.

The following code implements the stats1 example.

;

;

Simple example function to find and return

the sum and average of an array.

Page 181

Chapter 12.0 ◄ Functions

;

;

;

;

;

;

;

;

HLL call:

stats1(arr, len, sum, ave);

-----

Arguments:

arr, address – rdi

len, dword value – esi

sum, address – rdx

ave, address - rcx

global stats1

stats1:

push r12

; prologue

mov

mov

r12, 0

rax, 0

; counter/index

; running sum

sumLoop:

add

eax, dword [rdi+r12*4] ; sum += arr[i]

r12

inc

cmp

jl

r12, rsi

sumLoop

mov dword [rdx], eax

; return sum

cdq

idiv esi

; compute average

; return ave

mov

dword [rcx], eax

pop

ret

r12

; epilogue

The choice of the r12 register is arbitrary, however a 'saved register' was selected.

The call frame for this function would be as follows:

...

rip

(return address)

← rsp

r12

...

The minimal use of the stack helps reduce the function call run-time overhead.

Page 182

Chapter 12.0 ◄ Functions

1

2.10 Example, Statistical Function2 (non-leaf)

This extended example will demonstrate calling a simple void function to find the

minimum, median, maximum, sum and average of an array of numbers.

The High-Level Language (HLL) call for C/C++ is as follows:

stats2(arr, len, min, med1, med2, max, sum, ave);

For this example, it is assumed that the array is sorted in ascending order. Additionally,

for this example, the median will be the middle value. For an even length list, there are

two middle values, med1 and med2, both of which are returned. For an odd length list,

the single middle value is returned in both med1 and med2.

As per the C/C++ convention, the array, arr, is call-by-reference and the length, len, is

call-by-value. The arguments for min, med1, med2, max, sum, and ave are all call-by-

reference (since there are no values as yet). For this example, the array arr, min, med1,

med2, max, sum, and ave variables are all signed double-word integers. Of course, in

context, the len must be unsigned.

1

2.10.1 Caller

In this case, there are 8 arguments and only the first six can be passed in registers. The

last two arguments are passed on the stack. The assembly language code in the calling

routine for the call to the stats function would be as follows:

;

stats2(arr, len, min, med1, med2, max, sum, ave);

push ave

push sum

; 8th arg, add of ave

; 7th arg, add of sum

; 6th arg, add of max

; 5th arg, add of med2

; 4th arg, add of med1

; 3rd arg, addr of min

; 2nd arg, value of len

; 1st arg, addr of arr

mov

mov

mov

mov

mov

mov

r9, max

r8, med2

rcx, med1

rdx, min

esi, dword [len]

rdi, arr

call stats2

add rsp, 16

; clear passed arguments

The 7th and 8th arguments are passed on the stack and pushed in reverse order in

accordance with the standard calling convention. After the function is completed, the

arguments are cleared from the stack by adjusting the stack point register (rsp). Since

two arguments, 8 bytes each, were passed on the stack, 16 is added to the stack pointer.

Page 183

Chapter 12.0 ◄ Functions

Note, the setting of the esi register also sets the upper-order double-word to zero, thus

ensuring the rsi register is set appropriately for this specific usage since length is

unsigned.

No return value is provided by this void routine. If the function was a value returning

function, the value returned would be in the A register.

1

2.10.2 Callee

The function being called, the callee, must perform the prologue and epilogue operations

(as specified by the standard calling convention). Of course, the function must perform

the summation of values in the array, find the minimum, medians, and maximum,

compute the average, return all the values.

When call-by-reference arguments are passed on the stack, two steps are required to

return the value.

•

•

Get the address from the stack.

Use that address to return the value.

A common error is to attempt to return a value to a stack-based location in a single step,

which will not change the referenced variable. For example, assuming the double-word

value to be returned is in the eax register and the 7th argument is call-by-reference and

where the eax value is to be returned, the appropriate code would be as follows:

mov

mov

r12, qword [rbp+16]

dword [r12], eax

These steps cannot be combined into a single step. The following code

mov

dword [rbp+16], eax

Would overwrite the address passed on the stack and not change the reference variable.

The following code implements the stats2 example.

;

;

;

;

;

Simple example function to find and return the minimum,

maximum, sum, medians, and average of an array.

-----

HLL call:

stats2(arr, len, min, med1, med2, max, sum, ave);

;  Arguments:

;

arr, address – rdi

Page 184

Chapter 12.0 ◄ Functions

;

;

;

;

;

;

;

len, dword value – esi

min, address – rdx

med1, address - rcx

med2, address - r8

max, address - r9

sum, address – stack (rbp+16)

ave, address – stack (rbp+24)

global stats2

stats2:

push rbp

; prologue

mov

rbp, rsp

push r12

;

;

-----

Get min and max.

mov

mov

eax, dword [rdi]

dword [rdx], eax

; get min

; return min

mov

dec

mov

mov

r12, rsi

r12

eax, dword [rdi+r12*4]

dword [r9], eax

; get len

; set len-1

; get max

; return max

;

-----

;  Get medians

mov

mov

mov

div

rax, rsi

rdx, 0

r12, 2

r12

; rax = length/2

cmp

je

rdx, 0

evenLength

; even/odd length?

mov

mov

mov

jmp

r12d, dword [rdi+rax*4]

dword [rcx], r12d

dword [r8], r12d

medDone

; get arr[len/2]

; return med1

; return med2

evenLength:

mov

r12d, dword [rdi+rax*4]

; get arr[len/2]

Page 185

Chapter 12.0 ◄ Functions

mov

dec

dword [r8], r12d

rax

; return med2

mov

mov

r12d, dword [rdi+rax*4]

dword [rcx], r12d

; get arr[len/2-1]

; return med1

medDone:

;

-----

;  Find sum

mov

mov

r12, 0

rax, 0

; counter/index

; running sum

sumLoop:

add

eax, dword [rdi+r12*4]

; sum += arr[i]

inc

cmp

jl

r12

r12, rsi

sumLoop

mov

mov

r12, qword [rbp+16]

dword [r12], eax

; get sum addr

; return sum

; -----

;  Calculate average.

cdq

idiv rsi

mov

mov

; average = sum/len

; get ave addr

; return ave

r12, qword [rbp+24]

dword [r12], eax

pop

pop

ret

r12

rbp

; epilogue

The choice of the registers is arbitrary with the bounds of the calling convention.

Page 186

Chapter 12.0 ◄ Functions

The call frame for this function would be as follows:

.

. .

<

8th Argument> ← rbp + 24

7th Argument> ← rbp + 16

<

rip

rbp

r12

(return address)

← rbp

← rsp

.

. .

In this example, the preserved registers, rbp and then r12, are pushed. When popped,

they must be popped in the exact reverse order r12 and then rbp in order to correctly

restore their original values.

1

2.11 Stack-Based Local Variables

If local variables are required, they are allocated on the stack. By adjusting the rsp

register, additional memory is allocated on the stack for locals. As such, when the

function is completed, the memory used for the stack-based local variables is released

(and no longer uses memory).

Further expanding the previous example, if we assume all array values are between 0

and 99, and we wish to find the mode (number that occurs the most often), a single

double-word variable count and a one hundred (100) element local double-word array,

tmpArr[100] might be used.

As before, the frame register, rbp, is pushed on the stack and set pointing to itself. The

frame register plus an appropriate offset will allow accessing any arguments passed on

the stack. For example, rbp+16 is the location of the first stack-based argument (7th

integer argument).

After the frame register is pushed, an adjustment to the stack pointer register, rsp, is

made to allocate space for the local variables, a 100-element array in this example.

Since the count variable is a one double-word, 4-bytes is needed. The temporary array

is 100 double-word elements, 400 bytes is required. Thus, a total of 404 bytes is

required. Since the stack is implemented growing downward in memory, the 404 bytes

is subtracted from the stack pointer register.

Page 187

Chapter 12.0 ◄ Functions

Then any saved registers, rbx and r12 in this example, are pushed on the stack.

When leaving the function, the saved registers and then the locals must be cleared from

the stack. The preferred method of doing this is to pop the saved registers and then top

copy the rbp register into the rsp register, thus ensuring the rsp register points to the

correct place on the stack.

mov

rsp, rbp

This is generally better than adding the offset back to the stack since allocated space

may be altered as needed without also requiring adjustments to the epilogue code.

It should be clear that variables allocated in this manner are uninitialized. Should the

function require the variables to be initialized, possibly to 0, such initializations must be

explicitly performed.

For this example, the call frame would be formatted as follows:

...

<

value of len>

← rbp + 24

← rbp + 16

(return address)

← rbp

<

addr of list>

rip

rbp

tmpArr[99]

tmpArr[98]

.

.

. .

. .

tmpArr[1]

←

←

rbp - 400 = tmpArr[0]

rbp - 404 = count

rbx

r12

← rsp

.

. .

The layout and order of the local variables within the allocated 404 bytes is arbitrary.

Page 188

Chapter 12.0 ◄ Functions

For example, the updated prologue code for this expanded example would be:

push rbp

; prologue

mov

sub

rbp, rsp

rsp, 404

; allocate locals

push rbx

push r12

The local variables can be accessed relative to the frame pointer register, rbp. For

example, to initialize the count variable, now allocated to rbp-404, the following

instruction could be used:

mov  dword [rbp-404], 0

To access the tmpArr, the starting address must be obtained which can be performed

with the lea instruction. For example,

lea  rbx, dword [rbp-400]

Which will set the appropriate stack address in the rbx register where rbx was chosen

arbitrarily. The dword qualifier in this example is not required, and may be misleading,

since addresses are always 64-bits (on a 64-bit architecture). Once set as above, the

tmpArr starting address in rbx is used in the usual manner.

For example, a small incomplete function code fragment demonstrating the accessing of

stack-based local variables is as follows:

;

;

-----------------------------------------

Example function

global  expFunc

expFunc:

push rbp

; prologue

mov

sub

rbp, rsp

rsp, 404

; allocate locals

push rbx

push r12

;

;

-----

Initialize count local variable to 0.

mov

dword [rbp-404], 0

; -----

Page 189

Chapter 12.0 ◄ Functions

;  Increment count variable (for example)...

inc

dword [rbp-404]

; count++

;

-----

;  Loop to initialize tmpArr to all 0's.

lea

mov

rbx, dword [rbp-400]

r12, 0

; tmpArr addr

; index

zeroLoop:

mov

dword [rbx+r12*4], 0

r12

; tmpArr[index]=0

inc

cmp

jl

r12, 100

zeroLoop

;

;

-----

Done, restore all and return to calling routine.

pop

pop

mov

pop

ret

r12

rbx

rsp, rbp

rbp

; epilogue

; clear locals

Note, this example function focuses only on how stack-based local variables are

accessed and does not perform anything useful.

1

2.12 Summary

This section presents a brief summary of the standard calling convention requirements

which are as follows:

Caller Operations:

•

•

The first six integer arguments are passed in registers

◦

rdi, rsi, rdx, rcx, r8, r9

The 7th

and on arguments are passed on the stack-based

◦

Pushes the arguments on the stack in reverse order (right to left, so that the

first stack argument specified in the function call is pushed last).

◦

Pushed arguments are passed as quadwords.

Page 190

Chapter 12.0 ◄ Functions

•

•

The caller executes a call instruction to pass control to the function (callee).

Stack-based arguments are cleared from the stack.

◦

add rsp, <argCount*8>

Callee Operations:

•

Function Prologue

◦

If arguments are passed on the stack, the callee must save rbp to the stack

and move the value of rsp into rbp. This allows the callee to use rbp as a

frame pointer to access arguments on the stack in a uniform manner.

▪

The callee may then access its parameters relative to rbp. The quadword

at [rbp] holds the previous value of rbp as it was pushed; the next

quadword, at [rbp+8], holds the return address, pushed by the call. The

parameters start after that, at [rbp+16].

◦

◦

If local variables are needed, the callee decreases rsp further to allocate

space on the stack for the local variables. The local variables are accessible

at negative offsets from rbp.

The callee, if it wishes to return a value to the caller, should leave the value

in al, ax, eax, rax, depending on the size of the value being returned.

▪

A floating-point result is returned in xmm0.

◦

If altered, registers rbx, r12, r13, r14, r15 and rbp must be saved on the

stack.

•

•

Function Execution

The function code is executed.

Function Epilogue

◦

◦

◦

Restores any pushed registers.

If local variables were used, the callee restores rsp from rbp to clear the

stack-based local variables.

◦

◦

The callee restores (i.e., pops) the previous value of rbp.

The call returns via ret instruction (return).

Refer to the sample functions to see specific examples of the calling convention.

Page 191

Chapter 12.0 ◄ Functions

1

2.13 Exercises

Below are some quiz questions and suggested projects based on this chapter.

1

2.13.1 Quiz Questions

Below are some quiz questions based on this chapter.

1

2

3

4

5

) What are the two main actions of a function call?

) What are the two instructions that implement linkage?

) When arguments are passed using values, it is referred to as?

) When arguments are passed using addresses, it is referred to as?

) If a function is called fifteen (15) times, how many times is the code placed in

memory by the assembler?

6

7

) What happens during the execution of a call instruction (two things)?

) According to the standard calling convention, as discussed in class, what is the

purpose of the initial pushes and final pops within most procedures?

8

9

) If there are six (6) 64-bit integer arguments passed to a function, where

specifically should each of the arguments be passed?

) If there are six (6) 32-bit integer arguments passed to a function, where

specifically should each of the arguments be passed?

1

1

1

0) What does it mean when a register is designated as temporary?

1) Name two temporary registers?

2) What is the name for the set of items placed on the stack as part of a function

call?

1

1

3) What does it mean when a function is referred to as a leaf function?

4) What is the purpose of the add     rsp,   <immediate> after the call

statement?

1

5) If three arguments are passed on the stack, what is the value for the

immediate>?

<

1

6) If there are seven (7) arguments passed to a function, and the function itself

pushes the rbp, rbx, and r12 registers (in that order), what is the correct offset

of the stack-based argument when using the standard calling convention?

Page 192

Chapter 12.0 ◄ Functions

1

1

7) What, if any, is the limiting factor for how many times a function can be called?

8) If a function must return a result for the variable sum, how should the sum

variable be passed (call-by-reference or call-by-value)?

1

9) If there are eight (8) arguments passed to a function, and the function itself

pushes the rbp, rbx, and r12 registers (in that order), what are the correct offsets

for each of the two stack-based arguments (7th and 8th) when using the standard

calling convention?

2

0) What is the advantage of using stack dynamic local variables (as opposed to

using all global variables)?

1

2.13.2 Suggested Projects

Below are some suggested projects based on this chapter.

1

2

3

) Create a main and implement the stats1 example function. Use the debugger to

execute the program and display the final results. Create a debugger input file to

show the results.

) Create a main and implement the stats2 example function. Use the debugger to

execute the program and display the final results. Create a debugger input file to

show the results.

) Create a main program and a function that will sort a list of numbers in

ascending order. Use the following selection43 sort algorithm:

begin

for  i = 0 to len-1

small = arr(i)

index = i

for  j = i to len-1

if ( arr(j) < small ) then

small = arr(j)

index = j

end_if

end_for

arr(index) = arr(i)

arr(i) = small

end_for

end_begin

4

3 For more information, refer to: http://en.wikipedia.org/wiki/Selection_sort

Page 193

Chapter 12.0 ◄ Functions

The main should call the function on at least three different data sets. Use the

debugger to execute the program and display the final results. Create a debugger

input file to show the results.

4

) Update the program from the previous question to add a stats function that finds

the minimum, median, maximum, sum, and average for the sorted list. The stats

function should be called after the sort function to make the minimum and

maximum easier to find. Use the debugger to execute the program and display

the final results. Create a debugger input file to show the results.

5

) Update the program from the previous question to add an integer square root

function and a standard deviation function. To estimate the square root of a

number, use the following algorithm:

iSqrt est = iNumber

iNumber

iterate 50 times

iSqrt

( )+

iSqrt

est

est

iSqrt est =

2

The formula for standard deviation is as follows:

length−1

( [ ]−

list i average)2

∑

i=0

√

iStandardDeviation =

length

Note, perform the summation and division using integer values. Use the

debugger to execute the program and display the final results. Create a debugger

input file to show the results.

6

) Convert the integer to ASCII macro from the previous chapter into a void

function. The function should convert a signed integer into a right-justified

string of a given length. This will require including any leading blanks, a sign

(“+” or “-”), the digits, and the NULL. The function should accept the value for

the integer and the address of where to place the NULL terminated string, and

the value of the maximum string length - in that order. Develop a main program

to call the function on a series of different integers. The main should include the

appropriate data declarations. Use the debugger to execute the program and

display the final results. Create a debugger input file to show the results.

Page 194

Chapter 12.0 ◄ Functions

7

) Create a function to convert an ASCII string representing a number into an

integer. The function should read the string and perform appropriate error

checking. If there is an error, the function should return FALSE (a defined

constant set to 0). If the string is valid, the function should convert the string

into an integer. If the conversion is successful, the function should return TRUE

(a defined constant set to 1). Develop a main program to call the function on a

series of different integers. The main should include the appropriate data

declarations and applicable the constants. Use the debugger to execute the

program and display the final results. Create a debugger input file to show the

results.

Page 195

Chapter 12.0 ◄ Functions

Page 196

Chapter

13

Linux is basically a simple operating

system, but you have to be a genius to

understand the simplicity.

13.0 System Services

There are many operations that an application program must use the operating system to

perform. Such operations include console output, keyboard input, file services (open,

read, write, close, etc.), obtaining the time or date, requesting memory allocation, and

many others.

Accessing system services is how the application requests that the operating system

perform some specific operation (on behalf of the process). More specifically, the

system call is the interface between an executing process and the operating system.

This section provides an explanation of how to use some basic system service calls.

More information on additional system service calls is located in Appendix C, System

Service Calls.

1

3.1 Calling System Services

A system service call is logically similar to calling a function, where the function code

is located within the operating system. The function may require privileges to operate

which is why control must be transferred to the operating system.

When calling system services, arguments are placed in the standard argument registers.

System services do not typically use stack-based arguments. This limits the arguments

of a system services to six (6), which does not present a significant limitation.

To call a system service, the first step is to determine which system service is desired.

There are many system services (see Appendix C). The general process is that the

system service call code is placed in the rax register. The call code is a number that has

been assigned for the specific system service being requested. These are assigned as

part of the operating system and cannot be changed by application programs. To

simplify the process, this text will define a very small subset of system service call

codes to a set of constants. For this text, and the associated examples, the subset of

Page 197

Chapter 13.0 ◄ System Services

system call code constants are defined and shown in the source file to help provide

complete clarity for new assembly language programmers. For more experienced

programmers, typically developing larger or more complex programs, a complete list of

constants is in a file and included into the source file.

If any are needed, the arguments for system services are placed in the rdi, rsi, rdx, r10,

r8, and r9 registers (in that order). The following table shows the argument locations

which are consistent with the standard calling convention.

Register

rax

rdi

rsi

rdx

r10

r8

Usage

Call code (see table)

1st argument (if needed)

2nd argument (if needed)

3rd argument (if needed)

4th argument (if needed)

5th argument (if needed)

6th argument (if needed)

r9

This is very similar to the standard calling convention for function calls, however the 4th

argument, if needed, uses the r10 register.

Each system call will use a different number of arguments (from none up to 6).

However, the system service call code is always required.

After the call code and any arguments are set, the syscall instruction is executed. The

syscall instruction will pause the current process and transfer control to the operating

system which will attempt to perform the service specified in the rax register. When the

system service returns, the process will be resumed.

1

3.2 Newline Character

As a refresher, in the context of output, a newline means move the cursor to the start of

the next line. In many languages, including C, it is often noted as “\n” as part of a

string. C++ uses endl in the context of a cout statement. For example, “Hello World 1”

and “Hello\nWorld 2” would be displayed as follows:

Hello World 1

Hello

World 2

Page 198

Chapter 13.0 ◄ System Services

Nothing is displayed for the newline, but the cursor is moved to the start of the next line

as shown.

In Unix/Linux systems, the linefeed, abbreviated LF with an ASCII value of 10 (or

0

x0A), is used as the newline character. In Windows systems, the newline is carriage

return, abbreviated as CR with an ASCII value 13 (or 0x0D) followed by the LF. The

LF is used in the code examples in the text.

The reader may have seen instances where a text file is downloaded from a web page

and displayed using older versions Windows Notepad (pre-Windows 10) where all the

formatting is lost and it looks like the text is one very long line. This is typically due to

a Unix/Linux formatted file, which uses only LF's, being displayed with a Windows

utility that expects CR/LF pairs and does not display correctly when only LF's are

found. Other Windows software, like Notepad++ (open source text editor) will

recognize and handle the different newline formats and display correctly.

1

3.3 Console Output

The system service to output characters to the console is the system write (SYS_write).

Like a high-level language characters are written to standard out (STDOUT) which is

the console. The STDOUT is the default file descriptor for the console. The file

descriptor is already opened and available for use in programs (assembly and high-level

languages).

The arguments for the write system service are as follows:

Register SYS_write

rax

rdi

rsi

rdx

Call code = SYS_write (1)

Output location, STDOUT (1)

Address of characters to output

Number of characters to output

Assuming the following declarations:

STDOUT

SYS_write equ

equ

1

1

; standard output

; call code for write

msg

msgLen

db

dq

"Hello World"

11

Page 199

Chapter 13.0 ◄ System Services

For example, to output “Hello World” (it’s traditional) to the console, the system write

(SYS_write) would be used. The code would be as follows:

mov

mov

rax, SYS_write

rdi, STDOUT

mov

mov

rsi, msg

rdx, qword [msgLen]

; msg address

; length value

syscall

Refer to the next section for a complete program to display the above message. It

should be noted that the operating system does not check if the string is valid.

1

3.3.1 Example, Console Output

This example is a complete program to output some strings to the console. In this

example, one string includes new line and the other does not.

;

Example program to demonstrate console output.

; This example will send some messages to the screen.

; **********************************************

section .data

; -----

;  Define standard constants.

LF

equ

equ

equ

equ

10

0

1

; line feed

; end of string

NULL

TRUE

FALSE

0

EXIT_SUCCESS equ

0

; success code

STDIN

STDOUT

STDERR

equ

equ

equ

0

1

2

; standard input

; standard output

; standard error

SYS_read

SYS_write

SYS_open

SYS_close

SYS_fork

equ

equ

equ

equ

equ

0

1

2

3

; read

; write

; file open

; file close

; fork

57

Page 200

Chapter 13.0 ◄ System Services

SYS_exit

SYS_creat

SYS_time

equ

equ

equ

60

85

201

; terminate

; file open/create

; get time

;

;

-----

Define some strings.

message1

message2

newLine

db

db

db

"Hello World.", LF, NULL

"Enter Answer: ", NULL

LF, NULL

;------------------------------------------------------

section .text

global _start

_start:

;

;

-----

Display first message.

mov

rdi, message1

call printString

;

;

-----

Display second message and then newline

mov

rdi, message2

call printString

mov

rdi, newLine

call printString

;

;

-----

Example program done.

exampleDone:

mov

mov

rax, SYS_exit

rdi, EXIT_SUCCESS

syscall

; ******************************************************

;  Generic function to display a string to the screen.

;  String must be NULL terminated.

Page 201

Chapter 13.0 ◄ System Services

;

;

;

Algorithm:

Count characters in string (excluding NULL)

Use syscall to output characters

;

;

;

;

Arguments:

1) address, string

Returns:

nothing

global printString

printString:

push rbx

;

-----

;  Count characters in string.

mov

mov

rbx, rdi

rdx, 0

strCountLoop:

cmp

je

inc

inc

jmp

byte [rbx], NULL

strCountDone

rdx

rbx

strCountLoop

strCountDone:

cmp

je

rdx, 0

prtDone

;

-----

;  Call OS to output string.

mov

mov

mov

rax, SYS_write

rsi, rdi

rdi, STDOUT

; system code for write()

; address of chars to write

; standard out

;

RDX=count to write, set above

syscall

; system call

;

;

-----

String printed, return to calling routine.

prtDone:

Page 202

Chapter 13.0 ◄ System Services

pop

ret

rbx

The output would be as follows:

Hello World.

Enter Answer:_

The newline (LF) was provided as part of the first string (message1) thus placing the

cursor on the start of the next line. The second message would leave the cursor on the

same line which would be appropriate for reading input from the user (which is not part

of this example). A final newline is printed since no actual input is obtained in this

example.

The additional, unused constants are included for reference.

1

3.4 Console Input

The system service to read characters from the console is the system read (SYS_read).

Like a high-level language, for the console, characters are read from standard input

(STDIN). The STDIN is the default file descriptor for reading characters from the

keyboard. The file descriptor is already opened and available for use in program

(assembly and high-level languages).

Reading characters interactively from the keyboard presents an additional complication.

When using the system service to read from the keyboard, much like the write system

service, the number of characters to read is required. Of course, we will need to declare

an appropriate amount of space to store the characters being read. If we request 10

characters to read and the user types more than 10, the additional characters will be lost,

which is not a significant problem. If the user types less than 10 characters, for example

5

characters, all five characters will be read plus the newline (LF) for a total of six

characters.

A problem arises if input is redirected from a file. If we request 10 characters, and there

are 5 characters on the first line and more on the second line, we will get the six

characters from the first line (5 characters plus the newline) and the first four characters

from the next line for the total of 10. This is undesirable.

To address this, for interactively reading input, we will read one character at a time until

a LF (the Enter key) is read. Each character will be read and then stored, one at a time,

in an appropriately sized array.

Page 203

Chapter 13.0 ◄ System Services

The arguments for the read system service are as follows:

Register SYS_read

rax

rdi

rsi

rdx

Call code = SYS_read (0)

Input location, STDIN (0)

Address of where to store characters read

Number of characters to read

Assuming the following declarations:

STDIN

SYS_read equ

equ

0

0

; standard input

; call code for read

inChar

db

0

For example, to read a single character from the keyboard, the system read (SYS_read)

would be used. The code would be as follows:

mov

mov

mov

mov

rax, SYS_read

rdi, STDIN

rsi, inChar

rdx, 1

; msg address

; read count

syscall

Refer to the next section for a complete program to read characters from the keyboard.

1

3.4.1 Example, Console Input

The example is a complete program to read a line of 50 characters from the keyboard.

Since space for the newline (LF) along with a final NULL termination is included, an

input array allowing 52 bytes would be required.

This example will read up to 50 characters from the user and then echo the input back to

the console to verify that the input was read correctly.

; Example program to demonstrate console output.

; This example will send some messages to the screen.

; **********************************************

section .data

Page 204

Chapter 13.0 ◄ System Services

; -----

;  Define standard constants.

LF

NULL

equ

equ

10

0

; line feed

; end of string

TRUE

FALSE

equ

equ

1

0

EXIT_SUCCESS equ

0

; success code

STDIN

STDOUT

STDERR

equ

equ

equ

0

1

2

; standard input

; standard output

; standard error

SYS_read

SYS_write

SYS_open

SYS_close

SYS_fork

SYS_exit

SYS_creat

SYS_time

equ

equ

equ

equ

equ

equ

equ

equ

0

1

2

3

57

60

85

201

; read

; write

; file open

; file close

; fork

; terminate

; file open/create

; get time

;

;

-----

Define some strings.

STRLEN

equ

50

pmpt

newLine

db

db

"Enter Text: ", NULL

LF, NULL

section  .bss

chr

inLine

resb

resb STRLEN+2

1

; total of 52

;------------------------------------------------------

section .text

global _start

_start:

Page 205

Chapter 13.0 ◄ System Services

;

-----

;  Display prompt.

mov

rdi, pmpt

call printString

;

;

-----

Read characters from user (one at a time)

mov

mov

rbx, inLine

r12, 0

; inLine addr

; char count

readCharacters:

mov

mov

lea

mov

rax, SYS_read

; system code for read

; standard in

; address of chr

; count (how many to read)

; do syscall

rdi, STDIN

rsi, byte [chr]

rdx, 1

syscall

mov

cmp

je

al, byte [chr]

al, LF

readDone

; get character just read

; if linefeed, input done

inc

cmp

jae

r12

; count++

; if # chars ≥ STRLEN

;   stop placing in buffer

r12, STRLEN

readCharacters

mov

inc

byte [rbx], al

rbx

; inLine[i] = chr

; update tmpStr addr

jmp

readDone:

mov

readCharacters

byte [rbx], NULL

; add NULL termination

;

;

-----

Output the line to verify successful read

mov

rdi, inLine

call printString

;

;

-----

Example done.

Page 206

Chapter 13.0 ◄ System Services

exampleDone:

mov

mov

rax, SYS_exit

rdi, EXIT_SUCCESS

syscall

;

;

;

;

;

;

******************************************************

Generic procedure to display a string to the screen.

String must be NULL terminated.

Algorithm:

Count characters in string (excluding NULL)

Use syscall to output characters

;

;

;

;

Arguments:

1) address, string

Returns:

nothing

global printString

printString:

push rbx

;

-----

;  Count characters in string.

mov

mov

rbx, rdi

rdx, 0

strCountLoop:

cmp

je

inc

inc

jmp

byte [rbx], NULL

strCountDone

rdx

rbx

strCountLoop

strCountDone:

cmp

je

rdx, 0

prtDone

; -----

;  Call OS to output string.

mov

mov

rax, SYS_write

rsi, rdi

; system code for write()

; address of char's to write

Page 207

Chapter 13.0 ◄ System Services

mov

rdi, STDOUT

; standard out

RDX=count to write, set above

; system call

;

syscall

;

;

-----

String printed, return to calling routine.

prtDone:

pop

rbx

ret

If we were to completely stop reading at 50 (STRLEN) characters and the user enters

more characters, the characters might cause input errors for successive read operations.

To address any extra characters the user might enter, the extra characters are read from

the keyboard but not placed in the input buffer (inLine above). This ensures that the

extra input is removed from the input stream and but does not overrun the array.

The additional, unused constants are included for reference.

1

3.5 File Open Operations

In order to perform file operations such as read and write, the file must first be opened.

There are two file open operations, open and open/create. Each of the two open

operations are explained in the following sections.

After the file is opened, in order to perform file read or write operations the operating

system needs detailed information about the file, including the complete status and

current read/write location. This is necessary to ensure that read or write operations

pick up where they left off (from last time).

If the file open operation fails, an error code will be returned. If the file open operation

succeeds, a file descriptor is returned. This applies to both high-level languages and

assembly code.

The file descriptor is used by the operating system to access the complete information

about the file. The complete set of information about an open file is stored in an

operating system data structure named File Control Block (FCB). In essence, the file

descriptor is used by the operating system to reference the correct FCB. It is the

programmer's responsibility to ensure that the file descriptor is stored and used

correctly.

Page 208

Chapter 13.0 ◄ System Services

1

3.5.1 File Open

The file open requires that the file exists in order to be opened. If the file does not exist,

it is an error.

The file open operation also requires the parameter flag to specify the access mode. The

access mode must include one of the following:

•

•

•

Read-Only Access → O_RDONLY

Write-Only Access → O_WRONLY

Read/Write Access → O_RDWR

One of these access modes must be used. Additional access modes may be used by

OR'ing with one of these. This might include modes such as append mode (which is not

addressed in this text). Refer to Appendix C, System Services for additional

information regarding the file access modes.

The arguments for the file open system service are as follows:

Register SYS_open

rax

rdi

rsi

Call code = SYS_open (2)

Address of NULL terminated file name string

File access mode flag

Assuming the following declarations:

SYS_open

equ

2

; file open

O_RDONLY

O_WRONLY

O_RDWR

equ

equ

equ

000000q ; read only

000001q ; write only

000002q ; read and write

It should be noted that the constants are defined in octal or base 8 (as specified with the

q suffix). This matches how Linux file permission are sometimes specified.

After the system call, the rax register will contain the return value. If the file open

operation fails, rax will contain a negative value (i.e., < 0). The specific negative value

provides an indication of the type of error encountered. Refer to Appendix C, System

Services for additional information on error codes. Typical errors might include invalid

file descriptor, file not found, or file permissions error.

Page 209

Chapter 13.0 ◄ System Services

If the file open operation succeeds, rax contains the file descriptor. The file descriptor

will be required for further file operations and should be saved.

Refer to the section on Example File Read for a complete example that opens a file.

1

3.5.2 File Open/Create

A file open/create operation will create a file. If the file does not exist, a new file will

be created. If the file already exists, it will be erased and a new file created. Thus, the

previous contents of the file will be lost.

A file access mode must be specified. Since the file is being created, the access mode

must include the file permissions that will be set when the file is created. This would

include specifying read, write, and/or execute permissions for the user, group, or world

as is typical for Linux file permissions. The only permissions addressed in this example

are for the user or owner of the file. As such, other users (i.e., using other accounts) will

not be able to access the file our program creates. Refer to Appendix C, System

Services for additional information regarding the file access modes.

The arguments for the file open/create system service are as follows:

Register SYS_creat

rax

rdi

rsi

Call code = SYS_creat (85)

Address of NULL terminated file name string

File access mode flag

Assuming the following declarations:

SYS_creat equ

85

; file open

O_CREAT

O_TRUNC

O_APPEND

equ

equ

equ

0x40

0x200

0x400

S_IRUSR

S_IWUSR

S_IXUSR

equ

equ

equ

00400q

00200q

00100q

; owner, read permission

; owner, write permission

; owner, execute permission

The file status flags “S_IRUSR | S_IWUSR” would allow simultaneous read and write,

which is typical. The “|” is a logical OR operation, thus combining the selections.

Page 210

Chapter 13.0 ◄ System Services

If the file open/create operation does not succeed, a negative value is returned in the rax

register. If file open/create operation succeeds, a file descriptor is returned. The file

descriptor is used for all subsequent file operations.

Refer to the section on Example File Write for a complete example file open/create.

1

3.6 File Read

A file must be opened with the appropriate file access flags before it can be read.

The arguments for the file read system service are as follows:

Register SYS_read

rax

rdi

rsi

rdx

Call code = SYS_read (0)

File descriptor (of open file)

Address of where to place characters read

Count of characters to read

Assuming the following declarations:

SYS_read

equ

0

; file read

If the file read operation does not succeed, a negative value is returned in the rax

register. If the file read operation succeeds, the number of characters actually read is

returned.

Refer to the next section on example file read for a complete file read example.

1

3.7 File Write

The arguments for the file write system service are as follows:

Register SYS_write

rax

rdi

rsi

rdx

Call code = SYS_write (1)

File descriptor (of open file)

Address of characters to write

Count of characters to write

Page 211

Chapter 13.0 ◄ System Services

Assuming the following declarations:

SYS_write equ

1

; file write

If the file write operation does not succeed, a negative value is returned in the rax

register. If the file write operation does succeed, the number of characters actually

written is returned.

Refer to the section on Example File Read for a compete file write example.

1

3.8 File Operations Examples

This section contains some simple example programs to demonstrate very basic file I/O

operations. The more complex issues regarding file I/O buffering are addressed in a

subsequent chapter.

1

3.8.1 Example, File Write

This example program writes a short message to a file. The file created contains a

simple message, a URL in this example. The file name and message to be written to the

file are hard-coded. This helps simplify the example, but is not realistic.

Since the open/create service is used, the file will be created (even if an old version must

be overwritten).

;  Example program to demonstrate file I/O.  This example

;  will open/create a file, write some information to the

;  file, and close the file. Note, the file name and

;  write message are hard-coded for the example.

section .data

;

-----

;  Define standard constants.

LF

equ

equ

equ

equ

10

0

1

; line feed

; end of string

NULL

TRUE

FALSE

0

EXIT_SUCCESS equ

0

; success code

STDIN

STDOUT

equ

equ

0

1

; standard input

; standard output

Page 212

Chapter 13.0 ◄ System Services

STDERR

equ

2

; standard error

SYS_read

SYS_write

SYS_open

SYS_close

SYS_fork

SYS_exit

SYS_creat

SYS_time

equ

equ

equ

equ

equ

equ

equ

equ

0

1

2

3

57

60

85

201

; read

; write

; file open

; file close

; fork

; terminate

; file open/create

; get time

O_CREAT

O_TRUNC

O_APPEND

equ

equ

equ

0x40

0x200

0x400

O_RDONLY

O_WRONLY

O_RDWR

equ

equ

equ

000000q

000001q

000002q

; read only

; write only

; read and write

S_IRUSR

S_IWUSR

S_IXUSR

equ

equ

equ

00400q

00200q

00100q

;

;

-----

Variables for main.

newLine

header

db

db

db

db

db

db

dq

LF, NULL

LF, "File Write Example."

LF, LF, NULL

"url.txt", NULL

"http://www.google.com"

LF, NULL

fileName

url

len

$-url-1

writeDone

fileDesc

errMsgOpen

db

dq

db

"Write Completed.", LF, NULL

0

"Error opening file.", LF, NULL

"Error writing to file.", LF, NULL

errMsgWrite db

;--------------------------------------------------------

section .text

global _start

_start:

Page 213

Chapter 13.0 ◄ System Services

;

-----

;  Display header line...

mov

rdi, header

call printString

;

;

;

-----

Attempt to open file.

Use system service for file open

;

;

;

;

System Service - Open/Create

rax = SYS_creat (file open/create)

rdi = address of file name string

rsi = attributes (i.e., read only, etc.)

;

;

;

Returns:

if error -> eax < 0

if success -> eax = file descriptor number

;

;

;

;

The file descriptor points to the File Control

Block (FCB).  The FCB is maintained by the OS.

The file descriptor is used for all subsequent

file operations (read, write, close).

openInputFile:

mov

rax, SYS_creat

; file open/create

; file name string

; allow read/write

; call the kernel

mov

mov

rdi, fileName

rsi, S_IRUSR | S_IWUSR

syscall

cmp

jl

rax, 0

errorOnOpen

; check for success

; save descriptor

mov qword [fileDesc], rax

-----

Write to file.

;

;

;

;

In this example, the characters to write are in a

predefined string containing a URL.

;

;

;

;

System Service - write

rax = SYS_write

rdi = file descriptor

rsi = address of characters to write

Page 214

Chapter 13.0 ◄ System Services

; rdx = count of characters to write

;  Returns:

; if error -> rax < 0

; if success -> rax = count of characters actually read

mov

mov

mov

rax, SYS_write

rdi, qword [fileDesc]

rsi, url

mov

rdx, qword [len]

syscall

cmp

jl

rax, 0

errorOnWrite

mov

rdi, writeDone

call printString

;

;

;

;

;

-----

Close the file.

System Service - close

rax = SYS_close

rdi = file descriptor

mov

mov

rax, SYS_close

rdi, qword [fileDesc]

syscall

jmp

exampleDone

;

;

;

;

-----

Error on open.

note, rax contains an error code which is not used

for this example.

errorOnOpen:

mov

rdi, errMsgOpen

call printString

jmp

exampleDone

;

;

;

-----

Error on write.

note, rax contains an error code which, not used here

Page 215

Chapter 13.0 ◄ System Services

errorOnWrite:

mov

rdi, errMsgWrite

call printString

jmp

exampleDone

;

-----

;  Example program done.

exampleDone:

mov

mov

rax, SYS_exit

rdi, EXIT_SUCCESS

syscall

;

;

;

;

;

;

**********************************************************

Generic function to display a string to the screen.

String must be NULL terminated.

Algorithm:

Count characters in string (excluding NULL)

Use syscall to output characters

;

;

;

Arguments:

1) address, string

Returns:  nothing

global printString

printString:

push rbp

mov

rbp, rsp

push rbx

;  Count characters in string.

mov

mov

rbx, rdi

rdx, 0

strCountLoop:

cmp

je

inc

inc

jmp

byte [rbx], NULL

strCountDone

rdx

rbx

strCountLoop

Page 216

Chapter 13.0 ◄ System Services

strCountDone:

cmp

je

rdx, 0

prtDone

;  Call OS to output string.

mov

mov

mov

rax, SYS_write

rsi, rdi

rdi, STDOUT

; code for write()

; addr of characters

; file descriptor

;

count set above

syscall

; system call

;  String printed, return to calling routine.

prtDone:

pop

rbx

rbp

pop

ret

; *******************************************************

This example creates the file which is read by the next example.

1

3.8.2 Example, File Read

This example will read a file. The file to be read contains a simple message, the URL

from the previous example. The file name is hard-coded which helps simplify the

example, but is not realistic. The file name used matches the previous file write

example. If this example program is executed prior to the write example program being

executed, it will generate an error since the file will not be found. After the file write

example program is executed, this file read example program will read the file and

display the contents.

;  Example program to demonstrate file I/O.

;

;

This example will open/create a file, write some

information to the file, and close the file.

;  Note, the file name is hard-coded for this example.

;  This example program will open a file, read the

;  contents, and write the contents to the screen.

;  This routine also provides some very simple examples

;  regarding handling various errors on system services.

Page 217

Chapter 13.0 ◄ System Services

; -------------------------------------------------------

section .data

;

-----

;  Define standard constants.

LF

NULL

equ

equ

10

0

; line feed

; end of string

TRUE

FALSE

EXIT_SUCCESS equ

STDIN

STDOUT

STDERR

equ

equ

1

0

0

0

1

2

; success code

equ

equ

equ

; standard input

; standard output

; standard error

SYS_read

SYS_write

SYS_open

SYS_close

SYS_fork

SYS_exit

SYS_creat

SYS_time

equ

equ

equ

equ

equ

equ

equ

equ

0

1

2

3

57

60

85

201

; read

; write

; file open

; file close

; fork

; terminate

; file open/create

; get time

O_CREAT

O_TRUNC

O_APPEND

equ

equ

equ

0x40

0x200

0x400

O_RDONLY

O_WRONLY

O_RDWR

equ

equ

equ

000000q

000001q

000002q

; read only

; write only

; read and write

S_IRUSR

S_IWUSR

S_IXUSR

equ

equ

equ

00400q

00200q

00100q

;

;

-----

Variables/constants for main.

BUFF_SIZE

newLine

equ 255

db

LF, NULL

Page 218

Chapter 13.0 ◄ System Services

header

db

db

LF, "File Read Example."

LF, LF, NULL

fileName

fileDes c

db

dq

"url.txt", NULL

0

errMsgOpen

errMsgRead

db

db

"Error opening the file.", LF, NULL

"Error reading from the file.", LF, NULL

; -------------------------------------------------------

section .bss

readBuffer resb BUFF_SIZE

; -------------------------------------------------------

section .text

global _start

_start:

; -----

;  Display header line...

mov

rdi, header

call printString

; -----

;  Attempt to open file - Use system service for file open

;

;

;

;

;

;

;

System Service - Open

rax = SYS_open

rdi = address of file name string

rsi = attributes (i.e., read only, etc.)

Returns:

if error -> eax < 0

if success -> eax = file descriptor number

;

;

;

;

The file descriptor points to the File Control

Block (FCB).  The FCB is maintained by the OS.

The file descriptor is used for all subsequent file

operations (read, write, close).

Page 219

Chapter 13.0 ◄ System Services

openInputFile:

mov

mov

mov

rax, SYS_open

rdi, fileName

rsi, O_RDONLY

; file open

; file name string

; read only access

; call the kernel

syscall

cmp

jl

rax, 0

errorOnOpen

; check for success

; save descriptor

mov

qword [fileDesc], rax

;

;

;

-----

Read from file.

For this example, we know that the file has only 1 line.

;

;

;

;

;

;

;

;

System Service - Read

rax = SYS_read

rdi = file descriptor

rsi = address of where to place data

rdx = count of characters to read

Returns:

if error -> rax < 0

if success -> rax = count of characters actually read

mov

mov

mov

mov

rax, SYS_read

rdi, qword [fileDesc]

rsi, readBuffer

rdx, BUFF_SIZE

syscall

cmp

jl

rax, 0

errorOnRead

;

;

;

-----

Print the buffer.

add the NULL for the print string

mov

mov

rsi, readBuffer

byte [rsi+rax], NULL

mov

rdi, readBuffer

call printString

Page 220

Chapter 13.0 ◄ System Services

printNewLine

-----

Close the file.

System Service - close

rax = SYS_close

;

;

;

;

;

rdi = file descriptor

mov

rax, SYS_close

mov

rdi, qword [fileDesc]

syscall

jmp

exampleDone

;

;

;

;

-----

Error on open.

note, eax contains an error code which is not used

for this example.

errorOnOpen:

mov

rdi, errMsgOpen

call printString

jmp

exampleDone

;

;

;

;

-----

Error on read.

note, eax contains an error code which is not used

for this example.

errorOnRead:

mov

rdi, errMsgRead

call printString

jmp

exampleDone

; -----

;  Example program done.

exampleDone:

mov

mov

rax, SYS_exit

rdi, EXIT_SUCCESS

syscall

Page 221

Chapter 13.0 ◄ System Services

;

;

;

;

;

;

**********************************************************

Generic procedure to display a string to the screen.

String must be NULL terminated.

Algorithm:

Count characters in string (excluding NULL)

Use syscall to output characters

;

;

;

;

Arguments:

1) address, string

Returns:

nothing

global printString

printString:

push rbp

mov

rbp, rsp

push rbx

;

;

-----

Count characters in string.

mov

mov

rbx, rdi

rdx, 0

strCountLoop:

cmp

je

inc

inc

jmp

byte [rbx], NULL

strCountDone

rdx

rbx

strCountLoop

strCountDone:

cmp

je

rdx, 0

prtDone

;

-----

;  Call OS to output string.

mov

mov

mov

rax, SYS_write

rsi, rdi

rdi, STDOUT

; code for write()

; addr of characters

; file descriptor

;

count set above

syscall

; system call

Page 222

Chapter 13.0 ◄ System Services

;

;

-----

String printed, return to calling routine.

prtDone:

pop

rbx

rbp

pop

ret

The printString() function is the exact same in both examples and is only repeated to

allow each program to be assembled and executed independently.

1

3.9 Exercises

Below are some quiz questions and suggested projects based on this chapter.

1

3.9.1 Quiz Questions

Below are some quiz questions based on this chapter.

1

2

) When using a system service, where is the call code placed?

) Where is the code located when the syscall instruction is executed (in the

user program or in the operating system)?

3

) What is the call code and required arguments for a system service call to perform

console output?

4

5

6

7

) Why was only one character read for interactive keyboard input?

) What is returned for a successful file open system service call?

) What is returned for an unsuccessful file open system service call?

) If a system service call requires six (6) arguments, where specifically should

they be passed?

1

3.9.2 Suggested Projects

Below are some suggested projects based on this chapter.

1

) Implement the printString() example void function and a simple main to test of a

series of strings. Use the debugger to execute the program and display the final

results. Execute the program without the debugger and verify the appropriate

output is displayed to the console.

Page 223

Chapter 13.0 ◄ System Services

2

) Convert the example program to read input from keyboard into a readString()

function. The function should accept arguments for the string address and

maximum string length (in that order). The maximum length should include

space for the NULL (an extra byte), which means the function must not allow

more than the maximum minus one characters to be stored in the string. If

additional characters are entered by the user, they should be cleared from the

input stream, but not stored. The function should not include the newline in the

returned string. The function should return the number of characters in the string

not including the NULL. The printString() function from the previous problem

should be used unchanged. When done, create an appropriate main to test the

function. Use the debugger as necessary to debug the program. When working

correctly, execute the program from the command line which will display the

final results to the console.

3

) Based on the file write example, create a value returning fileWrite() function to

write a password to a file. The function should accept arguments for the address

of the file name and the address of the NULL terminated password string. The

file should be created, opened, the password string written to the file, and the file

closed. The function should return SUCCESS if the operations worked correctly

or NOSUCCESS if there is a problem. Problems might include not being able to

create the file or not being able to write to the file. Create an appropriate main to

test the function. Use the debugger as necessary to debug the program. When

working correctly, execute the program from the command line which will

display the final results to the console.

4

) Based on the file read example, create a value returning fileRead() function to

read a password from a file. The function should accept arguments for the

address of file name, the address of where to store the password string, the

maximum length of the password string, and the address of where to store the

password length. The function should open the file, read a string representing a

password, close the file, and return the number of characters in the password.

The maximum length should include space for the NULL, which means the

function read must not store more than the maximum minus one characters in the

string. The function should return SUCCESS if the operations worked correctly

or NOSUCCESS if there is a problem. Problems might include the file not

existing or other read errors. Create an appropriate main to test the function.

Use the debugger as necessary to debug the program. When working correctly,

execute the program from the command line which will display the final results

to the console.

Page 224

Chapter

14

My new computer is so fast, it executes an

infinite loop in 6 seconds.

14.0 Multiple Source Files

As a program grows in size and complexity, it is common that different parts of the

program are stored in different source files. This allows a programmer to ensure the

source files do not become unreasonably large and would also allow multiple

programmers to more easily work on different parts of the program.

The function calls, even those written by different programmers, will work together

correctly due to the standard calling convention as outlined in Chapter 12, Functions.

This is even true when interfacing with a high-level language.

This chapter will present a simple assembly language example to demonstrate how to

create and use source code in multiple files. In addition, an example of how to interface

with a C/C++ source file is provided.

1

4.1 Extern Statement

If a function is called from a source file and the function code is not located in the

current source file, the assembler will generate an error. The same applies to variables

accessed that are not located in the current file. In order to inform the assembler that the

function code or variables are in another file, the extern statement is used. The syntax

is as follows:

extern <symbolName>

The symbol name would be the name of the function or a variable that is located in a

different source file. In general, using global variables accessed across multiple files is

considered poor programming practice and should be used sparingly (if at all). Data is

typically passed between functions as arguments for the function call.

The examples in the text focus on using external functions only with no globally

declared variables.

Page 225

Chapter 14.0 ◄ Multiple Source Files

1

4.2 Example, Sum and Average

The following is a simple example of a main that calls an assembly language function,

stats(), to compute the integer sum and integer average for a list of signed integers. The

main and the function are in different source files and are presented as an example of

how multiple source files are used. The example itself is really too small to actually

require multiple source files.

1

4.2.1 Assembly Main

The main is as follows:

;  Simple example to call an external function.

;

;

----------------------------------------------------------

Data section

section .data

; -----

;  Define standard constants

LF

NULL

equ

equ

10

0

; line feed

; end of string

TRUE

FALSE

equ

equ

1

0

EXIT_SUCCESS equ

SYS_exit equ

0

60

; success code

; terminate

;

;

-----

Declare the data

lst1

len1

lst2

len2

dd  1, -2,  3, -4,  5

dd  7,  9,  11

dd  8

dd  2, -3,  4, -5,  6

dd -7, 10, 12, 14, 16

dd 10

Page 226

Chapter 14.0 ◄ Multiple Source Files

section .bss

sum1

ave1

resd

resd

1

1

sum2

ave2

resd

resd

1

1

; ----------------------------------------------------------

extern stats

section .text

global _start

_start:

;

;

;

----

Call the function

HLL Call:  stats(lst, len, &sum, &ave);

mov

mov

mov

mov

rdi, lst1

esi, dword [len1]

rdx, sum1

; data set 1

rcx, ave1

call stats

mov

mov

mov

mov

rdi, lst2

esi, dword [len2]

rdx, sum2

; data set 2

rcx, ave2

call stats

;

;

-----

Example program done

exampleDone:

mov

mov

rax, SYS_exit

rdi, EXIT_SUCCESS

syscall

The above main can be assembled with the same assemble command as described in

Chapter 5, Tool chain. The extern statement will ensure that the assembler does not

generate errors.

Page 227

Chapter 14.0 ◄ Multiple Source Files

1

4.2.2 Function Source

The function, in a different source file is as follows:

;  Simple example void function.

;

;

;

;

**********************************************************

Data declarations

Note, none needed for this example.

If needed, they would be declared here as always.

section .data

**********************************************************

section .text

;

; --------------------------------------------------

;  Function to find integer sum and integer average

;  for a passed list of signed integers.

;

;

Call:

stats(lst, len, &sum, &ave);

;

;

;

;

;

Arguments Passed:

1) rdi - address of array

2) rsi - length of passed array

3) rdx - address of variable for sum

4) rcx - address of variable for average

;

;

;

Returns:

sum of integers (via reference)

average of integers (via reference)

global stats

stats:

push r12

; -----

;  Find and return sum.

mov

mov

r11, 0

r12d, 0

; i=0

; sum=0

Page 228

Chapter 14.0 ◄ Multiple Source Files

sumLoop:

mov

eax, dword [rdi+r11*4] ; get lst[i]

add

inc

r12d, eax

r11

; update sum

; i++

cmp

jb

r11, rsi

sumLoop

mov

dword [rdx], r12d

; return sum

;

;

-----

Find and return average.

mov

cdq

eax, r12d

idiv esi

mov

dword [rcx], eax

; return average

;

-----

;  Done, return to calling function.

pop

ret

r12

The above source file can be assembled with the same assemble command as described

in Chapter 5, Tool chain. No extern statement is required since no external functions

are called.

1

4.2.3 Assemble and Link

Assuming the main source file is named main.asm and the functions source file is

named stats.asm, the following command will perform the assemble and link.

yasm -g dwarf2 -f elf64 main.asm -l main.lst

yasm -g dwarf2 -f elf64 stats.asm -l stats.lst

ld -g -o main main.o stats.o

The files names can be changed as desired.

As usual, the debugger is started with the ddd <executable> command. It would be

appropriate to note that using the debugger step command will step into the function,

including showing the function source code (even if the source is in a different file).

Page 229

Chapter 14.0 ◄ Multiple Source Files

The debugger next command will execute the entire function and not show the source.

If the function is working, the next command would be most useful. In order to debug

the function, the step command would be most useful.

If the “-g” option is omitted, the debugger will not be able to display the source code.

1

4.3 Interfacing with a High-Level Language

This section provides information on how a high-level language can call an assembly

language function and how an assembly language function can call a high-level

language function. This chapter presents examples for both.

In brief, the answer of how this is accomplished is through the standard calling

convention. As such, no additional or special code is needed when interfacing with a

high-level language. The compiler or assembler will need to be informed about the

external routines in order to avoid error messages about not being able to find the source

code for the non-local routines.

The general process for linking for multiple files was described in Chapter 5, Tool

Chain. The process of using multiple source files was described in Chapter 14, Multiple

Source Files. It does not matter if the object files are from a high-level language or from

an assembly language source.

1

4.3.1 Example, C++ Main / Assembly Function

When calling any functions that are in a separate source file, the compiler must be

informed that the function or functions source code are external to the current source

file. This is performed with an extern statement in C or C++. Other languages will

have a similar syntax. For a high-level language, the extern statement will include the

function prototype which will allow the compiler to verify the function parameters and

associated types.

The following is a simple example using a C++ main with an assembly language

function.

#include <iostream>

using namespace std;

extern "C" void stats(int[], int, int *, int *);

int main()

{

int lst[] = {1, -2, 3, -4, 5, 7, 9, 11};

Page 230

Chapter 14.0 ◄ Multiple Source Files

int len = 8;

int sum, ave;

stats(lst, len, &sum, &ave);

cout << "Stats:" << endl;

cout << "  Sum = " << sum << endl;

cout << "  Ave = " << ave << endl;

return 0;

}

At this point, the compiler does not know the external function is written in assembly

(nor does it matter).

The C compiler is pre-installed on Ubuntu. However, the C++ compiler is not installed

by default.

A C version of the same program is also presented for completeness.

#

include<stdio.h>

extern void stats(int[], int, int *, int *);

int main()

{

int lst[] = {1, -2, 3, -4, 5, 7, 9, 11};

int len = 8;

int sum, ave;

stats(lst, len, &sum, &ave);

printf ("Stats:\n");

printf ("  Sum = %d \n", sum);

printf ("  Ave = %d \n", ave);

return 0;

}

The stats() function referenced here should be used unchanged from the previous

example.

Page 231

Chapter 14.0 ◄ Multiple Source Files

1

4.3.2 Compile, Assemble, and Link

As noted in the Functions Chapter, the C++ compiler should be used. For example,

assuming that the C++ main is named main.cpp, and the assembly source file is named

stats.asm, the commands to compile, assemble, link, and execute as follows:

g++ -g -Wall -c main.cpp

yasm -g dwarf2 -f elf64 stats.asm -l stats.lst

g++ -g -o main main.o stats.o

Note, Ubuntu 18 and above will require the no-pie option on the g++ as shown:

g++ -g -no-pie -o main main.o stats.o

The file names can be changed as desired. Upon execution, the output would be as

follows:

./main

Stats:

Sum = 30

Ave = 3

If a C main is used, and assuming that the C main is named main.c, and the assembly

source file is named stats.asm, the commands to compile, assemble, link, and execute as

follows:

g++ -g -Wall -c main.cpp

yasm -g dwarf2 -f elf64 stats.asm -l stats.lst

g++ -g -o main main.o stats.o

Note, Ubuntu 18 and above will require the no-pie option on the gcc as shown:

g++ -g -no-pie -o main main.o stats.o

The C compiler, gcc, or the C++ compiler, g++, is used to perform the linking as it is

aware of the appropriate locations for the various C/C++ libraries.

1

4.4 Exercises

Below are some quiz questions and suggested projects based on this chapter.

Page 232

Chapter 14.0 ◄ Multiple Source Files

1

4.4.1 Quiz Questions

Below are some quiz questions based on this chapter.

1

) What is the statement to declare the functions, func1() and func2(), as external

assuming no arguments are needed for either function?

2

) What is the statement to declare the functions, func1() and func2(), as external if

two integer arguments are used for each function?

3

4

) What will happen if an external function is called but is not declared as external?

) If an externally declared function is called but the programmer did not actually

write the function, when would the error be flagged, assemble-time, link-time, or

run-time?

5

6

) If an externally declared function is called but the programmer did not actually

write the function, what might the error be?

) If the “-g” option is omitted from the assemble and link commands, would the

program be able to execute?

1

4.4.2 Suggested Projects

Below are some suggested projects based on this chapter.

1

) Implement the assembly language example program to find the sum and average

for a list of signed integers. Ensure that the main and function are in different

source files. Use the debugger to execute the program and display the final

results.

2

) Based on the example function stats(), split it into two value returning functions,

lstSum() and lstAverage(). As noted in Chapter 12, Functions, value returning

functions return their result in the A register. Since these are double-words, the

result will be returned in eax. Ensure that the main and both functions are in two

different source files. The two functions can be in the same source file. Use the

debugger to execute the program and display the final results.

3

) Extend the previous exercise to display the sum and average to the console. The

printString() example function (from multiple previous examples) should be

placed in a third source file (which can be used on other exercises). This project

will require a function to convert an integer to ASCII (as outlined in Chapter 10).

Use the debugger as needed to debug the program. When working, execute the

program without the debugger and verify that the correct results are displayed to

the console.

Page 233

Chapter 14.0 ◄ Multiple Source Files

4

) Implement one of the C/C++ example main programs (either one). Additionally,

implement the assembly language stats() function example. Develop a simple

bash script to perform the compile, assemble, and link. The link should be

performed with the applicable C/C++ compiler. Use the debugger as needed to

debug the program. When working, execute the program without the debugger

and verify that the correct results are displayed to the console.

Page 234

Chapter

15

My new computer is so fast it requires two

HALT instructions to stop it.

15.0 Stack Buffer Overflow

A stack buffer overflow44 can occur when a program overflows a stack-based dynamic

variable (as described in Chapter 12.9, Stack-Based Local Variables). For example, if a

program allocates and uses a stack-based local array holding 50 elements and more than

5

0 elements are stored in the array, an overflow occurs. Such overflows are generally

bad and typically cause program bugs and possibly even crash the program. The stack

will contain other important information such as other variables, preserved registers,

frame pointer, return address, and/or stack-based parameters. If such data is

overwritten, it will likely cause problems which can be very difficult to debug since the

symptom will likely be unrelated to where the problem actually occurs.

If a stack buffer overflow is caused deliberately as part of an attack it is referred to as

stack smashing. Due to the standard calling convention, the layout of the stack-based

call frame or activation record is fairly predictable. Such a stack buffer overflow can be

by a malicious individual to inject executable code into the currently running program to

perform some inappropriate actions. Under the right circumstances, such code injection

could allow a black-hat45 hacker to perform unwanted actions potentially taking over the

system.

The process of how a stack buffer overflow occurs and how it can be exploited are

provided in this chapter. This is presented in order to allow developers to clearly

understand the problem and thus learn how to protect themselves against such

vulnerabilities. The reader must be familiar with the details of the standard calling

convention as outlined in Chapter 12, Functions.

It should be noted that the stack buffer overflow problem exists in high-level languages.

Working in assembly languages makes it easier to more clearly see and understand the

details.

4

4

4 For more information, refer to: http://en.wikipedia.org/wiki/Stack_buffer_overflow

5 For more information, refer to: http://en.wikipedia.org/wiki/Black_hat

Page 235

Chapter 15.0 ◄ Stack Buffer Overflow

1

5.1 Understanding a Stack Buffer Overflow

When a program calls a function, the standard calling convention provides the guidance

for how the parameters are passed, how the return address is saved, how and which

registers must be preserved, and how stack-based local variables are to be allocated.

For example, consider the function call;

expFunc(arg1, arg2, arg3);

In addition, we will assume that the function, expFunc(), reads in a line of text from the

user and stores it in a locally declared array. The local variables include the array of 48

bytes and one quadword local variable (8 bytes). This can be accomplished easily in a

high-level language or as is done in Chapter 13, System Services, Console Input.

The resulting call frame would be as follows:

.

. .

rip

← return address

rbp

← preserved base pointer

←

←

Local array (48 bytes)

Local quad variable (8 bytes)

rbx

r12

r13

← preserved saved register

← preserved saved register

← preserved saved register

.

. .

Illustration 26: Stack Call Frame Example

Page 236

Chapter 15.0 ◄ Stack Buffer Overflow

When the function completes, all elements of the call frame are removed. The preserved

registers are restored to their original contents, the local variables are removed, and the

return address is copied off the stack and placed in the rip register which effects a jump

back to the original calling routine. As noted in Chapter 12, Functions, this layout

supports multiple levels of function calls including recursion.

The example in Chapter 13 that reads characters from the user and entered them into an

array explicitly checked the character count to ensure that the count does not exceed the

buffer size. It would be easy to forget to perform this important check. Indeed, some C

functions such as strcpy() do not perform verification of sizes and are thus considered

unsafe46 or described as an unsafe function.

In this example, over-writing the 48 character buffer will destroy the contents of the

stack that contains the original values for the rbp register and possibly the rip register.

If the stack content of the original value of the rip register is altered or corrupted in any

way, the function will not be able to return to the calling routine and would attempt to

jump to some random location. If the random location is outside the program scope,

which is likely, the program will generate a segment fault (i.e., “seg fault” or program

crash).

Debugging this type of error can be a significant challenge. The error appears at the

very end of the function (on the return). The problem is actually in the body of the

function and in fact may be due to code that is not included (as opposed to wrong code

that is there).

Testing if a program has this vulnerability would involve typing significantly more

characters than expected when prompted. For example, if name is prompted for and 200

characters are entered (possibly by holding down a key), the program crashing would be

a sign that such a vulnerability exists. An error message would indicate such a

vulnerability does not exist. While straightforward, this type of testing is often not

checked thoroughly or even omitted entirely.

1

5.2 Code to Inject

Before discussing how the stack buffer overflow might be exploited, we will review

what code might be injected. The code to be injected could be many things. We will

assume the program is being executed in a controlled environment with no console

access for the user. The lack of console access would limit what a user could do ideally

to only what the program allowed. This might be the case if the program is server-based

4

6 For more information, refer to:

http://en.wikipedia.org/wiki/C_standard_library#Buffer_overflow_vulnerabilities

Page 237

Chapter 15.0 ◄ Stack Buffer Overflow

interacting with a user through a web or application front-end. The server would be

protected from direct user access for obvious security reasons.

As such, one possible attack vector might be to obtain console access to the back-end

server (where such access is normally not allowed).

There is a system service that can execute another program. Using this system service,

exec, we could execute the sh program (shell) which is a standard Linux program. This

will create and open a shell providing console access. The newly created shell is limited

to the privileges of the process that started it. In a good security environment, the

process will be granted only the privileges it requires. In a poor security environment,

the process may have additional, unnecessary privileges which would make it easier for

an unauthorized black-hat hacker to cause problems.

Given the following data declarations:

NULL           equ     0

progName       db      "/bin/sh", NULL

An example of the exec system service would be as follows:

;  Example, system service call for exec.

mov     rax, 59

mov     rdi, progName

syscall

A new console will appear. The reader is encouraged to try this code example and see it

work.

The list file for this code fragment would be as follows:

40 00000000 48C7C03B000000          mov rax, 59

41 00000007 48C7C7[00000000]        mov rdi, progName

42 0000000E 0F05                    syscall

Recall that the first column is the line number, the second column is relative address in

the code section and the third column is the machine language or the hex representation

of the human readable instruction shown in the fourth column. The [00000000]

represents the relative address of the string in the data section (progName in this

example). It is zero since it is the first (and only) variable in the data section for this

example.

Page 238

Chapter 15.0 ◄ Stack Buffer Overflow

If the machine language shown is entered into memory and executed, a shell or console

would be opened. Getting the hex version of the code into memory can only be

performed via the keyboard (since there is no direct access to file system). This would

be done one character at a time. The 0x48 is the ASCII code for “0”, so “0” could be

entered for that byte. However, the 0x0f and many of the other characters are more

difficult to enter directly as ASCII.

The command shell will allow entry of hex code in hex. By typing control key and the

left shift followed by a lower-case u and then four hex digits (must be hex), the hex

values can be entered one at a time. For example, to enter the 0x3f, it would be;

CTRL SHIFT u 3 f

This can be done for most of the bytes except the 0x00. The 0x00 is a NULL which is a

non-printable ASCII characters (used to mark string terminations). As such, the NULL

cannot be entered from the keyboard. Additionally, the [00000000] address would not

make sense if the code is injected into another program.

To address these issues, we can re-write the example code and eliminate the NULL's

and change the address reference. The NULL's can be eliminated by using different

instructions. For example, setting rax to 59 can be accomplished by xor'ing rax with

itself and placing the 59 in al (having already ensured the upper 56 bits are 0 via the

xor). The string can be placed on the stack and the current rsp used as the address of the

string. The string, “\bin\sh” is 7 bytes and the stack operation will require a push of 8

bytes. Again, the NULL cannot be entered and is not counted. An extra, unnecessary

“/” can be added to the string which will not impact the operation providing exactly 8

bytes in the string. Since the architecture is little-endian, in order to ensure that the start

of the string is in low memory, it must be in the least significant byte of the push. This

will make the string appear backwards.

The revised program fragment would be as follows:

xor

rax, rax

; clear rax

push rax

; place NULLs on stack

; string -> "//bin/sh"

; put string in memory

; call code in rax

; rdi = addr of string

; system call

mov

rbx, 0x68732f6e69622f2f

push rbx

mov

mov

al, 59

rdi, rsp

syscall

Page 239

Chapter 15.0 ◄ Stack Buffer Overflow

The list file for the program fragment would be as follows:

52 00000013 4831C0                xor rax, rax

53 00000016 50                    push rax

54 00000017 48BB2F2F62696E2F73    mov rbx, 0x68732f6e69622f2f

55 00000017 68

56 00000021 53                    push rbx

57 00000022 B03B                  mov al, 59

58 00000024 4889E7                mov rdi, rsp

59 00000027 0F05                  syscall

In this revised code, there are no NULL's and the address reference is obtained from the

stack pointer (rsp) which points to the correct string.

There is an assembly language instruction nop which performs no operation with a

machine code of 0x90. In this example, the nop instruction is used simply to round out

the machine code to an even multiple of 8 bytes.

The series of hex values that would need to be entered is as follows:

0x48 0x31 0xC0 0x50 0x48 0xBB 0x2F 0x2F

0x62 0x69 0x6E 0x2F 0x73 0x68 0x53 0xB0

0x3B 0x48 0x89 0xE7 0x0F 0x05 0x90 0x90

While somewhat tedious, these characters can be entered by hand.

1

5.3 Code Injection

If the code to inject is available and can be entered, the next step would be actually

getting the code executed.

Based on the previous example call frame, the code would be entered preceded by a

series of nop's (0x90). The exact spot where the rip is stored in the stack can be

determined through trial and error. When the first byte of the 8 byte address is altered,

the program will not be able to return to the calling routine and will likely crash. If the

bytes of the rbp are corrupted, the program may fail in some way, but it will be different

than the immediate crash caused by the corrupted rip. The code entered would be

extended by 1 byte on each of many successive attempts. Finding this exact location in

this manner will take patience.

Once the rip location has been determined, the 8 bytes that are entered there will need to

be the address of where the injected code is in the stack where the user input was stored.

This also would be determined through trial and error. However, the exact address of

the start of the injected code is not required. Starting anywhere within the preceding

Page 240

Chapter 15.0 ◄ Stack Buffer Overflow

nop's would be sufficient. This is referred to as a NOP slide47 which will help “slide”

the CPU's instruction execution flow to the injected code.

.

. .

←

return address (altered)

9

9

090909090909090

090909090909090

← preserved base pointer (destroyed)

← trailing nop's

0

50FE78948539090

8732F6E69622F2F

BB483BB052C03148 ← injected code

6

9

9

090909090909090

090909090909090

← nop's

← Local array (48 bytes)

←

Local variable (8 bytes)

rbx

r12

r13

← preserved saved register

← preserved saved register

← preserved saved register

.

. .

Illustration 27: Stack Call Frame Corruption

A larger local array would allow even more room for a longer NOP Slide.

1

5.4 Code Injection Protections

A number of methods have been developed and implemented to protect against the stack

buffer overflow. Some of these methods are summarized here. It must be noted that

none of these methods are completely perfect.

4

7 For more information, refer to: http://en.wikipedia.org/wiki/NOP_slide

Page 241

Chapter 15.0 ◄ Stack Buffer Overflow

1

5.4.1 Data Stack Smashing Protector (or Canaries)

Data stack smashing protector, also referred to as stack canaries, is used to detect a stack

buffer overflow before execution of malicious code can occur. This works by placing

an integer, the value of which is randomly chosen at program start, in memory just

before the return address (rip) in the call frame. In order to overwrite the return address,

and thus execute the injected code, the canary value must also be overwritten. This

canary value is checked to make sure it has not changed before a routine pops the return

address.

For the GNU g++ compiler, this option (-f-stack-protector) is enabled by default. It

can be turned off with the -fno-stack-protector compiler option. Turning it off would

be necessary in order to perform testing using a C/C++ program of the injection

techniques outlined in this chapter.

1

5.4.2 Data Execution Prevention

Data Execution Prevention48 (DEP) is a security feature that marks areas of memory as

either "executable" or "nonexecutable". Only code in a marked "executable" area is

allowed to be executed. Code that is or injected into an area marked “nonexecutable”

will not be allowed to execute. This helps prevent stack buffer overflow code injection.

1

5.4.3 Data Address Space Layout Randomization

Address space layout randomization (ASLR) is a technique to prevent an attacker from

reliably jumping to the injected code. ASLR randomly arranges the address space

positions of key data areas of a process, including the base of the executable and the

positions of the stack, heap and libraries.

1

5.5 Exercises

Below are some quiz questions and suggested projects based on this chapter.

1

5.5.1 Quiz Questions

Below are some quiz questions based on this chapter.

1

) When stack buffer overflow is caused deliberately as part of an attack it is

referred as what?

2

) What does it mean when a C function is considered unsafe?

4

8 For more information, refer to: http://en.wikipedia.org/wiki/Data_Execution_Prevention

Page 242

Chapter 15.0 ◄ Stack Buffer Overflow

3

4

) Is a program that reads user input still vulnerable if the input buffer is

sufficiently large (e.g., >1024 bytes)?

) How might an attacker determine if an interactive program is vulnerable to a

buffer overflow attack?

5

6

) What is a “NOP slide”?

) The text example injected code to open a new shell. Provide at least one

different idea for injected code that would cause problems.

7

) Name three techniques designed to prevent stack buffer overflow attacks.

1

5.5.2 Suggested Projects

Below are some suggested projects based on this chapter.

1

) Implement the second example program fragment to open a new shell. Use the

debugger to execute the program and display the final results. Execute the

program without the debugger and verify that a new shell is opened.

2

) Implement the console input program from Chapter 13. Remove the code for the

buffer size check. Execute the program without the debugger and ensure the

appropriate input is read and that the output is displayed to the console. Verify

that entering too many characters will crash the program.

3

) Using the program from the previous question and the program fragment to open

a shell, attempt to inject the code into the running program. In order to save

time, print the value of the rsp at an appropriate location to allow the guessing of

the target address significantly easier.

Page 243

Chapter 15.0 ◄ Stack Buffer Overflow

Page 244

Chapter

16

The code that is the hardest to debug is the

code you know can't possibly be wrong.

16.0 Command Line Arguments

This chapter provides a summary of how the operating system handles command line

arguments and how assembly language routines can access the arguments.

The term “command line arguments” is used to refer to the characters, if any, entered on

the command line after the program name. The command line arguments are often used

to provide information or parameters to the program without the need for the I/O

associated with interactive prompting. Of course, if the parameters are not correct, the

entire line must be re-entered.

Command line arguments are used by the assembler and linker to provide information

for input files, output files, and various other options.

1

6.1 Parsing Command Line Arguments

The operating system is responsible for parsing, or reading, the command line

arguments and delivering the information to the program. It is the responsibility of the

program to determine what is considered correct and incorrect. When reading the

command line, the operating system will consider an argument a set of non-space

characters (i.e., a string). The space or spaces between arguments is removed or ignored

by the operating system. In addition, the program name itself is considered the first, and

possibly only, argument. If other arguments are entered, at least one space is required

between each argument.

All arguments are delivered to the program as character strings, even number

information. If needed, the program must convert the character data into the number

value (float or integer).

For example, executing the program expProg with the following command line

arguments:

./expProg  one    42 three

Page 245

Chapter 16.0 ◄ Command Line Arguments

will result in four arguments as follows:

1

2

3

4

.   ./expProg

.   one

.   42

.   three

The command line arguments are delivered to the program as parameters. As such, the

program is like a function being called by the operating system. So, the standard calling

convention is used to convey the parameters.

1

6.2 High-Level Language Example

In general, it is assumed that the reader is already familiar with the basic C/C++

command line handling. This section presents a brief summary of how the C/C++

language handles the delivery of command line information to the program.

The count of arguments is passed to the main program as the first integer parameter,

typically called argc. The second parameter, typically called argv, is an array of

addresses for each string associated with the corresponding argument.

For example, the following C++ example program will read the command line

arguments and display them to the screen.

#

#

include <iomanip>

include <iostream>

using namespace std;

int main(int argc, char* argv[])

{

string bars;

bars.append(50,'-');

cout << bars << endl;

cout << "Command Line Arguments Example"

<< endl << endl;

cout << "Total arguments provided: " <<

argc << endl;

cout << "The name used to start the program: "

<< argv[0] << endl;

Page 246

Chapter 16.0 ◄ Command Line Arguments

if (argc > 1) {

cout << endl << "The arguments are:" << endl;

for (int n = 1; n < argc; n++)

cout << setw(2) << n << ": " <<

argv[n] << endl;

}

cout << endl;

cout << bars << endl;

return 0;

}

Assuming the program is named argsExp, executing this program will produce the

following output:

./argsExp   one 34        three

-

-------------------------------------------------

Command Line Arguments Example

Total arguments provided: 4

The name used to start the program: ./argsExp

The arguments are:

1: one

2: 34

3: three

--------------------------------------------------

It should be noted that the parameter '34' is a string. This example simply printed the

string to the console. If a parameter is to be used as a numeric value, the program must

convert as required. This includes all error checking as necessary.

1

6.3 Argument Count and Argument Vector Table

Since the operating system will call the main program as a function, the standard calling

convention applies. The argument count and the argument vector address are passed as

parameters in rdi and rsi in accordance with the standard calling convention. The first

argument, in rdi, is the integer argument count. The second argument, in rsi, is the

argument vector table address.

Page 247

Chapter 16.0 ◄ Command Line Arguments

The argument vector table is an array containing the quadword addresses of the string

for each argument. Assuming the previous examples with 4 total arguments, the basic

argument vector table layout is as follows:

Table Contents

C/C++ Reference

quadword

address

of 4th argument

argv[3]

argv[2]

argv[1]

argv[0]

quadword

address

of 3rd argument

quadword

address

of 2nd argument

quadword

address

of 1st argument

argument vector (rsi) →

Illustration 28: Argument Vector Layout

Each string is NULL terminated by the operating system and will not contain a new line

character.

1

6.4 Assembly Language Example

An example assembly language program to read and display the command line

arguments is included for reference. This example simply reads and displays the

command line arguments.

;

Command Line Arguments Example

; -------------------------------------------------------

section .data

; -----

;  Define standard constants.

Page 248

Chapter 16.0 ◄ Command Line Arguments

LF

equ

equ

equ

equ

10

0

1

; line feed

; end of string

NULL

TRUE

FALSE

0

EXIT_SUCCESS equ

0

; success code

STDIN

STDOUT

STDERR

equ

equ

equ

0

1

2

; standard input

; standard output

; standard error

SYS_read

SYS_write

SYS_open

SYS_close

SYS_fork

SYS_exit

SYS_creat

SYS_time

equ

equ

equ

equ

equ

equ

equ

equ

0

1

2

3

57

60

85

201

; read

; write

; file open

; file close

; fork

; terminate

; file open/create

; get time

;

;

-----

Variables for main.

newLine

db

LF, NULL

; ------------------------------------------------------

section .text

global main

main:

;

;

;

;

;

-----

Get command line arguments and echo to screen.

Based on the standard calling convention,

rdi = argc (argument count)

rsi = argv (starting address of argument vector)

mov

mov

r12, rdi

r13, rsi

; save for later use...

;

;

;

;

-----

Simple loop to display each argument to the screen.

Each argument is a NULL terminated string, so can just

print directly.

Page 249

Chapter 16.0 ◄ Command Line Arguments

printArguments:

mov

rdi, newLine

call printString

mov

printLoop:

mov

rbx, 0

rdi, qword [r13+rbx*8]

call printString

mov

rdi, newLine

call printString

inc

cmp

jl

rbx

rbx, r12

printLoop

;

-----

;  Example program done.

exampleDone:

mov

mov

rax, SYS_exit

rdi, EXIT_SUCCESS

syscall

;

;

;

;

;

;

;

;

;

;

**********************************************************

Generic procedure to display a string to the screen.

String must be NULL terminated.

Algorithm:

Count characters in string (excluding NULL)

Use syscall to output characters

Arguments:

1) address, string

Returns:

nothing

global printString

printString:

push rbp

mov

push rbx

-----

Count characters in string.

rbp, rsp

;

;

Page 250

Chapter 16.0 ◄ Command Line Arguments

mov

mov

rbx, rdi

rdx, 0

strCountLoop:

cmp

je

inc

inc

jmp

byte [rbx], NULL

strCountDone

rdx

rbx

strCountLoop

strCountDone:

cmp

je

rdx, 0

prtDone

;

-----

;  Call OS to output string.

mov

mov

mov

rax, SYS_write

rsi, rdi

edi, STDOUT

; code for write()

; addr of characters

; file descriptor

;

count set above

syscall

; system call

;

;

-----

String printed, return to calling routine.

prtDone:

pop

rbx

rbp

pop

ret

; *******************************************************

The printString() function is repeated in this example and is unchanged from the

previous examples.

It must be noted that in order for this to work, the program should be assembled as usual

but linked with the GNU C compiler, either GCC or G++.

For example, assuming this example program is named cmdLine.asm, the assembly and

linking would be as follows:

yasm -g dwarf2 -f elf64 cmdLine.asm -l cmdLine.lst

gcc -g -o cmdLine cmdLine.o

Page 251

Chapter 16.0 ◄ Command Line Arguments

Note, Ubuntu 18 will require the no-pie option on the gcc command as shown:

gcc -g -no-pie -o cmdLine cmdLine.o

If the standard linker is used, the arguments will not be passed in the correct manner.

1

6.5 Exercises

Below are some quiz questions and suggested projects based on this chapter.

1

6.5.1 Quiz Questions

Below are some quiz questions based on this chapter.

1

) What software entity is responsible for parsing or reading the command line

arguments?

2

) What software entity is responsible for verifying or checking the command line

arguments?

3

4

5

6

7

) What is the first command line argument?

) Explain what argc and argv refer to.

) In an assembly language program, where is argc passed to the program?

) In an assembly language program, where is argv passed to the program?

) If seven spaces are entered between each of the command line arguments, how

are the spaces removed when the command line arguments are checked?

8

) If a number is expected as a command line argument, and the user enters “12x3”

(an invalid value), is an error generated by the operating system (i.e., the

loader)?

1

6.5.2 Suggested Projects

Below are some suggested projects based on this chapter.

1

) Implement the example program to read and display the command line

arguments. Use the debugger to execute the program and display the final

results. Execute the program without the debugger and verify the appropriate

output is displayed to the console.

Page 252

Chapter 16.0 ◄ Command Line Arguments

2

3

) Convert the command line example into a function that will display each of the

command line arguments to the console. Use the debugger as necessary to

debug the program. Execute the program without the debugger and verify the

appropriate output is displayed to the console.

) Create an assembly language program to accept a file name on the command line

and open the file and display the one line message contained in the file. A series

of small text file should be created each containing one of the very important

messages at the start of each chapter of this text. The program should perform

error checking on the file name, and if valid, open the file. If the file opens

successfully, the message should be read from the file displayed to the console.

Appropriate error messages should be displayed if the file cannot be opened or

read. The program may assume that each message will be < 256 characters. Use

the debugger as necessary to debug the program. Execute the program without

the debugger and verify the appropriate output is displayed to the console.

4

) Create an assembly language program that will accept three unsigned integer

numbers from the command line, add the three numbers, and display each of the

three original numbers and the sum. If too many or too few command line

arguments are provided, an error message should be displayed. This program

will require that each of the ASCII strings be converted into integer.

Appropriate error checking should be included. For example, “123” is correct

while “12a3” is incorrect. The main program should call functions as necessary

for the ASCII to integer conversion and the output. Use the debugger as

necessary to debug the program. Execute the program without the debugger and

verify the appropriate output is displayed to the console.

Page 253

Chapter 16.0 ◄ Command Line Arguments

Page 254

Chapter

17

If I had it all to do over again, I'd spell creat

with an "e".

-

Kernighan

17.0 Input/Output Buffering

This chapter provides information regarding Input/Output (I/O) buffering. I/O buffering

is a process that improves the overall throughput and efficiency of I/O operations. The

general topic of I/O buffering is quite broad and can encompass many things including

hardware, software, and data networking. This text addresses single buffer buffering for

reading information from a file. While this is a very specific application, the general

concepts regarding how and why buffering is performed apply generally to many

different applications.

High-level languages provide library functions that perform the I/O buffering and hide

the associated complexity from the programmer. This is very desirable when

programming. When working at a low-level, the buffering will need to be implemented

explicitly (by us). The goal is to provide a detailed understanding of why and how

buffering is performed in order to provide a more in-depth understanding of how a

computer operates. This should help displace notions that such I/O is magic. And, of

course, help us appreciate the compiler I/O library functions.

1

7.1 Why Buffer?

In order to fully understand the importance of buffering, it is useful to review the

memory hierarchy as outlined in Chapter 2, Architecture. As noted, accesses to

secondary storage are significantly more expensive in terms of run-time than accesses to

main memory. This would strongly encourage us to limit the number of secondary

storage accesses by using some temporary storage, referred to as a buffer, in main

memory.

Such a buffer would also help reduce the overhead associated with system calls. For

example, a file read system call would involve pausing our program and turning over

control to the OS. The OS would validate our request (e.g., ensure file descriptor is

valid) and then pass the request to the secondary storage control via the system bus. The

Page 255

Chapter 17.0 ◄ Input/Output Buffering

controller would do whatever is necessary to obtain the requested data and place it

directly into main memory location as instructed by the OS, again accessing the system

bus to perform the transfer. Note, this is referred to as Direct Memory Access49 (DMA).

Once the secondary storage controller has completed the transfer, it notifies the OS. The

OS will then notify and resume our program. This process represents system overhead

since our program is just waiting for completion of the system service request. It would

make sense to obtain more data rather than less data on each system service call. It

should be fairly obvious that limiting the number of system service requests will help

the overall performance of our program.

Additionally, as described in Chapter 13, System Services, the low-level file read and

write operations require the number of characters to read. A typical operation desired

by a programmer would be to read a line as is provided by high-level language functions

such as the C++ getline() function. It is unknown ahead of time how many characters

might be on that line. This makes the low-level file read operation more difficult since

the exact number of characters to read is required.

We, as humans, place special meaning and significance to the LF character (end of line).

In the computer, the LF is just another ASCII character, no different than any other. We

see a file as a set of lines, but in the computer the file is just a collection of bytes, one

after another.

The process used for reading interactive input involved reading one character at a time.

This could be used to address the issue of not knowing ahead of time how many

characters are on a line. As was done for interactive input, the program could just read

one at a time until an LF is found and stop.

Such a process will be unduly slow for reading large volumes of information from a file.

Interactive input expects the user to type characters. The computer is easily able to keep

up with a human. Even the fastest typist will not be able to out-type the computer

(assuming efficiently written code). Further, interactive input is generally limited to

relatively small amounts. Input from a file is not awaiting user action and is generally

not limited to small amounts of data. For a sufficiently large file, single character I/O

would be quite slow. Note, testing and quantizing the performance difference is left as

an exercise.

To address this, the input data will be buffered. In this context, we will read a chunk of

the file, say 100,000 characters, into a large array which is referred to as a buffer or

input buffer. When a “line” of text is required, the line will be obtained from the input

buffer. The first time a line is requested, the file is read and the buffer is filled. After

4

9 For more information, refer to: http://en.wikipedia.org/wiki/Direct_memory_access

Page 256

Chapter 17.0 ◄ Input/Output Buffering

the file read completes, the line is returned from the buffer (all characters up to and

including the LF). Successive calls to get the next line are satisfied by obtaining the

characters from the buffer without needing to read the file again. This can keep

happening until all the characters in the buffer have been returned at which point the file

will need to be read again. When there are no more characters, the input is completed

and the program can be terminated.

This process helps improve performance, but is more complicated. High-level language

functions, such as getline, hide this complexity from the user. When working at the

low-level we will need to write the function to get the next line ourselves.

It is important to understand the cause of the performance degradation in order to fully

understand why buffering will improve performance.

1

7.2 Buffering Algorithm

The first step when developing an algorithm is to understand the problem. We will

assume the file is already open and available for reading. For our buffering problem, we

wish to provide a myGetLine() function to a calling routine.

The routine might be called as follows:

status = myGetLine(fileDescriptor, textLine, MAXLEN);

The file opening and associated error checking is required but would be addressed

separately and is outlined in Chapter 13, System Services. Once a file descriptor is

obtained, it can be made available to the myGetLine() function. It is shown here as an

explicit parameter for clarity.

As you may already be aware there is no special end of file code or character. We must

infer the end of file based on the number of characters actually read. The file read

system service will return the actual number of characters read. If the process requests

1

00,000 characters to be read and less than 100,000 are read, the end of file has been

reached and all characters have been read. Further attempts at reading the file will

generate an error. While it is easy to recognize the end of file and thus the last time the

file needs to be read, the remaining characters in the buffer need to be processed. So

while the program may know the end of file has been found, the program must continue

until the buffer is depleted.

The calling routine should not need to know any of the details regarding the buffering,

buffer management, or file operations. If a line of text from the file is available, the

myGetLine() function will return the line from the file and a TRUE status. The line is

returned via the passed argument. An argument for the maximum length of the text line

Page 257

Chapter 17.0 ◄ Input/Output Buffering

is also provided to ensure that the text line array is not overwritten. If a line is not

available, possibly due to all lines having been already returned or an unrecoverable

read error, the function should return FALSE. An unrecoverable read error might occur

if the storage medium goes off-line (drive removed), the file is deleted from another

console, or a hardware error occurs. Such errors will make continued reading

impossible. In such an event, an error message will be displayed and a FALSE returned

which will halt the program (with incomplete results).

In general, the function would read characters, up to and including the LF, from the

large buffer and place them in the line buffer, named textLine in the above example call.

This is easily done when there are characters available in the buffer. It will require an

extra step to ensure that the buffer has characters. The file must be read if the buffer is

empty, either due to the very first call or when all characters in the buffer have been

returned. It is possible to check if characters are available by checking the current

location of the next character to read in the buffer against the buffer size. The current

location cannot be allowed to exceed the buffer size (or we would be accessing a

location past the end of the buffer array). This is fairly straightforward when the buffer

is filled entirely. The buffer may be partially filled due to a file size smaller than the

buffer size or on the last file read after a series of file reads. To address this, the end of

the buffer, initially set to the buffer size, must be set based on the actual number of

characters read. There is a special case when the number of characters in the file is an

exact multiple of the buffer size. If this occurs, the returned number of characters read

is 0. An additional check is required to address this possibility.

Now that the problem and all the associated subtle issues are understood, we can take

the next step of developing an algorithm. In general this process is typically iterative.

That is, a first cut is developed, reviewed and refined. This would occur multiple times

until a comprehensive algorithm is obtained. While some try to rush through this step, it

is a mistake. Saving minutes on the algorithm development step means extra hours of

debugging.

Based on this earlier example, the passed arguments include;

; file descriptor → file descriptor for open file,

; as required for read system service

; text line → starting address of

; max length → maximum length of the array for

; the text line

Page 258

Chapter 17.0 ◄ Input/Output Buffering

Some variables are required and defined as follows:

;

;

;

;

;

;

;

BUFFER_SIZE → parameter for size of buffer

currIndex → index for current location in

the buffer, initially set to BUFFER_SIZE

buffMaximum → current maximum size of buffer,

initially set to BUFFER_SIZE

eofFlag → boolean to mark if the end of file

has been found, initially set to false

Based on an understanding of the problem, a number of different algorithmic

approaches are possible. Using the arguments and local variables, one such approach is

presented for reference.

;

;

;

;

;

;

;

;

;

;

;

;

;

;

;

;

;

;

myGetLine(fileDescriptor, textLine, maxLength) {

repeat {

if current index  buffer maximum

read buffer (buffer size)

if error

handle read error

display error message

exit routine (with false)

reset pointers

if chars read < characters request read

set eofFlag = TRUE

get one character from buffer at current index

place character in text line buffer

increment current index

if character is LF

exit with true

}

}

This algorithm outline does not verify that the text line buffer is not overwritten nor

does it handle the case when the file size is an exact multiple of the buffer size.

Refining, implementing, and testing the algorithm is left to the reader as an exercise.

As presented, this algorithm will require statically declared variables. Stack dynamic

variables cannot be used here since the information is required to be maintained between

successive calls.

The variables and algorithm are provided as program comments. The algorithm is the

most important part of the program documentation and will not only help when writing

Page 259

Chapter 17.0 ◄ Input/Output Buffering

the code but in the debugging process. Whatever time you think you are saving by

skipping the documentation, it will cost far, far more time when debugging.

1

7.3 Exercises

Below are some quiz questions and suggested projects based on this chapter.

1

7.3.1 Quiz Questions

Below are some quiz questions based on this chapter.

1

2

3

) What is the end of line character or characters for Linux and Windows?

) Based on the explanations in this chapter, what is I/O buffering?

) In reference to a high-level language, where are the I/O buffering routines

located?

4

5

6

) What is the advantage of hiding the I/O buffering complexity from the

programmer?

) What is the key advantage of performing I/O buffering (as opposed to reading

one character at a time)?

) Why is it difficult to use the file read system service to read one text line from a

file?

7

8

9

1

1

) In terms of the memory hierarchy, why is buffering advantageous?

) In terms of system overhead, why is buffering advantageous?

) Why does the file read buffering algorithm require statically declared variables?

0) How is the end of file recognized by the program?

1) Provide one, of many, reasons that a file read request might return an error even

when the file has been opened successfully.

1

2) What must be done to address the case when the file size is an exact multiple of

the buffer size?

1

1

3) Why is the maximum length of the text line passed as an argument?

4) How does the presented algorithm ensure that the very first call the myGetLine()

will read the buffer?

Page 260

Chapter 17.0 ◄ Input/Output Buffering

1

7.3.2 Suggested Projects

Below are some suggested projects based on this chapter.

1

) Refine the presented algorithm to address maximum text line check and

possibility that the file size is an even multiple of the buffer size. This includes

formalizing the variable names and looping constructs (many possible choices).

2

) Create a simple main program that will test the myGetLine() function by opening

a file, calling the myGetLine() function, and displaying the lines to the console.

Test the program with a series of different size files including ones smaller,

larger, and much larger than the selected buffer size. Capture the program

output and compare the captured output to the original file to ensure your

program is correct.

3

) Create a program that will read two file names from the command line, read

lines from the first file, add line numbers, and write the modified line (with the

line number) to the second file. For example, your add lines program might be

initiated with:

./addLine inFile.txt newFile.txt

where the inFile.txt exists and contains standard ASCII text. If the inFile.txt file

does not exist an error should be generated and the program terminated. The

program should open/create the file newFile.txt and write the lines, with the line

numbers, to the newFile.txt file. The output file should be created, deleting an

old version if one exists. Use a text editor to verify that the line numbers track

correctly in the output file.

Page 261

Chapter 17.0 ◄ Input/Output Buffering

Page 262

Chapter

18

To err is human; to make a real mess, you

need a computer.

18.0 Floating-Point Instructions

This chapter provides a basic overview for a subset of the x86-64 floating-point

instructions. Only the most basic instructions are covered.

The text focuses on the x86-64 floating-point operations, which are not the same as the

3

2-bit floating-point operations.

The instructions are presented in the following order:

•

•

•

•

Data Movement

Integer / Floating-point Conversion Instructions

Arithmetic Instructions

Floating-point Control Instructions

A complete listing of the instructions covered in this text is located in Appendix B for

reference.

It should be noted that the floating-point arithmetic operations do not require the use of

a specific register and do not change types (sizes). This makes the floating-point

instructions easier to use.

1

8.1 Floating-Point Values

Floating-point values are typically represented as either single precision (32-bits) or

double precision (64-bits). In C and C++ single precision floating-point variables are

typically declared as float type and double precision floating-point variables are

declared as double type. As noted in the following sections, assembly language

instructions will use an s (lower-case letter S) qualifier to refer to single precision and a

d (lower-case letter D) qualifier to refer to double precision.

Page 263

Chapter 18.0 ◄ Floating-Point Instructions

1

8.2 Floating-Point Registers

There are a set of dedicated registers, referred to as XMM registers, used to support

floating-point operations. Floating-point operations must use the floating-point

registers. The XMM registers are 128-bits and 256-bits on the later processors.

Initially, we will only use the lower 32 or 64-bits.

There are 16 XMM registers, named xmm0 through xmm15. Refer to Chapter 2 for an

explanation and summary of the CPU registers.

1

8.3 Data Movement

Typically, data must be moved into a CPU floating-point register in order to be operated

upon. Once the calculations are completed, the result may be copied from the register

and placed into a variable. There are a number of simple formulas in the example

program that perform these steps. This basic data movement operations are performed

with the move instruction.

The general form of the move instruction is:

movss <dest>, <src>

movsd <dest>, <src>

For the movssinstruction, a single 32-bit source operand is copied into the destination

operand. For the movsd instruction, a single 64-bit source operand is copied into the

destination operand. The value of the source operand is unchanged. The destination

and source operand must be of the correct size for the instruction (32 or 64-bits).

Neither operand can be an immediate value. Both operands, cannot be memory,

however one can be. If a memory to memory operation is required, two instructions

must be used.

The move instruction loads one value, using the lower 32 or 64-bits, into or out of the

register. Other move instructions are required to load multiple values.

The floating-point move instructions are summarized as follows:

Instruction

Explanation

movss   <dest>, <src>

Copy 32-bit source operand to the 32-bit

destination operand.

Note 1, both operands cannot be memory.

Note 2, operands cannot be an immediate.

Page 264

Chapter 18.0 ◄ Floating-Point Instructions

Instruction

Explanation

Examples:

movss   xmm0, dword [x]

movss   dword [fltSVar], xmm1

movss   xmm3, xmm2

movsd   <dest>, <src>

Copy 64-bit source operand to the 64-bit

destination operand.

Note 1, both operands cannot be memory.

Note 2, operands cannot be an immediate.

movsd xmm0, qword [y]

movsd qword [fltDVar], xmm1

movsd xmm3, xmm2

Examples:

A more complete list of the instructions is located in Appendix B.

For example, assuming the following data declarations:

fSVar1

fSVar2

fDVar1

fDVar2

dd

dd

dq

dq

3.14

0.0

6.28

0.0

To perform the basic operations of:

fSVar2 = fSVar2

fDVar2 = fDVar1

; single precision variables

; double precision variables

The following instructions could be used:

movss

movss

xmm0, dword [fSVar1]

dword [fSVar2], xmm0

; fSVar2 = fSVar1

; fDVar2 = fDVar1

movsd

movsd

xmm1, qword [fDVar1]

qword [fDVar2], xmm1

movss

movsd

xmm2, xmm0

xmm3, xmm1

; xmm2 = xmm0 (32-bit)

; xmm3 = xmm1 (64-bit)

For some instructions, including those above, the explicit type specification (e.g., byte,

word, dword, qword) can be omitted as the other operand will clearly define the size. It

is included for consistency and good programming practices.

Page 265

Chapter 18.0 ◄ Floating-Point Instructions

1

8.4 Integer / Floating-Point Conversion Instructions

If integer values are required during floating-point calculations, the integers must be

converted into floating-point values. If single precision and double precision floating-

point values are required for a series of calculations, they must be converted to single or

double so that the operations are performed on a consistent size/type.

Refer to Chapter 3 for a more detailed explanation of the representation details for

floating-point values. It is assumed the reader understands the representation details and

recognizes the requirement to ensure consistent formats before performing floating

operations.

This basic data conversion operations are performed with the convert instruction.

The floating-point conversion instructions are summarized as follows:

Instruction

Explanation

cvtss2sd   <RXdest>, <src> Convert 32-bit floating-point source operand to

the 64-bit floating-point destination operand.

Note 1, destination operand must be floating-

point register.

Note 2, source operand cannot be an

immediate.

Examples:

cvtss2sd   xmm0, dword [fltSVar]

cvtss2sd   xmm3, eax

cvtss2sd   xmm3, xmm2

cvtsd2ss   <RXdest>, <src> Convert 64-bit floating-point source operand to

the 32-bit floating-point destination operand.

Note 1, destination operand must be floating-

point register.

Note 2, source operand cannot be an

immediate.

Examples:

cvtsd2ss   xmm0, qword [fltDVar]

cvtsd2ss   xmm1, rax

cvtsd2ss   xmm3, xmm2

Page 266

Chapter 18.0 ◄ Floating-Point Instructions

Instruction

Explanation

cvtss2si   <reg>, <src>

Convert 32-bit floating-point source operand to

the 32-bit integer destination operand.

Note 1, destination operand must be register.

Note 2, source operand cannot be an

immediate.

Examples:

cvtss2si   xmm1, xmm0

cvtss2si   eax, xmm0

cvtss2si   eax, dword [fltSVar]

cvtsd2si   <reg>, <src>

Convert 64-bit floating-point source operand to

the 32-bit integer destination operand.

Note 1, destination operand must be register.

Note 2, source operand cannot be an

immediate.

Examples:

cvtsd2si   xmm1, xmm0

cvtsd2si   eax, xmm0

cvtsd2si   eax, qword [fltDVar]

cvtsi2ss   <RXdest>, <src> Convert 32-bit integer source operand to the

2-bit floating-point destination operand.

3

Note 1, destination operand must be floating-

point register.

Note 2, source operand cannot be an

immediate.

Examples:

cvtsi2ss   xmm0, eax

cvtsi2ss   xmm0, dword [fltDVar]

cvtsi2sd   <RXdest>, <src> Convert 32-bit integer source operand to the

4-bit floating-point destination operand.

6

Note 1, destination operand must be floating-

point register.

Note 2, source operand cannot be an

immediate.

Examples:

cvtsi2sd   xmm0, eax

cvtsi2sd   xmm0, dword [fltDVar]

A more complete list of the instructions is located in Appendix B.

Page 267

Chapter 18.0 ◄ Floating-Point Instructions

1

8.5 Floating-Point Arithmetic Instructions

The floating-point arithmetic instructions perform arithmetic operations such as add,

subtract, multiplication, and division on single or double precision floating-point values.

The following sections present the basic arithmetic operations.

1

8.5.1 Floating-Point Addition

The general form of the floating-point addition instructions are as follows:

addss    <RXdest>, <src>

addsd    <RXdest>, <src>

Where operation is as follows:

<RXdest> = <RXdest> + <src>

Specifically, the source and destination operands are added and the result is placed in

the destination operand (over-writing the previous value). The destination operand must

be a floating-point register. The source operand may not be an immediate value. The

value of the source operand is unchanged. The destination and source operand must be

of the same size (double-words or quadwords). If a memory to memory addition

operation is required, two instructions must be used.

For example, assuming the following data declarations:

fSNum1

fSNum2

fSAns

dd

dd

dd

43.75

15.5

0.0

fDNum3

fDNum4

fDAns

dq

dq

dq

200.12

73.2134

0.0

To perform the basic operations of:

fSAns = fSNum1 + fSNum2

fDAns = fDNum3 + fDNum4

The following instructions could be used:

; fSAns = fSNum1 + fSNum2

movss

addss

movss

xmm0, dword [fSNum1]

xmm0, dword [fSNum2]

dword [fSAns], xmm0

Page 268

Chapter 18.0 ◄ Floating-Point Instructions

; fDAns = fDNum3 + fDNum4

movsd

addsd

movsd

xmm0, qword [fDNum3]

xmm0, qword [fDNum4]

qword [fDAns], xmm0

For some instructions, including those above, the explicit type specification (e.g.,

dword, qword) can be omitted as the other operand or the instruction itself clearly

defines the size. It is included for consistency and good programming practices.

The floating-point addition instructions are summarized as follows:

Instruction

Explanation

addss   <RXdest>, <src>

Add two 32-bit floating-point operands,

(<RXdest> + <src>) and place the result in

<

RXdest> (over-writing previous value).

Note 1, destination operands must be a

floating-point register.

Note 2, source operand cannot be an

immediate.

Examples:

addss   xmm0, xmm3

addss   xmm5, dword [fSVar]

addsd   <RXdest>, <src>

Add two 64-bit floating-point operands,

(<RXdest> + <src>) and place the result in

<

RXdest> (over-writing previous value).

Note 1, destination operands must be a

floating-point register.

Note 2, source operand cannot be an

immediate.

Examples:

addsd   xmm0, xmm3

addsd   xmm5, qword [fDVar]

A more complete list of the instructions is located in Appendix B.

1

8.5.2 Floating-Point Subtraction

The general form of the floating-point subtraction instructions are as follows:

subss    <RXdest>, <src>

subsd    <RXdest>, <src>

Page 269

Chapter 18.0 ◄ Floating-Point Instructions

Where operation is as follows:

<RXdest> = <RXdest> - <src>

Specifically, the source and destination operands are subtracted and the result is placed

in the destination operand (over-writing the previous value). The destination operand

must be a floating-point register. The source operand may not be an immediate value.

The value of the source operand is unchanged. The destination and source operand must

be of the same size (double-words or quadwords). If a memory to memory addition

operation is required, two instructions must be used.

For example, assuming the following data declarations:

fSNum1

fSNum2

fSAns

dd

dd

dd

43.75

15.5

0.0

fDNum3

fDNum4

fDAns

dq

dq

dq

200.12

73.2134

0.0

To perform the basic operations of:

fSAns = fSNum1 - fSNum2

fDAns = fDNum3 - fDNum4

The following instructions could be used:

; fSAns = fSNum1 - fSNum2

movss

subss

movss

xmm0, dword [fSNum1]

xmm0, dword [fSNum2]

dword [fSAns], xmm0

;

fDAns = fDNum3 - fDNum4

movsd

subsd

movsd

xmm0, qword [fDNum1]

xmm0, qword [fDNum2]

qword [fDAns], xmm0

For some instructions, including those above, the explicit type specification (e.g.,

dword, qword) can be omitted as the other operand or the instruction itself clearly

defines the size. It is included for consistency and good programming practices.

Page 270

Chapter 18.0 ◄ Floating-Point Instructions

The floating-point subtraction instructions are summarized as follows:

Instruction

Explanation

subss   <RXdest>, <src>

Subtract two 32-bit floating-point operands,

(<RXdest> - <src>) and place the result in

<

RXdest> (over-writing previous value).

Note 1, destination operands must be a

floating-point register.

Note 2, source operand cannot be an

immediate.

Examples:

subss   xmm0, xmm3

subss   xmm5, dword [fSVar]

subsd   <RXdest>, <src>

Subtract two 64-bit floating-point operands,

(<RXdest> - <src>) and place the result in

<

RXdest> (over-writing previous value).

Note 1, destination operands must be a

floating-point register.

Note 2, source operand cannot be an

immediate.

Examples:

subsd   xmm0, xmm3

subsd   xmm5, qword [fDVar]

A more complete list of the instructions is located in Appendix B.

1

8.5.3 Floating-Point Multiplication

The general form of the floating-point multiplication instructions are as follows:

mulss    <RXdest>, <src>

mulsd    <RXdest>, <src>

Where operation is as follows:

<RXdest> = <RXdest> * <src>

Specifically, the source and destination operands are multiplied and the result is placed

in the destination operand (over-writing the previous value). The destination operand

must be a floating-point register. The source operand may not be an immediate value.

Page 271

Chapter 18.0 ◄ Floating-Point Instructions

The value of the source operand is unchanged. The destination and source operand must

be of the same size (double-words or quadwords). If a memory to memory addition

operation is required, two instructions must be used.

For example, assuming the following data declarations:

fSNum1

fSNum2

fSAns

dd

dd

dd

43.75

15.5

0.0

fDNum3

fDNum4

fDAns

dq

dq

dq

200.12

73.2134

0.0

To perform the basic operations of:

fSAns = fSNum1 * fSNum2

fDAns = fDNum3 * fDNum4

The following instructions could be used:

; fSAns = fSNum1 * fSNum2

movss

mulss

movss

xmm0, dword [fSNum1]

xmm0, dword [fSNum2]

dword [fSAns], xmm0

;

fDAns = fDNum3 * fDNum4

movsd

mulsd

movsd

xmm0, qword [fDNum3]

xmm0, qword [fDNum4]

qword [fDAns], xmm0

For some instructions, including those above, the explicit type specification (e.g.,

dword, qword) can be omitted as the other operand or the instruction itself clearly

defines the size. It is included for consistency and good programming practices.

Page 272

Chapter 18.0 ◄ Floating-Point Instructions

The floating-point multiplication instructions are summarized as follows:

Instruction

Explanation

mulss   <RXdest>, <src>

Multiply two 32-bit floating-point operands,

(<RXdest> * <src>) and place the result in

<

RXdest> (over-writing previous value).

Note 1, destination operands must be a

floating-point register.

Note 2, source operand cannot be an

immediate.

Examples:

mulss   xmm0, xmm3

mulss   xmm5, dword [fSVar]

mulsd   <RXdest>, <src>

Multiply two 64-bit floating-point operands,

(<RXdest> * <src>) and place the result in

<

RXdest> (over-writing previous value).

Note 1, destination operands must be a

floating-point register.

Note 2, source operand cannot be an

immediate.

Examples:

mulsd   xmm0, xmm3

mulsd   xmm5, qword [fDVar]

A more complete list of the instructions is located in Appendix B.

1

8.5.4 Floating-Point Division

The general form of the floating-point division instructions are as follows:

divss    <RXdest>, <src>

divsd    <RXdest>, <src>

Where operation is as follows:

<RXdest> = <RXdest> / <src>

Specifically, the source and destination operands are divided and the result is placed in

the destination operand (over-writing the previous value). The destination operand must

be a floating-point register. The source operand may not be an immediate value. The

value of the source operand is unchanged. The destination and source operand must be

Page 273

Chapter 18.0 ◄ Floating-Point Instructions

of the same size (double-words or quadwords). If a memory to memory addition

operation is required, two instructions must be used.

For example, assuming the following data declarations:

fSNum1

fSNum2

fSAns

dd

dd

dd

43.75

15.5

0.0

fDNum3

fDNum4

fDAns

dq

dq

dq

200.12

73.2134

0.0

To perform the basic operations of:

fSAns = fSNum1 / fSNum2

fDAns = fDNum3 / fDNum4

The following instructions could be used:

;

fSAns = fSNum1 / fSNum2

movss

divss

movss

xmm0, dword [fSNum1]

xmm0, dword [fSNum2]

dword [fSAns], xmm0

; fDAns = fDNum3 / fDNum4

movsd

divsd

movsd

xmm0, qword [fDNum3]

xmm0, qword [fDNum4]

qword [fDAns], xmm0

For some instructions, including those above, the explicit type specification (e.g.,

dword, qword) can be omitted as the other operand or the instruction itself clearly

defines the size. It is included for consistency and good programming practices.

The floating-point division instructions are summarized as follows:

Instruction

Explanation

divss   <RXdest>, <src>

Divide two 32-bit floating-point operands,

(<RXdest> / <src>) and place the result in

<

RXdest> (over-writing previous value).

Note 1, destination operands must be a

floating-point register.

Note 2, source operand cannot be an

immediate.

Page 274

Chapter 18.0 ◄ Floating-Point Instructions

Instruction

Explanation

Examples:

divss   xmm0, xmm3

divss   xmm5, dword [fSVar]

divsd   <RXdest>, <src>

Divide two 64-bit floating-point operands,

(<RXdest> / <src>) and place the result in

<

RXdest> (over-writing previous value).

Note 1, destination operands must be a

floating-point register.

Note 2, source operand cannot be an

immediate.

Examples:

divsd   xmm0, xmm3

divsd   xmm5, qword [fDVar]

A more complete list of the instructions is located in Appendix B.

1

8.5.5 Floating-Point Square Root

The general form of the floating-point square root instructions are as follows:

sqrtss    <RXdest>, <src>

sqrtsd    <RXdest>, <src>

Where operation is as follows:

<src>

<dest> =  √

Specifically, the square root of the source operand is placed in the destination operand

(over-writing the previous value). The destination operand must be a floating-point

register. The source operand may not be an immediate value. The value of the source

operand is unchanged. The destination and source operand must be of the same size

(double-words or quadwords). If a memory to memory addition operation is required,

two instructions must be used.

For example, assuming the following data declarations:

fSNum1

fSAns

dd

dd

1213.0

0.0

fDNum3

fDAns

dq

dq

172935.123

0.0

Page 275

Chapter 18.0 ◄ Floating-Point Instructions

To perform the basic operations of:

fSNum1

fSNum3

fSAns =  √

fDAns =  √

The following instructions could be used:

;

fSAns = sqrt (fSNum1)

sqrtss

movss

xmm0, dword [fSNum1]

dword [fSAns], xmm0

;

fDAns = sqrt(fDNum3)

sqrtsd

movsd

xmm0, qword [fDNum3]

qword [fDAns], xmm0

For some instructions, including those above, the explicit type specification (e.g.,

dword, qword) can be omitted as the other operand or the instruction itself clearly

defines the size. It is included for consistency and good programming practices.

The floating-point addition instructions are summarized as follows:

Instruction

Explanation

sqrtss   <RXdest>, <src>

Take the square root of the 32-bit floating-

point source operand and place the result in

destination operand (over-writing previous

value).

Note 1, destination operands must be a

floating-point register.

Note 2, source operand cannot be an

immediate.

Examples:

sqrtss   xmm0, xmm3

sqrtss   xmm7, dword [fSVar]

Page 276

Chapter 18.0 ◄ Floating-Point Instructions

Instruction

Explanation

sqrtsd   <RXdest>, <src>

Take the square root of the 64-bit floating-

point source operand and place the result in

destination operand (over-writing previous

value).

Note 1, destination operands must be a

floating-point register.

Note 2, source operand cannot be an

immediate.

Examples:

sqrtsd   xmm0, xmm3

sqrtsd   xmm7, qword [fDVar]

A more complete list of the instructions is located in Appendix B.

1

8.6 Floating-Point Control Instructions

The control instructions refer to programming constructs such as IF statements and

looping. The integer comparison instruction, cmp, as described in Chapter 7 will not

work for floating-point values.

The floating-point comparison instructions compare two floating-point values. Like the

integer comparisons, the result of the comparison is stored in the rFlag register and

neither operand is changed. Immediately after the comparison, the rFlag register is

accessed to determine the results by using a conditional jump instruction. While all

floating-point comparisons are signed, an unsigned conditional jump is used

(ja/jae/jb/jbe). Program labels (i.e., a target of a conditional jump) are the same.

There are two forms of the floating-point comparison, ordered and unordered. The

ordered floating-point comparisons can cause a number of exceptions. The unordered

floating-point comparisons can only cause an exception for a S-NaN (signaling not a

number), more generically referred to as a NaN (not a number) as described in Chapter

3

, Data Representation.

The GNU C/C++ compiler favors the unordered floating-point compare instruction.

Since they are similar, this text will focus only on the unordered version.

1

8.6.1 Floating-Point Comparison

The general form of the floating-point comparison instructions are as follows:

Page 277

Chapter 18.0 ◄ Floating-Point Instructions

ucomiss    <RXsrc>, <src>

ucomisd    <RXsrc>, <src>

Where <RXsrc> and <src> are compared as floating-point values and must be the same

size. The results of the comparison are placed in the rFlag register. Neither operand is

changed. The <RXsrc> operand must be one of the xmm registers. The <src> register

can be a xmm register or a memory location, but may not be an immediate value. One

of the unsigned conditional jump instructions can be used to read the rFlag register.

The conditional control instructions include the jump equal (je) and jump not equal

(jne). The unsigned conditional control instructions include the basic set of comparison

operations; jump below than (jb), jump below or equal (jbe), jump above than (ja), and

jump above or equal (jae).

The general form of the signed conditional instructions along with an explanatory

comment are as follows:

je    <label>

jne   <label>

; if <op1> == <op2>

; if <op1> != <op2>

jb    <label>

jbe   <label>

ja    <label>

jae   <label>

; unsigned, if <op1> < <op2>

; unsigned, if <op1> <= <op2>

; unsigned, if <op1> > <op2>

; unsigned, if <op1> >= <op2>

For example, given the following pseudo-code for floating-point variables:

if (fltNum > fltMax)

fltMax = fltNum;

And, given the following data declarations:

fltNum

fltMax

dq

dq

7.5

5.25

Then, assuming that the values are updating appropriately within the program (not

shown), the following instructions could be used:

movsd

xmm1, qword [fltNum]

ucomisd xmm1, qword [fltMax]

; if fltNum <= fltMax

;    skip set new max

jbe

notNewFltMax

movsd

notNewFltMax:

qword [fltMax], xmm1

Page 278

Chapter 18.0 ◄ Floating-Point Instructions

As with integer comparisons, the floating-point compare and conditional jump provide

functionality for jump or not jump. As such, if the condition from the original IF

statement is false, the code to update the fltMax should not be executed. Thus, when

false, in order to skip the execution, the conditional jump will jump to the target label

immediately following the code to be skipped (not executed). While there is only one

line in this example, there can be many lines of code.

A more complex example might be as follows:

if (x != 0.0) {

ans = x / y;

errFlg = FALSE;

} else {

errFlg = TRUE;

}

This basic compare and conditional jump does not provide and IF-ELSE structure. It

must be created. Assuming the x and y variables are signed double-words that will be

set during the program execution, and the following declarations:

TRUE

FALSE

equ

equ

1

0

fltZero dq

0.0

10.1

3.7

0.0

FALSE

x

dq

dq

dq

db

y

ans

errFlg

The following code could be used to implement the above IF-ELSE statement.

movsd

xmm1, qword [x]

ucomisd xmm1, qword [fltZero]

; if statement

je

doElse

divsd

movsd

mov

xmm1, qword [y]

dword [ans], eax

byte [errFlg], FALSE

skpElse

jmp

doElse:

mov

byte [errFlg], TRUE

skpElse:

Page 279

Chapter 18.0 ◄ Floating-Point Instructions

Floating-point comparisons can be very tricky due to the inexact nature of the floating-

point representations and rounding errors. For example, the value 0.1 added 10 times

should be 1.0. However, implementing a program to perform this summation and

checking the result, will show that;

1

0

∑

i =1

0.1 ≠ 1.0

which can be very confusing for inexperienced programmers.

For more information regarding the details of floating-point, refer to the popular article

What Every Computer Scientist Should Know About Floating-Point Arithmetic50

.

The floating-point comparison instructions are summarized as follows:

Instruction

Explanation

ucomiss   <RXsrc>, <src>

Compare two 32-bit floating-point operands,

(<RXsrc> + <src>). Results are placed in the

rFlag register. Neither operand is changed.

Note 1, <RXsrc> operands must be a floating-

point register.

Note 2, source operand may be a floating-point

register or memory, but not be an immediate.

Examples:

ucomiss   xmm0, xmm3

ucomiss   xmm5, dword [fSVar]

ucomisd   <RXsrc>, <src>

Compare two 64-bit floating-point operands,

(<RXsrc> + <src>). Results are placed in the

rFlag register. Neither operand is changed.

Note 1, <RXsrc> operands must be a floating-

point register.

Note 2, source operand may be a floating-point

register or memory, but not be an immediate.

Examples:

ucomisd   xmm0, xmm3

ucomisd   xmm5, qword [fSVar]

A more complete list of the instructions is located in Appendix B.

5

0 See: http://docs.oracle.com/cd/E19957-01/806-3568/ncg_goldberg.html

Page 280

Chapter 18.0 ◄ Floating-Point Instructions

1

8.7 Floating-Point Calling Conventions

The standard calling conventions detailed in Chapter 12, Functions still fully apply.

This section addresses the usage of the floating-point registers when calling floating-

point functions.

When using floating-point registers, none of the registers are preserved across a

floating-point function call.

The first eight (8) floating-point arguments are passed in floating-point registers xmm0

–

xmm7. Any additional arguments are placed on the stack in backwards order in the

manner described in Chapter 12, Functions. A value returning floating-point function

will return the result in xmm0.

Since none of the floating-point registers are preserved, the code must be written

carefully.

1

8.8 Example Program, Sum and Average

This example is a simple assembly language program to calculate the sum and average

for a list of floating-point values.

;

Floating-Point Example Program

; ***********************************************************

section .data

; -----

;  Define constants.

NULL

TRUE

FALSE

equ 0

equ 1

equ 0

; end of string

EXIT_SUCCESS equ 0

SYS_exit equ 60

; Successful operation

; system call code for terminate

; -----

fltLst

dq 21.34,  6.15,  9.12, 10.05,  7.75

dq  1.44, 14.50,  3.32, 75.71, 11.87

dq 17.23, 18.25, 13.65, 24.24,  8.88

dd 15

length

Page 281

Chapter 18.0 ◄ Floating-Point Instructions

lstSum

lstAve

dq 0.0

dq 0.0

; ***********************************************************

section .text

global _start

_start:

;

-----

;  Loop to find floating-point sum.

mov ecx, [length]

mov rbx, fltLst

mov rsi, 0

movsd xmm1, qword [lstSum]

sumLp:

movsd

xmm0, qword [rbx+rsi*8]

xmm1, xmm0

rsi

; get fltLst[i]

; update sum

; i++

addsd

inc

loop

sumLp

movsd

qword [lstSum], xmm1

; save sum

;

-----

;  Compute average of entire list.

cvtsi2sd xmm0, dword [length]

cvtsd2si dword [length], xmm0

divsd

movsd

xmm1, xmm0

qword [lstAve], xmm1

;

;

-----

Done, terminate program.

last:

mov

rax, SYS_exit

rbx, EXIT_SUCCESS

mov

; exit w/success

syscall

The debugger can be used to examine the results and verify correct execution of the

program.

Page 282

Chapter 18.0 ◄ Floating-Point Instructions

1

8.9 Example Program, Absolute Value

This example is a simple assembly language program to find the absolute value of a

floating-point value in order to demonstrate floating-point comparisons. Recall that if a

value is negative, it must be made positive and if the value is already positive, nothing

should be done.

; Floating-Point Absolute Value Example

section .data

;

-----

;  Define constants.

TRUE

FALSE

equ

equ

1

0

SUCCESS

SYS_exit

equ

equ

0

60

; successful operation

; call code for terminate

;

;

-----

Define some test variables.

dZero

dNegOne

dq

dq

0.0

-1.0

fltVal

dq

-8.25

; *********************************************************

section .text

global _start

_start:

;

-----

;  Perform absolute value function on flt1

movsd

xmm0, qword [fltVal]

ucomisd xmm0, qword [dZero]

jae

isPos

mulsd

movsd

xmm0, qword [dNegOne]

qword [fltVal], xmm0

isPos:

Page 283

Chapter 18.0 ◄ Floating-Point Instructions

;

;

-----

Done, terminate program.

last:

mov

rax, SYS_exit

rbx, EXIT_SUCCESS

mov

; exit w/success

syscall

In this example, the final result for |fltVal| was saved to memory. Depending on the

context, this may not be required.

1

8.10 Exercises

Below are some quiz questions and suggested projects based on this chapter.

1

8.10.1 Quiz Questions

Below are some quiz questions based on this chapter.

1

2

) List the floating-point registers.

) How many bytes are used by single precision floating-point values and how

many bytes are used for double precision floating-point values?

3

4

) Explain why 0.1 added 10 times does not equal 1.0.

) Where is the result of a value returning floating-point function such as sin(x)

returned?

5

) For a value returning floating-point function, which floating-point registers must

be preserved across the function call?

1

8.10.2 Suggested Projects

Below are some suggested projects based on this chapter.

1

) Implement the example program to find the sum and average for a list of

floating-point values. Use the debugger to execute the program and verify the

final results.

2

) Implement the floating-point absolute value function as two macros, one fAbsf

for 32-bit floating-point values and fAbsd for 64-bit floating-point values.

Create a simple main program that uses each macro three times on various

different values. Use the debugger to execute the program and verify the results.

Page 284

Chapter 18.0 ◄ Floating-Point Instructions

) Implement a program to perform the summation:

3

1

0

∑

i =1

0.1

Compare the results of the summation to the value 1.0 and display a message

Are Same” if the summation result equals 1.0 and the message “Are Not Same”

“

if the result of the summation does not equal 1.0. Use the debugger as needed to

debug the program. When working, execute the program without the debugger

and verify that the expected results are displayed to the console.

Page 285

Chapter 18.0 ◄ Floating-Point Instructions

Page 286

Chapter

19

#

define QUESTION ((bb) || !(bb))

Shakespeare

-

19.0 Parallel Processing

In the context of computing, parallel processing51, or more generically concurrency52

refers to multiple processes appearing to execute simultaneously.

,

In broad terms, concurrency implies multiple different (not necessarily related)

processes simultaneously making progress. This can be accomplished in multiple ways.

For example, process A could be executing on core 0 while process B is simultaneously

executing on core 1 thus executing in parallel. Another possibility is that the execution

of process A and B could be interleaved on a single core where process A might execute

for a period of time and then be paused while process B is then executed for a period of

time and then paused while process A is resumed. This continues until either or both are

completed. If this interleaved execution is performed often and fast enough it will

appear that each is executing simultaneously.

The term parallel processing implies that processes are executing simultaneously. These

processes may be unrelated or working together as a coordinated unit to solve a single

complex problem.

This chapter provides an overview of some basic approaches to parallel processing as

applied to a single problem. Assume that a large problem can be divided into various

sub-problems and those sub-problems can be executed independently. The sub-

solutions would be brought together to provide a final solution for the problem. If the

sub-problems can be executed simultaneously, a final solution might be found

significantly faster than if the sub-problems are executed in series (one after another).

The potential rewards for implementing concurrency are significant but so are the

challenges. Unfortunately, not all problems can be easily divided into such sub-

problems.

5

5

1 For more information, refer to: http://en.wikipedia.org/wiki/Parallel_processing

2 For more information, refer to: http://en.wikipedia.org/wiki/Concurrency_(computer_science)

Page 287

Chapter 19.0 ◄ Parallel Processing

The basic approaches to parallel processing are distributed computing53 and

multiprocessing54 (also referred to as threaded computations). Each approach is

explained with an emphasis on the technical issues associated with shared memory

multiprocessing. The larger topic of learning to create parallel algorithms is outside the

scope of this text.

1

9.1 Distributed Computing

Distributed computing or distributed processing refers to the general idea of taking a

large problem, dividing it into multiple sub-problems, and executing the sub-problems

on different computers connected through a network. Typically, a master or main server

node will distribute the sub-problems to various available computation nodes. When

completed, the computational nodes will send the intermediate results back to the

master. As needed, the master may send additional sub-problems to the computational

nodes. The master will also track and combine the intermediate results into a final

solution. The details of how this is actually performed vary significantly and are

directly related to the specifics of the problem.

There are many large scale examples of distributed computing projects55. One such

project is Folding@home56 which is a large distributed computing project for disease

research that simulates protein folding, computational drug design, and other types of

molecular dynamics. The project uses the idle processing resources of well over

1

00,000 different personal computers owned by volunteers who have installed the

software on their systems.

This approach has the advantage of scaling to very large number of distributed

computers. The disadvantages are associated with the communication limitations

associated with the network.

1

9.2 Multiprocessing

As noted in Chapter 2, Architecture Overview, most current CPU chips include multiple

cores. The CPU cores all have equal access to the main memory resource.

Multiprocessing is a form of parallel processing that specifically refers to using multiple

cores to perform simultaneous execution of multiple processes.

5

5

5

5

3 For more information, refer to: http://en.wikipedia.org/wiki/Distributed_computing

4 For more information, refer to: http://en.wikipedia.org/wiki/Multiprocessing

5 For more information, refer to: http://en.wikipedia.org/wiki/List_of_distributed_computing_projects

6 For more information, refer to: http://en.wikipedia.org/wiki/Folding@home

Page 288

Chapter 19.0 ◄ Parallel Processing

Focusing on a single large project, a main or initial process might generate sub-

processes, referred to as threads57. When the initial process is started and placed in

memory by the system loader58, the Operating System creates a new process including

the required Operating System data structures, memory allocations, and page/swap file

allotments. A thread is often referred to as a light-weight process since it will use the

initial process’s allocations and address space thus making it quicker to create. It will

also be quicker to terminate as the deallocations are not performed. Since the threads

share memory with the initial process and any other threads, this presents the potential

for very fast communication between simultaneously executing threads. It also has the

added requirement that the communications in the form of memory writes be carefully

coordinated to ensure results are not corrupted. Such a problem is referred to as a race

condition59. Race conditions are addressed in the following section.

The threaded approach will not scale to the extent of the distributed approach. CPU

chips have a limited number of cores which places an upper bound on the number of

simultaneous computations that can occur. This limit does not apply to distributed

computing. The limitations regarding network communication speeds do not apply to

threaded computations.

1

9.2.1 POSIX Threads

POSIX Threads60, commonly called pThreads, is a widely available thread library on

Ubuntu and many other operating systems. The example in this section will use the

pThreads thread library Application Programmer Interface (API) for creating and

joining threads.

The initial or main process is created during the load process. The main process may

create additional threads with the pthread_create() library function. While there is

no specific limit to the number of threads that can be created, there is a limit to the

number of cores available thus creating a practical limit for the number of threads.

The initial or main process can see if the thread has completed with the

pthread_join() library function. If the thread has not completed, the join function

will wait until it does. Ideally, in order to maximize the overall parallel operations, the

main process would perform other computations while the thread or threads are

executing and only check for thread completion when the other work has been

completed.

5

5

5

6

7 For more information, refer to: http://en.wikipedia.org/wiki/Thread_(computing)

8 For more information, refer to Chapter 5, Tool Chain

9 For more information, refer to: http://en.wikipedia.org/wiki/Race_condition

0 For more information, refer to: http://en.wikipedia.org/wiki/POSIX_Threads

Page 289

Chapter 19.0 ◄ Parallel Processing

There are other pThread functions to address mutexes61, condition variables, and

synchronization62 between threads which are not addressed in this very brief overview.

It should be noted that there are other approaches to threading not addressed here.

1

9.2.2 Race Conditions

A race condition is a generic term referring to when multiple threads simultaneously

write to the same location at the same time. Threaded programming is typically done in

a high-level language. However, fully understanding the specific cause of a race

condition is best done at the assembly language level.

A simple example is provided to purposely create a race condition and examine the

problem in detail. This example is not computationally useful.

Assuming we wish to compute the following formula MAX times, where MAX is a

defined constant. For example;

myValue

myValue =

+ Y

( )

X

We could write a high-level language program something along the lines of:

for (int i=0; i < MAX; i++)

myValue = (myValue / X) + Y;

If we wish to speed-up this computation, perhaps because MAX is extremely large, we

might create two thread functions, each to perform MAX/2 computations. Each of the

thread functions would have shared access to the variables myValue, X, and Y. Thus,

the code might be;

for (int i=0; i < MAX/2; i++)

myValue = (myValue / X) + Y;

This code would be repeated in each of the thread functions. It may not be obvious, but

assuming both threads are simultaneously executing, this will cause a race condition on

the myValue variable. Specifically, each of the two threads are attempting to update the

variable simultaneously and some of the updates to the variable may be lost.

6

6

1 For more information, refer to: http://en.wikipedia.org/wiki/Mutual_exclusion

2 For more information, refer to: http://en.wikipedia.org/wiki/Synchronization_(computer_science)

Page 290

Chapter 19.0 ◄ Parallel Processing

To further simplify this example, we will assume that X and Y are both set to 1. As

such, the result of each calculation would be to increment myValue by 1. If myValue is

initialized to 0, repeating the calculation MAX times, should result in myValue being set

to MAX. This simplification will allow easy verification of the results. It is not

computationally useful in any meaningful way.

Implementing this in assembly would first require a main program that creates each of

the two threads. Then, each of the thread functions would need to be created. In this

very simple example, each thread function would be the same (performing MAX/2

iterations and that MAX is an even number).

Assuming one of the thread functions is named threadFunction0() and given the below

pThread thread data structure;

pthreadID0

dd

0, 0, 0, 0, 0

The following code fragment would create and start threadFunction0() executing.

;

;

pthread_create(&pthreadID0, NULL,

&threadFunction0, NULL);

rdi, pthreadID0

mov

mov

mov

mov

rsi, NULL

rdx, threadFunction0

rcx, NULL

call pthread_create

This would need to be performed for each thread function.

The following code fragment will check if a thread function is completed;

;

pthread_join (pthreadID0, NULL);

mov

mov

rdi, qword [pthreadID0]

rsi, NULL

call pthread_join

If the thread function is not done, the join call will wait until it is completed.

The thread function, threadFunction0(), itself might contain the following code;

;

-----

global threadFunction0

threadFunction0:

;  Perform MAX / 2 iterations to update myValue.

Page 291

Chapter 19.0 ◄ Parallel Processing

mov

shr

mov

mov

rcx, MAX

rcx, 1

r10, qword [x]

r11, qword [y]

; divide by 2

incLoop0:

; myValue = (myValue / x) + y

mov

cqo

div

add

mov

rax, qword [myValue]

r10

rax, r11

qword [myValue], rax

loop incLoop0

ret

The code for the second thread function would be similar.

If both threads are simultaneously executing, they are both trying to update the myValue

variable. For example, assuming that thread function 0 is executing on core 0 and

thread function 1 is executing on core 1 (arbitrarily chosen), the following execution

trace is possible;

Step Code: Core 0, Thread 0

Code: Core 1, Thread 1

1

2

3

4

5

6

mov  rax, qword [myValue]

cqo

mov  rax, qword [myValue]

cqo

div  qword [x]

add  rax, qword [y]

div  qword [x]

mov  qword [myValue], rax add  rax, qword [y]

mov  qword [myValue], rax

As a reminder, each core has its own set of registers. Thus, the core 0 rax register is

different than the core 1 rax register.

If the variable myValue is currently at 730, two thread executions should increase it to

7

32. On core 0 code, at step 1 the 730 is copied into core 0, rax. On core 1, at step 2,

the 730 is copied into core 1, rax. As execution progresses, steps 2-4 are performed and

core 0 rax is incremented from 730 to 731. During this time, on core 1, steps 1-3 are

Page 292

Chapter 19.0 ◄ Parallel Processing

completed and the 730 is also incremented to 731. As the next step, step 5, is executed

on core 0, the 731 is written to the myValue variable. As the next step, step 6 is

executed on core 1, the value 731 is again written to the myValue variable. Two

execution traces should have incremented the variable twice. However, since the value

was obtained in core 1 before core 0 was able to write the final value, one of the

increments was lost or duplicated. The end result of this overlap is that the final value

of myValue will not be correct. And, since the amount of overlapping execution is not

predictable, the degree of incorrectness is not easily predicted and may vary between

different executions.

For example, if the value of MAX is fairly small, such as 10,000, there would not likely

be any overlap. Each thread would start and complete so quickly, that the chances of

overlapping execution would be very small. The problem still exists but would likely

appear mostly correct on such executions, making it easy to ignore occasional

anomalous output. As MAX is increased, the odds of overlapping execution increase.

Such problems can be very challenging to debug and require an understanding of the

low-level operations and the technical architecture.

1

9.3 Exercises

Below are some quiz questions and suggested projects based on this chapter.

1

9.3.1 Quiz Questions

Below are some quiz questions based on this chapter.

1

2

3

4

) Explain the difference between concurrency and parallel processing.

) Name the two common approaches to parallel computations.

) In distributed processing, where might the parallel computations take place?

) Provide the names of two examples of large distributed computing projects.

Include a one-sentence description of each.

5

6

) In multiprocessing, where might the parallel computations take place?

) Provide one advantage and one disadvantage of the distributed computing

approach to parallel processing.

7

8

) Provide one advantage and one disadvantage of the multiprocessing approach to

parallel processing.

) Explain what a race condition is.

Page 293

Chapter 19.0 ◄ Parallel Processing

9

1

) Will a race condition occur when a shared variable is read by multiple

simultaneously executing threads? Explain why or why not.

0) Will a race condition occur when a shared variable is written by multiple

simultaneously executing threads (without any coordination)? Explain why or

why not.

1

9.3.2 Suggested Projects

Below are some suggested projects based on this chapter.

1

) Implement the outlined example program and create two thread functions where

each thread function computes the formula MAX/2 times. Set MAX to

1

,000,000,000 (one billion).

1

. Initially, structure the main function to call the first thread function and wait

until it completes until creating the second thread function and waiting until

it completes. This will force the threads to execute sequentially (not in

parallel). Include a function to convert the integer into a string and display

the result to the console. Use the debugger as necessary to debug the

program. When working, execute the program without the debugger and

verify that the displayed results are the same as MAX.

2

3

. Use the Unix time63 command to establish a base execute time. Record the

total elapsed time.

. Restructure the program so that both threads are created and then waiting for

both to complete. This will allow the execution of the threads to occur in

parallel. Use the debugger as necessary to debug the program. When

working, execute the program without the debugger and note the final value

of MAX. Ensure a full understanding of why the displayed value for MAX

is incorrect. Additionally, use the Unix time command on the modified

program to verify that it uses less elapsed time.

2

) Update the program from the previous question to resolve the race condition.

Use the debugger as necessary to debug the program. One very simple way to

accomplish this is to use temporary variables for each thread and combine them

after both thread functions have completed and display the final combined result.

When working, execute the program without the debugger and verify that the

displayed results for results is the same as MAX.

6

3 For more information, refer to: http://en.wikipedia.org/wiki/Time_(Unix)

Page 294

Chapter

20

If a program is useful, it must be changed.

If a program is useless, it must be

documented.

20.0 Interrupts

In a general sense, an interrupt64 is a pause or hold in the current flow. For example, if

you are talking on the phone and the doorbell rings, the phone conversation is placed on

hold, and the door answered. After the salesperson is sent away, the phone conversation

is resumed (where the conversation left off).

In computer programming, an interrupt is also a pause, or hold, of the currently

executing process. Typically, the current process is interrupted so that some other work

can be performed. An interrupt is usually defined as an event that alters the sequence of

instructions executed by a processor. Such events correspond to signals generated by

software and/or hardware. For example, most Input/Output (I/O) devices generate an

interrupt in order to transmit or receive data. Software programs can also generate

interrupts to initiate I/O as needed, request OS services, or handle unexpected

conditions.

Handling interrupts is a sensitive task. Interrupts can occur at any time; the kernel tries

to get the interrupt addressed as soon as possible. Additionally, an interrupt can be

interrupted by another interrupt.

2

0.1 Multi-user Operating System

A modern multi-user Operating System (OS) supports multiple programs executing, or

appearing to be executing, simultaneously by sharing resources as necessary. The OS is

responsible for managing and sharing the resources. These resources include the CPU

cores, primary memory (i.e., RAM), secondary storage (i.e., disk or SSD), display

screen, keyboard, and mouse. For example, multiple programs must share the available

CPU resources (core or cores as applicable).

The interrupt mechanism is the primary means that the OS uses in order to provide the

resource sharing. Consequently, understanding how interrupts are processed by the

6

4 For more information, refer to: http://en.wikipedia.org/wiki/Interrupt

Page 295

Chapter 20.0 ◄ Interrupts

computer provides insight into how the operating system is able to provide multi-

processing functions. When an interrupt occurs, the current process is interrupted (i.e.,

placed on hold), the interrupt is handled (which depends on the specific reason for the

interrupt), and then eventually the process is resumed. The OS may choose to perform

other tasks or processes before the original process is resumed. The interrupt is handled

by a special software routine called an Interrupt Service Routine (also called Interrupt

Handler, Device Driver, etc.). By using interrupts and quickly switching between

various processes, the OS is able to provide the illusion that all processes are executing

simultaneously.

Not all code can be interrupted. For example, due to the nature of some operating

system kernel functions, there are regions in the kernel which must not be interrupted at

all. This includes updating some sensitive operating system data structures.

2

0.1.1 Interrupt Classification

To better understand interrupts and interrupt processing, some background on the timing

and categories of interrupts is useful.

2

0.1.2 Interrupt Timing

The timing of interrupts may occur synchronously or asynchronously. These terms are

fairly common terms in computer processing and are explained in the following

sections.

20.1.2.1 Asynchronous Interrupts

In the context of computer interrupts, an asynchronously occurring interrupt means that

the interrupt may occur at an arbitrary time with respect to program execution.

Asynchronous interrupts are unpredictable relative to any specific location within the

executing process. For example, an external hardware device might interrupt the

currently executing process at an unpredictable location.

20.1.2.2 Synchronous Interrupts

Synchronously occurring interrupts typically occur while under CPU control and are

caused by or on behalf of the currently executing process. The synchronous nature is

related to where the interrupt occurs and not a specific clock time or CPU cycle time.

Synchronous interrupts typically reoccur at the same location (assuming nothing has

changed to resolve the original cause).

Page 296

Chapter 20.0 ◄ Interrupts

2

0.1.3 Interrupt Categories

Interrupts are typically categorized as hardware or software.

20.1.3.1 Hardware Interrupt

Hardware interrupts are typically generated by hardware. Hardware interrupts can be

issued by

•

•

•

I/O devices (keyboard, network adapter, etc.)

Interval timers

Other CPUs (on multiprocessor systems)

Hardware interrupts are asynchronously occurring. An example of a hardware interrupt

is when a key is typed on the keyboard. The OS cannot know ahead of time when, or

even if, the key will be pressed. To handle this situation, the keyboard support hardware

will generate an interrupt. If the OS is executing an unrelated program, that program is

temporarily interrupted while the key is processed. In this example, that specific

processing consists of storing the key in a buffer and returning to the interrupted

process. Ideally, this brief interruption will have little impact in the interrupted process.

20.1.3.1.1 Exceptions

An exception is a term for an interrupt that is caused by the current process and needs

the attention of the kernel. Exceptions are synchronously occurring. In this context,

synchronous implies that the exception will occur in a predictable or repeatable manner.

Exceptions are typically divided into categories as follows:

•

•

•

Faults

Traps

Abort

An example of a fault is a page fault which is a request for loading part of the program

from disk storage into memory. The interrupted process restarts with no loss of

continuity.

A trap is typically used for debugging. The process restarts with no loss of continuity.

An abort is typically an indication that a serious error condition occurred and must be

handled. This includes division by zero, attempt to access an invalid memory address,

or attempt to execute an invalid/illegal instruction. An illegal instruction might be an

instruction that is only allowed to be executed by privileged/authorized processes. An

Page 297

Chapter 20.0 ◄ Interrupts

invalid instruction might be caused by attempting to execute a data item (which will not

make sense). Based on the severity of the error condition, the process is often

terminated. If the process is not terminated, another routine may be executed to attempt

to resolve the problem and re-execute the original routine (but not necessarily from the

interrupted location). The C/C++/Java try/catch block is an example of this.

It must be noted that there is not an absolute agreed upon definition for these terms.

Some texts use slightly different terminology.

20.1.3.2 Software Interrupts

A software interrupt is produced by the CPU while processing instructions. This is

typically a programmed exception explicitly requested by the programmer. Such

interrupts are typically synchronously occurring and often used to request system

services from the OS. For example, requesting system services such as I/O.

2

0.2 Interrupt Types and Levels

Interrupts have various types and privileges associated with them. The following

sections provide an explanation of the types and privileges. Interrupted processes may

execute at a lower privilege than the interrupt processing code. In order for interrupts to

be effective, the OS must securely handle this privilege escalation and deescalation

securely and quickly.

2

0.2.1 Interrupt Types

The two different types or kinds of interrupts are:

•

•

Maskable interrupts

Non-maskable interrupts

Maskable interrupts are typically issued by I/O devices. As the name 'maskable'

implies, maskable interrupts can be ignored, or masked, for a short time period. This

allows the associated interrupt processing to be delayed.

Non-maskable interrupts (NMI's) must be handled immediately. This includes some OS

functions and critical malfunctions such as hardware failures. Non-maskable interrupts

are always processed by the CPU.

Page 298

Chapter 20.0 ◄ Interrupts

2

0.2.2 Privilege Levels

Privilege Levels refer to the privilege level at which the interrupt code executes. This

may be a higher privilege level than the interrupted code is executing. The processor

executes code in one of four privilege levels as follows:

Level

Description

Level 0

Full access to all hardware resources (no restrictions).

Used by only the lowest level OS functions.

Level 1

Level 2

Level 3

Somewhat restricted access to hardware resources. Used

by library routines and software that interacts with

hardware.

More restricted access to hardware resources. Used by

library routines and software that has limited access to

some hardware.

No direct access to hardware resources. Application

programs run at this level.

Should an application program executing at level 3 be interrupted by a hardware

interrupt for the keyboard, the keyboard interrupt handler must execute at level 0.

Page 299

Chapter 20.0 ◄ Interrupts

The following diagram shows the relationship of the levels.

Level 0

Level 1

Level 2

Level 3

Illustration 29: Privilege Levels

The operating system interrupt processing mechanism will handle this privilege

elevation and restoration in a secure manner. That requires that the interrupt source and

privileges be verified as part of the interrupt handling mechanism.

2

0.3 Interrupt Processing

When an interrupt occurs, it must be handled or processed securely, quickly, and

correctly. The general idea is that when the currently executing process is interrupted it

must be placed on hold, and the appropriate interrupt handling code found and executed.

The specific interrupt processing required depends on the cause or purpose of the

interrupt. Once the interrupt is serviced, the original process execution will eventually

be resumed.

2

0.3.1 Interrupt Service Routine (ISR)

The code that is executed in response to an interrupt is typically called an Interrupt

Service Routine or ISR. The code is sometimes referred to as interrupt handler, handler,

service routine, or ISR code. For consistency, this document will use the term ISR.

Page 300

Chapter 20.0 ◄ Interrupts

ISR code is challenging to develop due to the issues related to the concurrency and race

conditions. Additionally, it is difficult to isolate problems and debug ISR code.

2

0.3.2 Processing Steps

The general steps involved in processing an interrupt are outlined in the following

sections.

20.3.2.1 Suspension

Execution of the current program is suspended. As a minimum, this requires saving the

rip and rFlags registers to system stack. The remaining registers are likely to be

preserved (as a further step), depending on the specific interrupt. The rFlags flag

register must be preserved immediately since the interrupt may have been generated

asynchronously and those registers will change as successive instructions are executed.

This multi-stage process ensures that the program context can be fully restored.

20.3.2.2 Obtaining ISR Address

The ISR addresses are stored in a table referred to as an Interrupt Descriptor Table65

(IDT). For each ISR, the IDT contains the ISR address and some additional information

including task gate (priority and privilege information) for the ISR. Each entry in the

IDT is a total of 8 bytes for a total of 16 bytes per IDT entry. There is a maximum of

2

56 (0-255) possible entries in the IDT.

To obtain the starting address of an ISR, the interrupt number is multiplied by 16 (since

each entry is 16 bytes) which is used as offset into the IDT where the ISR address is

obtained (for that interrupt).

The start of the IDT is pointed to by a dedicated register, IDTR, which is only accessible

by the OS and requires level 0 privilege to access.

The addresses of the ISR routines are specific to the OS version and the specific

hardware complement of the system. The IDT is created when the system initially boots

and reflects the specific system configuration. This is critical in order for the OS to

work correctly and consistently on different system hardware configurations.

20.3.2.3 Jump to ISR

Once the ISR address is obtained from the IDT some validation is performed. This

includes ensuring the interrupt is from a legal/valid source and if a privilege level

change is required and allowed. Once the verifications have been completed

6

5 Note, for Windows this data structure is referred to as the Interrupt Vector Table (IVT).

Page 301

Chapter 20.0 ◄ Interrupts

successfully, the address of the ISR from the IDT is placed in the rip register, thus

effecting a jump to the ISR routine.

20.3.2.4 Suspension Execute ISR

At this point, depending on the specific ISR, a complete process context switch may be

performed. A process context switch involves saving the entire set of CPU registers for

the interrupted process.

In Linux-based OS's, ISR are typically divided into two parts, referred to as the top-half

and bottom-half. Other OS's refer to these as the First-Level Interrupt Handler (FLIH)

and the Second-Level Interrupt Handlers (SLIH).

The top-half or FLIH is executed immediately and is where any critical activities are

performed. The activities are specific to the ISR, but might include acknowledging the

interrupt, resetting hardware (if necessary), and recording any information only

available at the time of interrupt. The top-half may perform some blocking of other

interrupts (which needs to be minimized).

The bottom-half is where any processing activities (if any) are performed. This helps

ensure that the top-half is completed quickly and that any non-critical processing is

deferred to a more convenient time. If a bottom-half exists, the top-half will create and

schedule the execution of the bottom-half.

Once the top-half completes, the OS scheduler will select a new process.

20.3.2.5 Resumption

When the OS is ready to resume the interrupted process, the program context is restored

and an iret instruction is executed (to pop rFlags and rip registers, thus completing

the restoration).

2

0.4 Suspension Interrupt Processing Summary

The following diagram presents an overview of the general flow used for processing

interrupts by the system.

Page 302

Chapter 20.0 ◄ Interrupts

Main Memory

Operating

System

4

ISR

5

1

Executing

Process

3

2

IDT

Illustration 30: Interrupt Processing Overview

The steps are detailed as follows:

1

. Execution of current program is suspended

save rip and rFlags registers to stack

. Obtain starting address of Interrupt Service Routine (ISR)

-

2

-

-

-

interrupt number multiplied by 16

used as offset into Interrupt Descriptor Table (IDT)

obtains ISR address (for that interrupt) from IDT

Page 303

Chapter 20.0 ◄ Interrupts

3

4

. Jump to Interrupt Service Routine

set rip to address from IDT

. Interrupt Service Routine Executes

-

-

-

-

save context (i.e., any additional registers altered)

process interrupt (specific to interrupt generated)

schedule any later data processing activities

5

. Interrupted Process Resumption

-

-

-

resume scheduling based on OS scheduler

restore context

perform iret (to pop rFlags and rip registers)

This interrupt processing mechanism allows a dynamic, run-time lookup for the ISR

address.

2

0.5 Exercises

Below are some quiz questions and suggested projects based on this chapter.

2

0.5.1 Quiz Questions

Below are some quiz questions based on this chapter.

1

2

3

4

5

) What is the operating system responsible for? Name some of the resources.

) What is an interrupt?

) What is an exception?

) What is an ISR and what is it for?

) Where (name of the data structure) does the operating system obtain the address

when an interrupt occurs and what is contained in it?

6

7

8

) When an interrupt occurs, how is the appropriate offset into the IDT calculated?

) What is the difference between the iret and ret instructions?

) Why does the OS use the interrupt mechanism instead of just performing a

standard call.

9

) What is meant by asynchronously occurring interrupts?

Page 304

Chapter 20.0 ◄ Interrupts

1

1

0) What is meant by synchronously occurring interrupts?

1) When an interrupt occurs, the rip and rFlags registers are pushed on the stack.

Much like the call statement, the rip register is pushed to save the return address.

Explain why is the rFlag register pushed on the stack.

1

1

1

2) Name two hardware interrupts.

3) List one way for a program to generate an exception.

4) What is the difference between a maskable and non-maskable interrupt?

2

0.5.2 Suggested Projects

Below are some suggested projects based on this chapter.

1

) Write a program to obtain and list the contents of the IDT. This will require an

integer to ASCII/Hex program in order to display the applicable addresses in

hex. Use the debugger as necessary to debug the program. When working,

execute the program without the debugger to display results.

Page 305

Chapter 20.0 ◄ Interrupts

Page 306

Appendix

A

21.0 Appendix A – ASCII Table

This appendix provides a copy of the ASCII Table for reference.

Char Dec Hex

Char Dec Hex

spc 32 0x20

Char Dec Hex

Char Dec Hex

NUL

SOH

STX

ETX

EOT

ENQ

ACK

BEL

BS

0

1

2

3

4

5

6

7

8

9

0x00

0x01

0x02

0x03

0x04

0x05

0x06

0x07

0x08

0x09

@

A

B

C

D

E

F

64 0x40

65 0x41

66 0x42

67 0x43

68 0x44

69 0x45

70 0x46

71 0x47

72 0x48

73 0x49

74 0x4A

75 0x4B

76 0x4C

77 0x4D

78 0x4E

79 0x4F

80 0x50

81 0x51

82 0x52

83 0x53

`

a

b

c

d

e

f

96 0x60

97 0x61

98 0x62

99 0x63

100 0x64

101 0x65

102 0x66

103 0x67

104 0x68

105 0x69

106 0x6A

107 0x6B

108 0x6C

109 0x6D

110 0x6E

111 0x6F

112 0x70

113 0x71

114 0x72

115 0x73

!

"

#

$

%

&

'

33 0x21

34 0x22

35 0x23

36 0x24

37 0x25

38 0x26

39 0x27

40 0x28

41 0x29

42 0x2A

43 0x2B

44 0x2C

45 0x2D

46 0x2E

47 0x2F

48 0x30

49 0x31

50 0x32

51 0x33

G

H

I

g

h

i

(

TAB

)

LF 10 0x0A

VT 11 0x0B

FF 12 0x0C

CR 13 0x0D

SO 14 0x0E

SI 15 0x0F

DLE 16 0x10

DC1 17 0x11

DC2 18 0x12

DC3 19 0x13

*

+

,

J

j

K

L

M

N

O

P

k

l

-

m

n

o

p

q

r

s

.

/

0

1

2

3

Q

R

S

Page 307

Appendix A – ASCII Table

DC4 20 0x14

NAK 21 0x15

SYN 22 0x16

ETB 23 0x17

CAN 24 0x18

EM 25 0x19

SUB 26 0x1A

ESC 27 0x1B

FS 28 0x1C

GS 29 0x1D

RS 30 0x1E

US 31 0x1F

4

5

6

7

8

9

:

52 0x34

53 0x35

54 0x36

55 0x37

56 0x38

57 0x39

58 0x3A

59 0x3B

60 0x3C

61 0x3D

62 0x3E

63 0x3F

T

U

V

W

X

Y

Z

[

84 0x54

85 0x55

86 0x56

87 0x57

88 0x58

89 0x59

90 0x5A

91 0x5B

92 0x5C

93 0x5D

94 0x5E

95 0x5F

t

u

v

w

x

y

z

{

116 0x74

117 0x75

118 0x76

119 0x77

120 0x78

121 0x79

122 0x7A

123 0x7B

124 0x7C

125 0x7D

126 0x7E

;

<

=

>

?

\

|

]

}

^

_

~

DEL 127 0x7F

For additional information and a more complete listing of the ASCII codes (including

the extended ASCII characters), refer to http://www.asciitable.com/

Page 308

Appendix

B

22.0 Appendix B – Instruction Set Summary

This appendix provides a listing and brief description of the instructions covered in this

text. This set of instructions is a subset of the complete instruction set. For a complete

listing of the instructions, refer to the references noted in chapter 1.

2

2.1 Notation

The following table summarizes the notational conventions used.

Operand Notation Description

<reg>

Register operand. The operand must be a register.

<

<

reg8>, <reg16>, Register operand with specific size requirement. For

reg32>, <reg64>

example, reg8 means a byte sized register (e.g., al, bl,

etc.) only and reg32 means a double-word sized register

(e.g., eax, ebx, etc.) only.

<dest>

Destination operand. The operand may be a register or

memory. Since it is a destination operand, the contents

will be overwritten with the new result (based on the

specific instruction).

<RXdest>

Floating-point destination register operand. The operand

must be a floating-point register. Since it is a destination

operand, the contents will be overwritten with the new

result (based on the specific instruction).

<

src>

imm>

Source operand. Operand value is unchanged after the

instruction.

<

Immediate value. May be specified in decimal, hex, octal,

or binary.

Page 309

Appendix B – Instruction Set Summary

Operand Notation Description

<mem>

Memory location. May be a variable name or an indirect

reference (i.e., a memory address).

<

op> or <operand> Operand, register or memory.

<

<

op8>, <op16>,

op32>, <op64>

Operand, register or memory, with specific size

requirement. For example, op8 means a byte sized

operand only and reg32 means a double-word sized

operand only.

<label>

Program label.

2

2.2 Data Movement Instructions

Below is a summary of the basic data movement and addressing instructions.

Instruction

Explanation

mov   <dest>, <src>

Copy source operand to the destination

operand.

Note 1, both operands cannot be memory.

Note 2, destination operands cannot be an

immediate.

lea   <reg64>, <mem>

Place address of <mem> into reg64.

2

2.3 Data Conversion instructions

Below is a summary of the basic data conversion instructions.

Instruction

Explanation

movzx   <dest>, <src>

Unsigned widening conversion.

Note 1, both operands cannot be memory.

Note 2, destination operands cannot be an

immediate.

movzx   <reg16>, <op8>

movzx   <reg32>, <op8>

movzx   <reg32>, <op16>

movzx   <reg64>, <op8>

movzx   <reg64>, <op16>

Note 3, immediate values not allowed.

cbw

Convert byte in al into word in ax.

Note, only works for al to ax register.

Page 310

Appendix B – Instruction Set Summary

Instruction

Explanation

cwd

Convert word in ax into double-word in dx:ax.

Note, only works for ax to dx:ax registers.

cwde

Convert word in ax into double-word in eax.

Note, only works for ax to eax register.

cdq

Convert double-word in eax into quadword in

edx:eax.

Note, only works for eax to edx:eax registers.

cdqe

cqo

Convert double-word in eax into quadword in

rax.

Note, only works for rax register.

Convert quadword in rax into word in double-

quadword in rdx:rax.

Note, only works for rax to rdx:rax registers.

movsx   <dest>, <src>

Signed widening conversion (via sign

extension).

Note 1, both operands cannot be memory.

Note 2, destination operands cannot be an

immediate.

movsx   <reg16>, <op8>

movsx   <reg32>, <op8>

movsx   <reg32>, <op16>

movsx   <reg64>, <op8>

movsx   <reg64>, <op16>

movsxd  <reg64>, <op32>

Note 3, immediate values not allowed.

2

2.4 Integer Arithmetic Instructions

Below is a summary of the basic integer arithmetic instructions.

Instruction

Explanation

add   <dest>, <src>

Add two operands, (<dest> + <src>) and place

the result in <dest> (over-writing previous

value).

Note 1, both operands cannot be memory.

Note 2, destination operand cannot be an

immediate.

inc   <operand>

Increment <operand> by 1.

Note, <operand> cannot be an immediate.

adc   <dest>, <src>

Add two operands, (<dest> + <src>) and any

Page 311

Appendix B – Instruction Set Summary

Instruction

Explanation

previous carry (stored in the carry bit in the

rFlag register) and place the result in <dest>

(over-writing previous value).

Note 1, both operands cannot be memory.

Note 2, destination operand cannot be an

immediate.

Examples:

adc   rcx, qword [dVvar1]

adc   rax, 42

sub   <dest>, <src>

Subtract two operands, (<dest> - <src>) and

place the result in <dest> (over-writing

previous value).

Note 1, both operands cannot be memory.

Note 2, destination operand cannot be an

immediate.

dec   <operand>

mul <src>

Decrement <operand> by 1.

Note, <operand> cannot be an immediate.

Multiply A register (al, ax, eax, or rax) times

the <src> operand.

mul <op8>

mul <op16>

mul <op32>

mul <op64>

Byte: ax = al * <src>

Word: dx:ax = ax * <src>

Double: edx:eax = eax * <src>

Quad: rdx:rax = rax * <src>

Note, <src> operand cannot be an immediate.

imul <src>

imul <dest>, <src/imm32>

imul <dest>, <src>, <imm32>

Signed multiply instruction.

For single operand:

Byte: ax = al * <src>

imul <op8>

imul <op16>

imul <op32>

imul <op64>

imul <reg16>, <op16/imm>

imul <reg32>, <op32/imm>

imul <reg64>, <op64/imm>

imul <reg16>, <op16>, <imm>

imul <reg32>, <op32>, <imm>

imul <reg64>, <op64>, <imm>

Word: dx:ax = ax * <src>

Double: edx:eax = eax * <src>

Quad: rdx:rax = rax * <src>

Note, <src> operand cannot be an immediate.

For two operands:

<reg16> = <reg16> * <op16/imm>

<

<

reg32> = <reg32> * <op32/imm>

reg64> = <reg64> * <op64/imm>

Page 312

Appendix B – Instruction Set Summary

Instruction

Explanation

For three operands:

<

<

<

reg16> = <op16> * <imm>

reg32> = <op32> * <imm>

reg64> = <op64> * <imm>

div <src>

Unsigned divide A/D register (ax, dx:ax,

edx:eax, or rdx:rax) by the <src> operand.

Byte: al = ax / <src>, rem in ah

Word: ax = dx:ax / <src>, rem in dx

Double: eax = eax / <src>, rem in edx

Quad: rax = rax / <src>, rem in rdx

Note, <src> operand cannot be an immediate.

div <op8>

div <op16>

div <op32>

div <op64>

idiv <src>

Signed divide A/D register (ax, dx:ax,

edx:eax, or rdx:rax) by the <src> operand.

Byte: al = ax / <src>, rem in ah

Word: ax = dx:ax / <src>, rem in dx

Double: eax = eax / <src>, rem in edx

Quad: rax = rax / <src>, rem in rdx

Note, <src> operand cannot be an immediate.

idiv <op8>

idiv <op16>

idiv <op32>

idiv <op64>

2

2.5 Logical, Shift, and Rotate Instructions

Below is a summary of the basic logical, shift, arithmetic shift, and rotate instructions.

Instruction

Explanation

and   <dest>, <src>

Perform logical AND operation on two

operands, (<dest> and <src>) and place the

result in <dest> (over-writing previous value).

Note 1, both operands cannot be memory.

Note 2, destination operand cannot be an

immediate.

or   <dest>, <src>

Perform logical OR operation on two operands,

(<dest> || <src>) and place the result in <dest>

(over-writing previous value).

Note 1, both operands cannot be memory.

Note 2, destination operand cannot be an

immediate.

Page 313

Appendix B – Instruction Set Summary

Instruction

Explanation

xor   <dest>, <src>

Perform logical XOR operation on two

operands, (<dest> ^ <src>) and place the

result in <dest> (over-writing previous value).

Note 1, both operands cannot be memory.

Note 2, destination operand ca not be an

immediate.

not   <op>

Perform a logical not operation (one's

complement on the operand 1's→0's and

0

's→1's).

Note, operand cannot be an immediate.

shl   <dest>, <imm>

shl   <dest>, cl

Perform logical shift left operation on

destination operand. Zero fills from right (as

needed).

The <imm> or the value in cl register must be

between 1 and 64.

Note, destination operand cannot be an

immediate.

shr   <dest>, <imm>

shr   <dest>, cl

Perform logical shift right operation on

destination operand. Zero fills from left (as

needed).

The <imm> or the value in cl register must be

between 1 and 64.

Note, destination operand cannot be an

immediate.

sal   <dest>, <imm>

sal   <dest>, cl

Perform arithmetic shift left operation on

destination operand. Zero fills from right (as

needed).

The <imm> or the value in cl register must be

between 1 and 64.

Note, destination operand cannot be an

immediate.

sar   <dest>, <imm>

sar   <dest>, cl

Perform arithmetic shift right operation on

destination operand. Sign fills from left (as

needed).

The <imm> or the value in cl register must be

between 1 and 64.

Note, destination operand cannot be an

immediate.

Page 314

Appendix B – Instruction Set Summary

Instruction

Explanation

rol   <dest>, <imm>

rol   <dest>, cl

Perform rotate left operation on destination

operand.

The <imm> or the value in cl register must be

between 1 and 64.

Note, destination operand cannot be an

immediate.

ror   <dest>, <imm>

ror   <dest>, cl

Perform rotate right operation on destination

operand.

The <imm> or the value in cl register must be

between 1 and 64.

Note, destination operand cannot be an

immediate.

2

2.6 Control Instructions

Below is a summary of the basic control instructions.

Instruction

Explanation

cmp   <op1>, <op2>

Compare <op1> with <op2>.

Results are stored in the rFlag register.

Note 1, operands are not changed.

Note 2, both operands cannot be memory.

Note 3, <op1> operand cannot be an

immediate.

je   <label>

jne   <label>

jl   <label>

Based on preceding comparison instruction,

jump to <label> if <op1> == <op2>.

Label must be defined exactly once.

Based on preceding comparison instruction,

jump to <label> if <op1> != <op2>.

Label must be defined exactly once.

For signed data, based on preceding

comparison instruction, jump to <label> if

<op1> < <op2>.

Label must be defined exactly once.

Page 315

Appendix B – Instruction Set Summary

Instruction

Explanation

jle   <label>

jg   <label>

jge   <label>

jb   <label>

jbe   <label>

ja   <label>

jae   <label>

loop   <label>

For signed data, based on preceding

comparison instruction, jump to <label> if

<op1> <op2>.

Label must be defined exactly once.

For signed data, based on preceding

comparison instruction, jump to <label> if

<op1> > <op2>.

Label must be defined exactly once.

For signed data, based on preceding

comparison instruction, jump to <label> if

<op1> <op2>.

Label must be defined exactly once.

For unsigned data, based on preceding

comparison instruction, jump to <label> if

<op1> < <op2>.

Label must be defined exactly once.

For unsigned data, based on preceding

comparison instruction, jump to <label> if

<op1> <op2>.

Label must be defined exactly once.

For unsigned data, based on preceding

comparison instruction, jump to <label> if

<op1> > <op2>.

Label must be defined exactly once.

For unsigned data, based on preceding

comparison instruction, jump to <label> if

<op1> <op2>.

Label must be defined exactly once.

Decrement rcx register and jump to <label> if

rcx is ≠ 0.

Note, label must be defined exactly once.

Page 316

Appendix B – Instruction Set Summary

2

2.7 Stack Instructions

Below is a summary of the basic stack instructions.

Instruction

Explanation

push   <op64>

Push the 64-bit operand on the stack. Adjusts

rsp accordingly. Operand is unaltered.

pop   <op64>

Pop the 64-bit operand from the stack. Adjusts

rsp accordingly. The operand may not be an

immediate value. Operand is overwritten.

2

2.8 Function Instructions

Below is a summary of the basic instructions for implementing function calls.

Instruction

Explanation

call   <funcName>

Calls a function. Push the 64-bit rip register

and jump to the <funcName>.

ret

Return from a function. Pop the stack into the

rip register, effecting a jump to the line after

the call.

2

2.9 Floating-Point Data Movement Instructions

Below is a summary of the basic instructions for floating-point data movement

instructions.

Instruction

Explanation

movss   <dest>, <src>

Copy 32-bit source operand to the 32-bit

destination operand.

Note 1, both operands cannot be memory.

Note 2, operands cannot be an immediate.

Examples:

movss   xmm0, dword [x]

movss   dword [fltVar], xmm1

movss   xmm3, xmm2

Page 317

Appendix B – Instruction Set Summary

Instruction

Explanation

movsd   <dest>, <src>

Copy 64-bit source operand to the 64-bit

destination operand.

Note 1, both operands cannot be memory.

Note 2, operands cannot be an immediate.

Examples:

movsd   xmm0, qword [y]

movsd   qword [fltdVar], xmm1

movsd   xmm3, xmm2

2

2.10 Floating-Point Data Conversion Instructions

Below is a summary of the basic instructions for floating-point data conversion

instructions.

Instruction

Explanation

cvtss2sd   <RXdest>, <src> Convert 32-bit floating-point source operand to

the 64-bit floating-point destination operand.

Note 1, destination operand must be floating-

point register.

Note 2, source operand cannot be an immediate.

Examples: cvtss2sd   xmm0, dword [fltSVar]

cvtss2sd   xmm3, eax

cvtss2sd   xmm3, xmm2

cvtsd2ss   <RXdest>, <src> Convert 64-bit floating-point source operand to

the 32-bit floating-point destination operand.

Note 1, destination operand must be floating-

point register.

Note 2, source operand cannot be an immediate.

Examples: cvtsd2ss   xmm0, qword [fltDVar]

cvtsd2ss   xmm1, rax

cvtsd2ss   xmm3, xmm2

cvtss2si   <reg>, <src>

Convert 32-bit floating-point source operand to

the 32-bit integer destination operand.

Note 1, destination operand must be register.

Note 2, source operand cannot be an immediate.

Page 318

Appendix B – Instruction Set Summary

Instruction

Explanation

Examples: cvtss2si   xmm1, xmm0

cvtss2si   eax, xmm0

cvtss2si   eax, dword [fltSVar]

cvtsd2si   <reg>, <src>

Convert 64-bit floating-point source operand to

the 32-bit integer destination operand.

Note 1, destination operand must be register.

Note 2, source operand cannot be an immediate.

Examples: cvtss2si   xmm1, xmm0

cvtss2si   eax, xmm0

cvtss2si   eax, qword [fltDVar]

cvtsi2ss   <RXdest>, <src> Convert 32-bit integer source operand to the 32-

bit floating-point destination operand.

Note 1, destination operand must be floating-

point register.

Note 2, source operand cannot be an immediate.

Examples: cvtsi2ss   xmm0, eax

cvtsi2ss   xmm0, dword [fltDVar]

cvtsi2sd   <RXdest>, <src> Convert 32-bit integer source operand to the 64-

bit floating-point destination operand.

Note 1, destination operand must be floating-

point register.

Note 2, source operand cannot be an immediate.

Examples: cvtsi2sd   xmm0, eax

cvtsi2sd   xmm0, dword [fltDVar]

2

2.11 Floating-Point Arithmetic Instructions

Below is a summary of the basic instructions for floating-point arithmetic instructions.

Page 319

Appendix B – Instruction Set Summary

Instruction

Explanation

addss   <RXdest>, <src>

Add two 32-bit floating-point operands,

(<RXdest> + <src>) and place the result in

<

RXdest> (over-writing previous value).

Note 1, destination operands must be a

floating-point register.

Note 2, source operand cannot be an

immediate.

Examples:

addss   xmm0, xmm3

addss   xmm5, dword [fSVar]

addsd   <RXdest>, <src>

Add two 64-bit floating-point operands,

(<RXdest> + <src>) and place the result in

<

RXdest> (over-writing previous value).

Note 1, destination operands must be a

floating-point register.

Note 2, source operand cannot be an

immediate.

Examples:

addsd   xmm0, xmm3

addsd   xmm5, qword [fDVar]

subss   <RXdest>, <src>

Subtract two 32-bit floating-point operands,

(<RXdest> - <src>) and place the result in

<

RXdest> (over-writing previous value).

Note 1, destination operands must be a

floating-point register.

Note 2, source operand cannot be an

immediate.

Examples:

subss   xmm0, xmm3

subss   xmm5, dword [fSVar]

subsd   <RXdest>, <src>

Subtract two 64-bit floating-point operands,

(<RXdest> - <src>) and place the result in

<

RXdest> (over-writing previous value).

Note 1, destination operands must be a

floating-point register.

Note 2, source operand cannot be an

immediate.

Page 320

Appendix B – Instruction Set Summary

Instruction

Explanation

Examples:

subsd   xmm0, xmm3

subsd   xmm5, qword [fDVar]

mulss   <RXdest>, <src>

Multiply two 32-bit floating-point operands,

(<RXdest> * <src>) and place the result in

<

RXdest> (over-writing previous value).

Note 1, destination operands must be a

floating-point register.

Note 2, source operand cannot be an

immediate.

Examples:

mulss   xmm0, xmm3

mulss   xmm5, dword [fSVar]

mulsd   <RXdest>, <src>

Multiply two 64-bit floating-point operands,

(<RXdest> * <src>) and place the result in

<

RXdest> (over-writing previous value).

Note 1, destination operands must be a

floating-point register.

Note 2, source operand cannot be an

immediate.

Examples:

mulsd   xmm0, xmm3

mulsd   xmm5, qword [fDVar]

divss   <RXdest>, <src>

Divide two 32-bit floating-point operands,

(<RXdest> / <src>) and place the result in

<

RXdest> (over-writing previous value).

Note 1, destination operands must be a

floating-point register.

Note 2, source operand cannot be an

immediate.

Examples:

divss   xmm0, xmm3

divss   xmm5, dword [fSVar]

Page 321

Appendix B – Instruction Set Summary

Instruction

Explanation

divsd   <RXdest>, <src>

Divide two 64-bit floating-point operands,

(<RXdest> / <src>) and place the result in

<

RXdest> (over-writing previous value).

Note 1, destination operands must be a

floating-point register.

Note 2, source operand cannot be an

immediate.

Examples:

divsd   xmm0, xmm3

divsd   xmm5, qword [fDVar]

sqrtss   <RXdest>, <src>

Take the square root of the 32-bit floating-

point source operand and place the result in

destination operand (over-writing previous

value).

Note 1, destination operands must be a

floating-point register.

Note 2, source operand cannot be an

immediate.

Examples:

sqrtss   xmm0, xmm3

sqrtss   xmm7, dword [fSVar]

sqrtsd   <RXdest>, <src>

Take the square root of the 64-bit floating-

point source operand and place the result in

destination operand (over-writing previous

value).

Note 1, destination operands must be a

floating-point register.

Note 2, source operand cannot be an

immediate.

Examples:

sqrtsd   xmm0, xmm3

sqrtsd   xmm7, qword [fDVar]

Page 322

Appendix B – Instruction Set Summary

2

2.12 Floating-Point Control Instructions

Below is a summary of the basic instructions for floating-point control instructions.

Instruction

Explanation

ucomiss   <RXsrc>, <src>

Compare two 32-bit floating-point operands,

(<RXsrc> + <src>). Results are placed in the

rFlag register. Neither operand is changed.

Note 1, <RXsrc> operands must be a floating-

point register.

Note 2, source operand may be a floating-point

register or memory, but not be an immediate.

Examples:

ucomiss   xmm0, xmm3

ucomiss   xmm5, dword [fSVar]

ucomisd   <RXsrc>, <src>

Compare two 64-bit floating-point operands,

(<RXsrc> + <src>). Results are placed in the

rFlag register. Neither operand is changed.

Note 1, <RXsrc> operands must be a floating-

point register.

Note 2, source operand may be a floating-point

register or memory, but not be an immediate.

Examples:

ucomisd   xmm0, xmm3

ucomisd   xmm5, dword [fSVar]

Page 323

Appendix B – Instruction Set Summary

Page 324

Appendix

C

23.0 Appendix C – System Services

This appendix provides a listing and brief description of a subset of the system service

calls. This list is for 64-bit Ubuntu systems. A more complete list can be obtained from

multiple web sources.

2

3.1 Return Codes

The system call will return a code in the rax register. If the value returned is less than

, that is an indication that an error has occurred. If the operation is successful, the

0

value returned will depend on the specific system service. Refer to the Error Codes

section for additional information regarding the values of the error codes.

2

3.2 Basic System Services

The following table summarizes the more common system services.

Call Code System Service Description

(rax)

0

SYS_read Read characters

rdi = file descriptor (of where to read from)

rsi = address of where to store characters

rdx = count of characters to read

If unsuccessful, returns negative value. If successful, returns count

of characters actually read.

1

SYS_write Write characters

rdi = file descriptor (of where to write to)

Page 325

Appendix C – System Services

Call Code System Service Description

(rax)

rsi = address of characters to write

rdx = count of characters to write

If unsuccessful, returns negative value. If successful, returns count

of characters actually written.

2

SYS_open Open a file

rdi = address of NULL terminated file name

rsi = file status flags (typically O_RDONLY)

If unsuccessful, returns negative value. If successful, returns file

descriptor.

3

8

SYS_close Close an open file

rdi = file descriptor of open file to close

If unsuccessful, returns negative value.

SYS_lseek Reposition the file read/write file offset.

rdi = file descriptor (of where to write to)

rsi = offset

rdx = origin

If unsuccessful, returns negative value.

5

7

9

SYS_fork Fork current process.

SYS_execve Execute a program

5

rdi = Address of NULL terminated string for

name of program to execute.

6

0

SYS_exit

Terminate executing process.

rdi = exit status (typically 0)

Page 326

Appendix C – System Services

Call Code System Service Description

(rax)

8

5

SYS_creat Open/Create a file.

rdi = address of NULL terminated file name

rsi = file mode flags

If unsuccessful, returns negative value. If successful, returns file

descriptor.

9

6

SYS_gettimeofday Get date and time of day

rdi = address of time value structure

rsi = address of time zone structure

If unsuccessful, returns negative value. If successful, returns

information in the passed structures.

2

3.3 File Modes

When performing file operations, the file mode provides information to the operating

system regarding the file access permissions that will be allowed.

When opening an existing file, one of the following file modes must be specified.

Mode

Value Description

O_RDONLY

0

Read only. Allow reading from the

file, but to not allow writing to the file.

Most common operation.

O_WRONLY

O_RDWR

1

2

Write only. Typically used if

information is to be appended to a file.

Allow simultaneous reading and

writing.

When creating a new file, the file permissions must be specified. Below are the

complete set of file permissions. As is standard for Linux file systems, the permission

values are specified in Octal.

Page 327

Appendix C – System Services

Mode

Value Description

S_IRWXU

00700q User (file owner) has read, write, and

execute permission.

S_IRUSR

S_IWUSR

S_IXUSR

00400q User (file owner) has read permission.

00200q User (file owner) has write permission.

00100q User (file owner) has execute

permission.

S_IRWXG

00070q Group has read, write, and execute

permission.

S_IRGRP

S_IWGRP

S_IXGRP

S_IRWXO

00040q Group has read permission.

00020q Group has write permission.

00010q Group has execute permission.

00007q Others have read, write, and execute

permission.

S_IROTH

S_IWOTH

S_IXOTH

00004q Others have read permission.

00002q Others have write permission.

00001q Others have execute permission.

The text examples only address permissions for the user or owner of the file.

2

3.4 Error Codes

If a system service returns an error, the value of the return code will be negative. The

following is a list of the error code. The code value is provided along with the Linux

symbolic name. High-level languages typically use the name which is not used at the

assembly level and is only provided for reference.

Error Code Symbolic Description

Name

-1

-2

-3

-4

EPERM Operation not permitted.

ENOENT No such file or directory.

ESRCH No such process.

EINTR Interrupted system call.

Page 328

Appendix C – System Services

-

-

-

-

-

5

6

7

8

9

EIO

I/O Error.

ENXIO No such device or address.

E2BIG Argument list too long.

ENOEXEC Exec format error.

EBADF Bad file number.

ECHILD No child process.

EAGAIN Try again.

-10

-11

-12

-13

-14

-15

-16

-17

-18

-19

-20

-21

-22

-23

-24

-25

-26

-27

-28

-29

-30

-31

-32

ENOMEM Out of memory.

EACCES Permission denied.

EFAULT Bad address.

ENOTBLK Block device required.

EBUSY Device or resource busy.

EEXIST File exists.

EXDEV Cross-device link.

ENODEV No such device.

ENOTDIR Not a directory.

EISDIR Is a directory.

EINVAL Invalid argument.

ENFILE File table overflow.

EMFILE Too many open files.

ENOTTY Not a typewriter.

ETXTBSY Text file busy.

EFBIG File too large.

ENOSPC No space left on device.

ESPIPE Illegal seek.

EROFS Read-only file system.

EMLINK Too many links.

EPIPE Broken pipe.

Page 329

Appendix C – System Services

-

33

34

EDOM Math argument out of domain of

function.

-

ERANGE Math result not representable.

Only the most common error codes are shown. A complete list can be found via the

Internet or by looking on the current system includes files. For Ubuntu, this is typically

located in /usr/include/asm-generic/errno-base.h.

Page 330

Appendix

D

24.0 Appendix D – Quiz Question Answers

This appendix provides answers for the quiz questions in each chapter.

2

4.1 Quiz Question Answers, Chapter 1

There are no quiz questions for Chapter 1.

2

4.2 Quiz Question Answers, Chapter 2

1

2

3

4

5

) See Section 2.1, Illustration 1, Computer Architecture.

) Bus or Interconnection.

) Secondary storage.

) Primary storage or main memory (RAM).

) Keeps a copy of the data closer to the CPU, eliminating the extra time required

to access the RAM via the Bus.

6

7

8

9

) 4 bytes.

) 1 byte.

) The LSB is 40 and the MSB is 00 .

16

16

) The answer is as follows:

High memory

0016

4

4

C16

B16

Low memory

4016

Page 331

Appendix D – Quiz Question Answers

1

0) The layout is:

←

eax

→

→

←

ax

rax =

ah al

1

1) The answers are as follows:

1

2

3

4

5

6

7

8

. 8

. 64

. 16

. 32

. 64

. 8

. 8

. 16

1

1

1

1

2) The rip register.

3) The rsp register.

4) The rax register is: 0000000000000000 .

16

5) The answers are as follows:

1

2

3

4

. EF16

. CDEF16

. 89ABCDEF16

. 0123456789ABCDEF16

2

4.3 Quiz Question Answers, Chapter 3

1

) The answers are as follows:

1

2

. -128 to +127

. 0 to 255

Page 332

Appendix D – Quiz Question Answers

3

4

5

6

. −32,768 to +32,767

. 0 to 65,535

. −2,147,483,648 to +2,147,483,647

. 0 to 4,294,967,295

2

3

4

5

6

) The answers are as follows:

1

2

3

4

. 5

. 9

. 13

. 21

) The answers are as follows:

1

2

3

4

. 0xFD

. 0x0B

. 0xF7

. 0xEB

) The answers are as follows:

1

2

3

4

. 0xFFEF

. 0x0011

. 0xFFE1

. 0xFF76

) The answers are as follows:

1

2

3

4

. 0xFFFFFFF5

. 0xFFFFFFE5

. 0x00000007

. 0xFFFFFEFB

) The answers are as follows:

1

2

. -5

. -22

Page 333

Appendix D – Quiz Question Answers

3

4

. -13

. -8

7

8

) 0.5 is represented as 0.1

10 2

) The answers are as follows:

1

2

3

4

. -12.25

. +12.25

. -6.5

. -7.5

9

1

) The answers are as follows:

1

2

3

4

. 0x41340000

. 0xC1890000

. 0x41AF0000

. 0xBF400000

0) The answers are as follows:

1

2

3

4

5

. 0x41

. 0x61

. 0x30

. 0x38

. 0x09

1

1) The answers are as follows:

1

2

3

. “World” = 0x57 0x6F 0x72 0x6C 0x64

. “123” = 0x31 0x32 0x33

. “Yes!?” = 0x59 0x65 0x73 0x21 0x 3F

2

4.4 Quiz Question Answers, Chapter 4

1

2

) yasm

) With the ; (semicolon).

Page 334

Appendix D – Quiz Question Answers

3

4

5

6

) Section data.

) Section bss.

) Section text.

) The answers are as follows:

1

2

3

4

. bNum db 10

. wNum dw 10291

. dwNum dd 2126010

. qwNum dq 10000000000

7

8

) The answers are as follows:

1

2

3

4

. bArr resb 100

. wArr resw 3000

. dwArr resd 200

. qArr resq 5000

) The declarations are:

global _start

_start:

2

4.5 Quiz Question Answers, Chapter 5

1

2

) The relationship is 1:1 (one to one).

) Creation of symbol table, macro expansion, and evaluation of constant

expressions.

3

4

) Final generation of code, create list file if requested, create object file.

) Combine one or more object files into a single executable, update all relocatable

addresses, search user and system libraries, create cross reference file if

requested, and create final executable file.

5

) Attempt to open executable file (verifying existence and permissions), read

header information, ask operating system to create new process, if successful,

read rest of executable file and load into memory (where specified by operating

system), and inform operating system when load is completed. Note, the loader

does not run the process.

Page 335

Appendix D – Quiz Question Answers

6

) Examples might include:

1

2

. BUFFSIZE + 1

. MAX + OFFSET

Note, assumes that upper case implies defined constant. Many examples

possible.

7

8

9

) See Section 5.1, Illustration 4, Overview: Assemble, Link, Load.

) At run-time.

) The symbol name and the symbol address.

2

4.6 Quiz Question Answers, Chapter 6

1

2

3

4

5

6

) By typing: ddd <progName>

) The “-g” option.

) Executes the program, always starting from the beginning.

) The continue command continues to the next breakpoint.

) Via the menu option Status → Registers.

) The first is the register name, the second is the hex representation of the value,

and the third is the decimal representation of the value (excluding some registers

such as rip and rsp which are always shown in hex).

7

8

) There are multiple ways to exit the debugger including typing exit in the

command window, clicking the x (upper left corner), or using the menu options

File → Exit.

) There are multiple ways to set a breakpoint including double-clicking on the

line, typing b <lineNumber>, or typing b <labelName> (if a label exists on the

desired line).

9

1

1

) The debugger command to read commands from a file is; source <fileName>.

0) The green arrow points to the next instruction to be executed.

1) The answers are as follows:

1

2

3

. x/db &bVar1

. x/dh &wVar1

. x/dw &dVar1

Page 336

Appendix D – Quiz Question Answers

4

5

6

7

. x/dg &qVar1

. x/30db &bArr1

. x/50dh &wArr1

. x/75dw &dArr1

1

2) The answers are as follows:

1

2

3

4

5

6

7

. x/xb &bVar1

. x/xh &wVar1

. x/xw &dVar1

. x/xg &qVar1

. x/30xb &bArr1

. x/50xh &wArr1

. x/75xw &dArr1

1

1

3) The command is: x/ug $rsp

4) The command is: x/5ug $rsp

2

4.7 Quiz Question Answers, Chapter 7

1

) The answers are as follows:

1

2

3

4

5

6

7

8

9

1

. Legal

. Legal

. Illegal, 354 does not fit into a byte

. Legal

. Illegal, sizes do not match

. Illegal, cannot change the value 54

. Legal

. Legal, while legal it would probably result in an incorrect value

. Legal

0. Legal

Page 337

Appendix D – Quiz Question Answers

1

1

1

1

1

1

1. Legal, while legal it would probably result in an incorrect value

2. Illegal, cannot move memory to memory

3. Illegal, cannot move memory to memory

4. Legal

5. Illegal, r16 is not a valid register

6. Legal

2

) The answers are as follows:

1

. Copies the byte value at bVar1 into the rsi register treating as an unsigned

value thus setting the upper 56 bits to 0.

2

. Copies the byte value at bVar1 into the rsi register treating as a signed value,

thus sign extending the upper 56-bits (1's for negative, 0's for positive).

3

4

5

) The answers are as follows:

1

2

.

.

mov ah, 0

cbw

) The answers are as follows:

1

2

.

.

movzx eax, ax

cwde

) The answers are as follows:

1

2

.

.

mov dx, 0

cwd

6

7

) The cwd instruction only converts the signed value in ax into a sign value in

dx:ax (and nothing else). The movsx instruction copies the word source

operand into the double-word destination operand.

) On the first instruction, the destination operand size must be explicitly specified

since the source operand, an immediate value of 1, does not have an inherent

size associated with it. On the second instruction, the destination operand size

can be determined from the source operand (since the eax register is a double-

word in this case).

Page 338

Appendix D – Quiz Question Answers

) The answers are as follows (grouped in sets of 4 for clarity):

8

9

1

1

1

1

1

1

2

. rax = 0x0000 0000 0000 0009

. rbx = 0x0000 0000 0000 000B

) The answers are as follows (grouped in sets of 4 for clarity):

1

2

. rax = 0x0000 0000 0000 0007

. rbx = 0x0000 0000 0000 0002

0) The answers are as follows (grouped in sets of 4 for clarity):

1

2

. rax = 0x0000 0000 0000 0009

. rbx = 0xFFFF FFFF FFFF FFF9

1) The answers are as follows (grouped in sets of 4 for clarity):

1

2

. rax = 0x0000 0000 0000 000C

. rdx = 0x0000 0000 0000 0000

2) The answers are as follows (grouped in sets of 4 for clarity):

1

2

. rax = 0x0000 0000 0000 0001

. rdx = 0x0000 0000 0000 0002

3) The answers are as follows (grouped in sets of 4 for clarity):

1

2

. rax = 0x0000 0000 0000 0002

. rdx = 0x0000 0000 0000 0003

4) The answers are as follows:

1

2

. The destination operand cannot be an immediate value (42).

. An immediate operand is not allowed since the size/type cannot be

determined.

3

4

. The mov instruction does not allow a memory to memory operation.

. An address requires 64-bits which will not fit into the ax register.

1

1

1

5) The idiv instruction will divide by edx:eax and edx was not set.

6) The operands for the divide a signed (-500), but an unsigned divide is used.

7) The word divide used will place the result in the dx:ax registers, but the eax

register is used to obtain the result.

Page 339

Appendix D – Quiz Question Answers

1

8) The three-operand multiply instructions are only allowed for a limited set of

signed multiplication operations.

2

4.8 Quiz Question Answers, Chapter 8

1

) The first instruction places the value from qVar1 into the rdx register. The

second instruction places the address of qVar1 into the rdx register.

2

) The answers are as follows:

1

2

3

4

5

6

7

8

. Immediate

. Memory

. Immediate

. Illegal, destination operand cannot be an immediate value

. Register

. Memory

. Memory

. Illegal, source and destination operands not the same size.

3

4

) The answers are as follows (grouped in sets of 4 for clarity):

. eax = 0x0000 000A

) The answers are as follows (grouped in sets of 4 for clarity):

1

1

2

. eax = 0x0000 0003

. edx = 0x0000 0002

5

6

) The answers are as follows (grouped in sets of 4 for clarity):

1

2

3

4

. eax = 0x0000 0009

. ebx = 0x0000 0002

. rcx = 0x0000 0000 0000 0000

. rsi = 0x0000 0000 0000 000C

) The answers are as follows (grouped in sets of 4 for clarity):

1

2

. rax = 0x0000 0010

. rcx = 0x0000 0000 0000 0000

Page 340

Appendix D – Quiz Question Answers

3

4

. edx = 0x0000 0000

. rsi = 0x0000 0000 0000 0004

7

8

) The answers are as follows (grouped in sets of 4 for clarity):

1

2

3

4

. eax = 0x0000 0002

. rcx = 0x0000 0000 0000 0000

. edx = 0x0000 0005

. rsi = 0x0000 0000 0000 0003

) The answers are as follows (grouped in sets of 4 for clarity):

1

2

3

4

. eax = 0x0000 0018

. edx = 0x0000 0000

. rcx = 0x0000 0000 0000 0000

. rsi = 0x0000 0000 0000 0005

2

4.9 Quiz Question Answers, Chapter 9

1

2

3

4

) The rsp register.

) First, rsp = rsp - 8 and then rax register is copied to [rsp] (in that order).

) 8 bytes.

) The answers are as follows (grouped in sets of 4 for clarity):

1

2

3

. r10 = 0x0000 0000 0000 0003

. r11 = 0x0000 0000 0000 0002

. r12 = 0x0000 0000 0000 0001

5

6

) The array is reversed in memory.

) Memory is used more efficiently.

2

4.10 Quiz Question Answers, Chapter 10

1

2

) An unambiguous, ordered sequence of steps involved in solving a problem.

) The answer is as follows:

1

. Understand the Problem

Page 341

Appendix D – Quiz Question Answers

2

3

4

. Create the Algorithm

. Implement the Program

. Test/Debug the Program

3

) No, the steps are applicable to any language or complex problem (even beyond

programming).

4

5

6

7

) An assemble-time error.

) An assemble-time error.

) An assemble-time error.

) A run-time error.

2

4.11 Quiz Question Answers, Chapter 11

1

2

3

) At the top (above the data, BSS, and text sections).

) Once for each time the macro is invoked.

) The %% will ensure that a unique label name is generated each time the macro is

used.

4

5

6

) If the %% is omitted on a label, the label will be copied, as is, and thus appear to

be duplicated.

) Yes. This might be used to exit a macro to an error handling code block (not

within the macro).

) The macro argument substitution occurs at assemble-time.

2

4.12 Quiz Question Answers, Chapter 12

1

2

3

4

5

6

) Linkage and Argument Transmission.

) The call and the ret instructions.

) Call-by-value.

) Call-by-reference.

) Once, regardless of how many times it is called.

) The current rip is placed on the stack and the rip is changed to the address of

called function.

7

) Save and restore the contents of the callee preserved registers.

Page 342

Appendix D – Quiz Question Answers

8

9

1

) In: rdi, rsi, rdx, rcx, r8, and r9.

) In: edi, esi, edx, ecx, r8d, and r9d.

0) That a function may change the value in the register without needing to save and

restore it.

1

1

1

1

1

1

1

1

1

2

1) Two of: rax, rcx, rdx, rsi, rdi, r8, r9, r10, and r11.

2) Call frame, function call frame, or activation record.

3) A leaf function does not call other functions.

4) Clear the passed arguments off the stack.

5) Twenty-four (24) since three arguments at 8 bytes each (8 x 3).

6) The offset is [rbp+16] regardless of which saved registers are pushed.

7) Available memory.

8) Call-by-reference.

9) The 7th argument is at [rbp+16] and the 8th argument is at [rbp+24].

0) Memory efficiency since stack dynamic local variables only use memory when

needed (when the function is being executed).

2

4.13 Quiz Question Answers, Chapter 13

1

2

3

) The rax register.

) The operating system.

) The call code is SYS_write (1). The 1st argument in rdi is the output location

STDOUT, the 2nd argument in rsi is the starting address of the characters to

output, and the 3rd argument in rdx is the number of characters to write.

4

5

6

7

) It is unknown how many characters will be entered.

) The rax register will contain the file descriptor.

) The rax register will contain an error code.

) In: rdi, rsi, rdx, r10, r8, and r9.

2

4.14 Quiz Question Answers, Chapter 14

1

) The statement is: extern func1, func2

Page 343

Appendix D – Quiz Question Answers

2

3

4

5

6

) The statement is: extern func1, func2

) The assembler will generate an error.

) Link-time.

) The linker will generate an unsatisfied external reference error.

) Yes. However, in the debugger, the code would not be displayed.

2

4.15 Quiz Question Answers, Chapter 15

1

2

3

4

) The buffer overflow exploit is typically called stack smashing.

) The C function does not check the array bounds of the input arguments.

) Yes.

) Typing a very large number of characters when input is requested and, if the

program crashes.

5

6

7

) A series of nop instructions designed to make the target of a buffer overflow

exploit easier to hit.

) Many possible answers. Delete a file, open a network connection, kill a process,

etc.

) Use of canaries, implementation of Data Execution Prevention (DEP), and use of

Data Address Space Layout Randomization.

2

4.16 Quiz Question Answers, Chapter 16

1

2

3

4

) The operating system. Specifically, the loader.

) The program being executed.

) The name of the executable file.

) The argc refers to argument count and argv refers to the argument vector

(starting address of the table of addresses for the string representing each

argument).

5

6

7

) In the rdi register.

) In the rsi register.

) The spaces are removed by the operating system so the program does not have to

do anything.

Page 344

Appendix D – Quiz Question Answers

) No. The program is required to check and determine if that is an error.

8

2

4.17 Quiz Question Answers, Chapter 17

1

) The end of line character for Linux is linefeed (LF) and the end of line character

for Windows is carriage return, line feed (CR, LF).

2

3

4

5

6

) Store a subset of the information for quick access.

) They are in the language I/O library functions (i.e., cout, cin, etc.).

) Simplify the programming.

) I/O performance improvement.

) The system service functions require a specific number of characters to read

which is not known ahead of time for “one line” of text.

7

8

) Keeps a subset of the information at the next higher level in the hierarchy (which

is faster than the next lower level).

) Reduces the overhead associated bus contention and memory latency for

excessive system reads.

9

1

1

) The variable values must be retained between function calls.

0) The end of file must be inferred from the number of characters actually read.

1) Many reasons possible, including file being deleted in another Window after the

open or the drive (USB) being removed after the open.

1

1

1

2) The actual number of characters read will be 0 which must be checked explicitly.

3) To ensure the passed line buffer array is not overwritten.

4) By initializing the variables to indicate that all buffer characters have been read.

2

4.18 Quiz Question Answers, Chapter 18

1

2

3

) The registers are: xmm0, xmm1, xmm2, . . ., xmm15.

) Single precision is 4 bytes and double precision is 8 bytes.

) Cumulative rounding error associated with the inexact representation of 0.1 in

binary.

4

5

) Float functions return the value in xmm0.

) None of the floating-point registers are preserved.

Page 345

Appendix D – Quiz Question Answers

2

4.19 Quiz Question Answers, Chapter 19

1

) Concurrency implies multiple different (not necessarily related) processes

simultaneously making progress. Parallel processing implies that processes are

executing simultaneously.

2

3

4

) Distributed computing and multiprocessing.

) On different computers connected via a network.

) Many possible answers, including Folding@Home and SETI@Home. An

Internet search can provide a more complete listing.

5

6

) On different cores in the CPU.

) Distributed computing allows a very large number of compute nodes but

requires communications over a network which has inherent communication

delays.

7

8

) Multiprocessing allows very fast communications between processes via shared

memory but supports only a limited amount of simultaneous executing threads

related to the number of cores available.

) Multiple threads simultaneously writing to a shared variable with no control or

coordination.

9

1

) No. No problem exists since the variable is not being changed.

0) Yes. Since the variable is being changed, one thread may alter the value after

the other has obtained the value.

2

4.20 Quiz Question Answers, Chapter 20

1

) The operating system is responsible for managing the resources. The resources

include CPU cores, primary memory, secondary storage, display screen,

keyboard, and mouse.

2

3

) An event that alters the sequence of instructions executed by a processor.

) An interrupt that is caused by the current process and needs attention of the

kernel.

4

5

) An ISR is an Interrupt Service Routine which is executed when an interrupt

occurs to service (perform required actions) that interrupt.

) Interrupt Descriptor Table (IDT) which contains the addresses of the Interrupt

Service Routines (ISRs) and the gate information.

Page 346

Appendix D – Quiz Question Answers

6

7

) The interrupt number is multiplied by 16.

) The ret instruction will pop the return address from the stack and place it in the

rip register. The iret instruction will pop the return address and the preserved

flag register contents from the stack and place it in the rip register and rFlag

registers.

8

) The call requires the target address. Since the ISR addresses may change due to

hardware changes or software updates, the interrupt mechanism performs a run-

time look up for the ISR address.

9

1

1

) That the interrupt timing, when or even if the interrupt might occur, cannot be

predicted in the context of the executing code.

0) That the interrupt timing can be predicted in the context of the executing code.

This is typical of system service calls or exceptions such as division by 0.

1) Each instruction changes the rFlag register. After the interrupt is completed, the

flag register must be restored to its original value to ensure that the interrupted

process is able to resume.

1

2) Many possible answers, including I/O devices such as keyboard and mouse,

network adapter, secondary storage devices, or other peripherals.

1

1

3) Many possible answers, including dividing by 0.

4) A maskable interrupt may be ignored briefly where a non-maskable interrupt

must be handled immediately.

Page 347

Appendix D – Quiz Question Answers

Page 348

Index

25.0 Alphabetical Index

0

5

x............................................................72 automatics.............................................172

3..........................................................288 base address..........................................132

activation record...................................178 Base Pointer Register.............................12

add with carry.........................................83 Basic System Services..........................325

Addresses and Values.....................75, 129 biased exponent......................................25

Addressing Modes................................129 BSS.........................................................17

American Standard Code for Information BSS Section............................................35

Interchange.............................................28 buffer....................................................256

Architecture Overview.............................7 Buffering Algorithm.............................257

argc.......................................................246 byte addressable..............................15, 132

Argument Count...................................247 cache memory...........................................9

Argument Transmission.......................175 Cache Memory.................................14, 19

Argument Vector Table........................247 call code................................................197

argv.......................................................246 call frame..............................................178

Arithmetic Logic Unit..............................9 Call Frame............................................178

ASCII......................................................28 call-by-reference...................................175

Assemble and Link...............................229 call-by-value.........................................175

Assemble Commands.............................43 callee.....................................................175

Assemble/Link Script.............................50 Callee............................................181, 184

Assemble/Link/Load Overview..............41 caller.....................................................175

Assembler...............................................43 Caller............................................181, 183

Assembler Directives..............................47 Calling Convention...............................175

Assembler Error....................................161 Calling System Services.......................197

Assembly Main.....................................226 Central Processing Unit............................9

assembly source file...............................33 character.................................................28

Asynchronous Interrupts......................296 code generation.......................................47

Page 349

Alphabetical Index

Code Injection......................................240 Direct Memory Access.........................256

Code Injection Protections...................241 Displaying Register Contents.................60

Code to Inject.......................................237 Displaying Stack Contents.....................65

Command Line Arguments..................245 distributed computing...........................288

Comments...............................................33 Distributed Computing.........................288

Compile, Assemble, and Link..............232 distributed processing...........................288

concurrency..........................................287 DMA.....................................................256

Conditional Control Instructions..........112 dq............................................................35

Console Input........................................203 dt.............................................................35

Console Output.....................................199 dw...........................................................35

constant expression.................................46 dynamic linking......................................49

Conversion Instructions..........................76 Dynamic Linking....................................49

CPU register...........................................10 epilogue................................................175

Data Address Space Layout

Error Codes...........................................328

Randomization......................................242 Error Terminology................................161

Data Execution Prevention...................242 Example Program...................................37

Data Movement..............................73, 264 Example Program, Absolute Value......283

Data representation.................................21 Example Program, List Summation.....133

Data Section............................................34 Example Program, Pyramid Areas and

Data Stack Smashing Protector (or

Volumes................................................135

Canaries)...............................................242 Example Program, Sum and Average. .281

db............................................................34 Example, C++ Main / Assembly Function

dd............................................................35 ..............................................................230

DDD Configuration Settings..................57 Example, Console Input.......................204

DDD Debugger.......................................55 Example, Console Output.....................200

DDD/GDB Commands Summary..........62 Example, File Read..............................217

DDD/GDB Commands, Examples.........64 Example, File Write..............................212

ddq..........................................................35 Example, Statistical Function 1 (leaf). .180

Debugger................................................52 Example, Statistical Function2 (non-leaf)

Debugger Command, next....................172 ..............................................................183

Debugger Command, step....................172 Example, Sum and Average.................226

Debugger Commands...........................172 Exceptions............................................297

Debugger Commands File (non-

Executing Programs................................58

interactive)..............................................66 extern....................................................225

Debugging Macros...............................169 Extern Statement..................................225

Defining Constants.................................34 external reference...................................48

dereferencing........................................130 File Control Block................................208

Destination operand........................72, 309 file descriptor................199, 203, 208, 211

Page 350

Alphabetical Index

File Modes............................................327 Immediate value.............................72, 309

File Open..............................................209 index.....................................................132

File Open Operations............................208 indirection.............................................130

File Open/Create...................................210 input buffer...........................................256

File Operations Examples.....................212 Input/Output Buffering.........................255

File Read...............................................211 Instruction Pointer Register....................12

File Write..............................................211 Instruction Set Overview........................71

First Pass.................................................46 Integer / Floating-Point Conversion

Flag Register...........................................12 Instructions...........................................266

Floating-Point Addition........................268 Integer Arithmetic Instructions...............80

Floating-Point Arithmetic Instructions.268 integer numbers......................................21

Floating-Point Calling Conventions.....281 Interactive Debugger Commands File....65

Floating-Point Comparison..................277 Interfacing with a High-Level Language

Floating-Point Control Instructions......277 ..............................................................230

Floating-point destination register operand Interrupt Categories..............................297

.

.......................................................72, 309 Interrupt Classification.........................296

Floating-Point Division........................273 Interrupt Processing..............................300

Floating-Point Instructions...................263 Interrupt Service Routine (ISR)............300

Floating-Point Multiplication...............271 Interrupt Timing...................................296

Floating-Point Registers.......................264 Interrupt Types.....................................298

Floating-point Representation................24 Interrupt Types and Levels...................298

Floating-Point Square Root..................275 Interrupts...............................................295

Floating-Point Subtraction....................270 Iteration.................................................118

Floating-Point Values...........................263 Jump Out of Range...............................115

forward reference....................................46 Jump to ISR..........................................301

Function Declaration............................173 Labels...................................................112

Function Source....................................228 leaf function..........................................178

Functions..............................................171 Least Significant Byte............................16

General Purpose Registers......................10 light-weight process..............................289

GPRs.......................................................10 Linkage.................................................174

Hardware Interrupt...............................297 Linker.....................................................47

heap.................................................17, 148 Linking Multiple Files............................48

I/O buffering.........................................255 Linking Process......................................48

IEEE 32-bit Representation....................25 list file.....................................................43

IEEE 64-bit Representation....................27 List File...................................................43

IEEE 754 32-bit floating-point standard 24 List Summation....................................133

IF statement..........................................112 little-endian.............................16, 150, 239

Immediate Mode Addressing...............130 Loader.....................................................51

Page 351

Alphabetical Index

Logic Error...........................................162 Operand Notation...........................72, 309

logical AND operation.........................106 operands..................................................71

logical NOT operation..........................106 operation.................................................71

logical OR operation.............................106 ordered floating-point comparisons......277

logical XOR operation..........................106 parallel processing................................287

machine language...............17, 43, 45, 238 Parallel Processing................................287

Macro Definition..................................166 Parameter Passing.................................176

Macros..................................................165 Parsing Command Line Arguments.....245

Main Memory.........................................15 pop operation........................................145

memory hierarchy...........................17, 255 POSIX Threads.....................................289

Memory Hierarchy.................................17 preserved register..................................177

memory layout................................17, 147 Primary Storage..................................7, 19

Memory Mode Addressing...................130 Privilege Levels....................................299

Most Significant Byte.............................16 Process Stack........................................145

Multi-Line Macros................................166 Processing Steps...................................301

Multi-user Operating System...............295 processor registers....................................9

Multiple Source Files...........................225 Program Development..........................155

multiprocessing.....................................288 Program Format......................................33

Multiprocessing....................................288 prologue................................................175

NaN.................................................27, 277 push operation......................................145

Narrowing conversions...........................76 race condition.......................................289

Narrowing Conversions..........................76 Race Conditions....................................290

Newline Character................................198 Random Access Memory (RAM).............7

Next / Step..............................................60 RBP.........................................................12

nexti......................................................169 Red Zone..............................................180

Non-interactive Debugger Commands File register....................................................10

.

...............................................................67 Register Mode Addressing...................130

non-volatile memory................................8 Register operand.............................72, 309

NOP slide..............................................241 Register Usage......................................177

normalized scientific notation................25 resb.........................................................36

not a number...................................27, 277 resd.........................................................36

Not a Number (NaN)..............................27 resdq.......................................................36

Notation................................................309 resq.........................................................36

Notational Conventions..........................71 Resumption...........................................302

Numeric Values......................................33 resw.........................................................36

object file................................................43 Return Codes........................................325

Obtaining ISR Address.........................301 rFlags......................................................12

offset.....................................................132 RIP..........................................................12

Page 352

Alphabetical Index

RSP.........................................................12 Standard Calling Convention...............173

Run / Continue........................................60 Starting DDD..........................................55

Run-time Error......................................161 stepi.......................................................169

S-NaN...................................................277 string.......................................................29

saved register........................................177 Summary...............................................190

Second Pass............................................46 Suspension............................................301

secondary storage.....................................8 Suspension Execute ISR.......................302

Secondary Storage..............................7, 19 Suspension Interrupt Processing Summary

section .bss..............................................35 ..............................................................302

section .data............................................34 symbol table............................................46

section .text.............................................36 Synchronous Interrupts.........................296

segment fault........................................237 system call............................................197

Setting Breakpoints................................57 System Services....................................197

short-jump.............................................115 Text Section............................................36

sign extension.......................................108 thread....................................................289

sign-extend.............................................78 Tool Chain..............................................41

signed......................................................21 Two-Pass Assembler..............................45

Signed Conversions................................78 two's complement................................22ꢀf.

Single Instruction Multiple Data............13 Unconditional Control Instructions......112

Single-Line Macros..............................165 Understanding a Stack Buffer Overflow

Software Interrupts...............................298 ..............................................................236

source file...............................................43 Unicode...................................................29

Source operand...............................72, 309 uninitialized data............................17, 148

stack buffer overflow............................235 unordered floating-point comparisons..277

Stack Buffer Overflow.........................235 unsafe function.....................................237

Stack Dynamic Local Variables...........172 unsigned..................................................21

Stack Example......................................151 Unsigned Conversions............................77

stack frame............................................178 Updated Linking Instructions...............171

Stack Implementation...........................147 Using a Macro......................................166

Stack Instructions.................................146 volatile memory........................................8

Stack Layout.........................................147 Why Buffer?.........................................255

Stack Operations...................................149 Widening conversions............................76

Stack Pointer Register............................12 Widening Conversions...........................76

stack smashing......................................235 %define.................................................165

Stack-Based Local Variables................187

Page 353